Saskatchewan

Labour

Occupational Health

and Safety

Occupational Health and Safety

Committee Manual

# Table of contents

Introduction	1

What’s in this guide	2

Chapter 1: Maintaining a safe workplace	5

Introduction	5

Fundamentals	5

What is occupational health and safety? 	6

What causes accidents? 	7

The internal responsibility system for occupational

health and safety	12

What is the goal of an internal responsibility system? 	13

What is the philosophy of internal responsibility?	14

What are the components of an effective IRS?	16

Direct responsibility for occupational health and safety	17

What are the legislated duties of employers, supervisors

and workers in the IRS?	18

What is due diligence?	18

What is shared responsibility for health and safety?	19

Worker rights	20

Critical role of committees in the IRS	21

What are the major responsibilities of committees?	22

Role of the legislation in supporting internal responsibility	23

Role of the Occupational Health

and Safety (OH&S) Division	24

How does the IRS link up with the OH&S Division?	25

Review	26

Chapter 2: Identifying, assessing and controlling hazards	27

Introduction	27

Definitions	27

What is a hazard?	27

What is an occupational illness?	27

What is probability?	28

What is severity?	28

What is a risk?	28

What is risk analysis?	28

Principles of hazard identification, assessment and control	29

Collect information	29

Assess the risk	30

Set priorities	30

Communicate information	32

Develop, select and implement controls	32

Identifying and assessing health hazards	33

How can health hazards be identified?	33

How can health hazards be assessed?	36

Identifying and assessing safety hazards	41

Controlling hazards	42

Example: managing the safety of controlled products

Under WHMIS	43

Technical steps in hazard control	44

Selecting controls	47

Monitoring the effectiveness of controls	48

What else can be done?	48

Review	49

Chapter 3: The committees’ role in accident prevention	51

Introduction	51

How committees are expected to function	52

Where committees are required	52

How committees are structured	53

Selecting members	53

Posting names	54

Selecting co-chairpersons (regulation 43)	54

Terms of office (regulation 39)	54

Quorum at meetings (regulation 40)	54

Frequency of meetings (regulation 41)	55

Additional meetings	55

Minutes of meetings (regulation 42)	55

Duties of committees	56

Dealing with the concerns of workers	56

Handling refusals to work under section 23 of the Act	59

Advising and assisting the employer	63

Helping the employer with policies, plans and programs	63

Helping the employer orientate new

and inexperienced workers	65

Helping the employer plan worker training	65

Worker training programs	66

Communicating with workers	68

Protection for committee members	68

Review	69

Chapter 4: Responsibilities of employers

and employees towards the committee	73

Introduction	73

Responsibilities of employers	73

Training co-chairpersons	75

Providing information	76

Providing time	77

Helping the employer to inspect the workplace regularly	77

Providing progress reports	78

Helping the committee conduct investigations	78

Resolving concerns reported by the committee	80

Providing access to records	81

Responsibilities of managers, supervisors and workers	81

Managers	81

Supervisors	82

Workers	82

Review	83

Chapter 5: Holding effective committee meetings	85

Introduction	85

Meeting requirements	86

Planning meetings	86

Managing meetings and solving problems	85

Some items to concentrate on	88

Look for underlying causes of problems	90

Consider both facts and feelings	90

Involve stakeholders	92

Consider what is known and not known about the problem	92

Review the sequence of events	92

Developing recommendations	92

After the meeting	94

Role of the employer	95

Review	96

Chapter 6: Maintaining committee effectiveness	97

Introduction	97

Teambuilding	97

Establish membership	97

Clarify the expectations of committee members	98

Develop group norms	98

Agree upon goals	98

Set measurable objectives to achieve the goals	98

Establish roles and responsibilities for each member	99

Agree on ways of handling disagreements	99

Clarify how members will support each other	100

Consider how to involve influential employees

not on the committee	100

Consider expectations placed on the committee	100

Consider organizational factors affecting

the work of the team	100

Consider how to evaluate the performance

of the committee	101

Training	101

New members	101

Review	102

Chapter 7: Inspections	103

Introduction	103

Types of committee inspections	104

Regular inspections	104

Inspections with an occupational health officer	105

Intermittent inspections	105

Training for inspections	105

Planning inspections	105

Inventories and checklists	106

What to inspect	108

Pre-inspection meetings	109

What to do during inspections	109

Handling the results	110

When an unsafe condition is found	111

Review	112

Chapter 8: Investigating accidents	113

Introduction	113

Reportable accidents and dangerous occurrences	114

Planning investigations	114

Standards	115

Carrying out investigations	115

Secure the scene and report the accident or

dangerous occurrence	116

Study the scene	116

Interview witnesses	117

Investigate the physical evidence	117

Finding the causes	118

Analyze the accident factors	118

Find the accident causes	119

Taking action	120

Review	120

Chapter 9: Legislation	123

Introduction	123

The difference between an Act and regulations	123

How to locate and apply information in the legislation	124

Use the tables of contents, index

and section headings to find information	124

Look in the Preliminary Matters sections

to find definitions	124

Look in the appendix to find reference lists and tables	124

Key legislation for committees	125

Review	134

##### Appendices

Appendix 1: Notification requirements for new operations	136

Appendix 2: Forms	137

How to complete a minutes form	137

Committee recommendation form	140

Committee accident investigation form	144

###### Appendix 3: Communication, conflict resolution

and problem solving techniques for committee members	150

Introduction	150

Four principles of health and safety communication	150

Verbal communication techniques	151

Active listening	152

Written communication	153

Raising controversial issues	155

Other communication tools	156

Appendix 4: Helping the employer plan worker training	158

Training procedures	158

Monitoring orientation and training	159

Bibliography	161

Index

Introduction

Saskatchewan's approach to occupational health and safety legislation stems from the philosophy that responsibility for occupational health and safety is shared in the workplace. The Occupational Health and Safety Act, 1993 (called the Act) and The Occupational Health and Safety Regulations, 1996 (called the regulations) set out a structure for this internal responsibility system (called the IRS).

Occupational health committees (called committees or OHCs) are required in workplaces with ten or more workers. Worker health and safety representatives (representatives) are required in high hazard industries with from five to nine workers. Committees and representatives are the primary mechanisms for employers and workers to work together in identifying and resolving health and safety concerns. Experience with committees has proven they reduce accidents and illnesses in the workplace.

A committee helps the employer and workers to improve health and safety in the workplace by helping to identify and resolve concerns. This is done, in part, by:

- helping the employer identify, assess and control hazards

- providing a channel of communication between the employer and workers

- conducting regular inspections

- talking with workers about their health and safety concerns

- helping the employer develop and monitor policies, plans and programs

- participating in accident investigations and investigations of dangerous occurrences (called “near misses”)

- meeting to discuss concerns and make recommendations for corrective action to discuss with the employer

What is in this guide

This guide is intended to help committees and employers understand their responsibilities for establishing and maintaining healthy and safe workplaces. It is not a detailed description of the legislation. Terms used, such as “work together” or “should” do not necessarily mean that there is a legal requirement for the committee to do everything discussed.

Legal requirements are partially listed in Chapter 9. The Act and regulations require the committee to perform specific tasks, such as investigating refusals to work (section 23) and dangerous occurrences (regulation 9). Generally, the legislation requires the employer to involve the committee in developing and promoting all required policies, plans, programs and procedures and in reviewing accident, injury and other prescribed reports.

It is unrealistic to expect every committee to be involved in writing all prescribed policies, plans, programs and procedures. However, the employer must provide the committee with enough time and resources to provide input during their development.

Read the entire manual or only what you need right now. Review the legislation as you go. Adapt the information to suit your needs.

Chapter 1: Maintaining a safe workplace

This chapter reviews the role of the employer and committee, how committees are expected to function and their duties within the internal responsibility system. It also discusses the role of the Occupational Health and Safety Division (called the OH&S Division) of Saskatchewan Labour.

Chapter 2: Identifying, assessing and controlling hazards

Read this chapter to learn the basics of hazard identification, assessment and control.

Chapter 3: The committees’ role in accident prevention

This chapter discusses the nuts and bolts of committee structure and organization. It reviews how committees operate, where they are required, how they should be set up and what their duties are.

Chapter 4: Responsibilities of employers and employees towards the committee

Review this chapter for information about the responsibilities of employers, managers, supervisors and workers to the committee.

Chapter 5: Holding effective committee meetings

Each committee is expected to meet at least once every three months. Read this chapter to find out how to hold effective meetings.

Chapter 6: Maintaining committee effectiveness

This chapter discusses how committee members can develop a strong sense of teamwork, obtain training and orientate new members.

Chapter 7: Inspections

Read this chapter for information about how committees plan, carry out and benefit from workplace inspections.

Chapter 8: Investigating accidents

This chapter provides information about the duties of committees and employers in investigating accidents and dangerous occurrences.

Chapter 9: Legislation

Read this chapter for information about the responsibilities of committees and employers as set out in the Act and regulations.

Appendix

This reviews special topics, such as notification requirements for new operations and how to complete a committee minutes form. It provides form templates and discusses effective communication techniques, problem solving methods and dispute resolution processes.

Notes

Maintaining a safe workplace

- Fundamentals

- The internal responsibility system for occupational health and safety

- Direct responsibility for occupational health and safety

- Critical role of committees in the IRS

- Role of the legislation in supporting internal responsibility

- Role of the Occupational Health and Safety (OH&S) Division

## Introduction

This chapter reviews what causes accidents and how an effective internal responsibility system (called the IRS) for health and safety can prevent them. We’ll discuss responsibilities within the IRS, including the role of the occupational health committee (called a committee). We’ll deal with how Saskatchewan’s occupational health and safety legislation and the Occupational Health and Safety Division (called the OH&S Division) of Saskatchewan Labour support the IRS.

## Fundamentals

Smart employers realize that it is easier and cheaper to manage risk than administer loss. They know the root causes of accidents and incidents (called “near misses”) also cause quality control, environmental protection, production and labour relations problems. They understand accident prevention also prevents other losses. Modern organizations combine health and safety with environmental protection and quality control activities. Successful employers have found effective health and safety goes hand in hand with competitiveness.

Workers have a right to a safe and healthy workplace. They normally suffer the most from accidents and occupational illnesses. Since workers may know more about the daily hazards they face than the employer, it makes sense for them to have an essential role in hazard identification, assessment and control.

Saskatchewan’s health and safety approach reflects this experience. Every officer, director, manager, contractor, supplier, self-employed person, supervisor and worker must do everything that can reasonably be expected to maintain healthy and safe working conditions (in other words, “show due diligence”). Joint worker/management occupational health committees (committees) play an invaluable role in helping the internal responsibility system for health and safety succeed. The IRS is discussed later in this chapter.

### What is occupational health and safety?

The human suffering and financial loss caused by accidents and illnesses at work each year in Saskatchewan is tremendous. Accident statistics and their financial costs are set out in Saskatchewan Workers’ Compensation Board (WCB) figures. Every statistic represents incalculable human suffering. The suffering of the injured is often intensified by the knowledge that most accidents are preventable. Effective occupational health and safety programs prevent accidents and reduce suffering.

Occupational health and safety involves more than simply correcting unsafe actions and conditions. Under section 2(1)(p) of the Act, it means:

- the promotion and maintenance of the highest degree of physical, mental and social well-being of workers

- the prevention among workers of ill health caused by their working conditions

- the protection of workers in their employment from factors adverse to their health

- the placing and maintenance of workers in working environments that are adapted to their individual physiological and psychological conditions

- the promotion and maintenance of a working environment that is free of harassment

Where these things are not present, accidents, illnesses and loss result.

### What causes accidents?

This book uses a simplified version of the accident causation theory proposed by Edward E. Adams. The falling domino sequences shown in this section illustrate the Adams model. The IRS concept is compatible with this model. Each domino represents a component of the IRS.

The Adams model suggests that:

- Failures in the senior management structure (carrying out the health and safety responsibilities of the employer) produce an inadequate health and safety environment (so called root causes of accidents).

- These structural failures lead to inadequate health and safety behaviour by middle management.

- In turn, this causes inadequate health and safety performance by supervisors.

- This creates unsafe activities and conditions at the level of the worker (so called shop floor errors or direct causes).

- Shop floor errors lead to accidents or dangerous occurrences (called near misses).

- These produce human and economic loss.

The falling domino sequence is illustrated below.

The dominos cans begin falling at any point. For example, failures at the middle management level can cause problems in supervisory behaviour, eventually leading to an accident.

The role of each “domino” in preventing accidents and illnesses in the IRS is discussed in the following sections.

#### Management structure

This represents the commitment and role of the employer, directors, owners and senior officers in:

- Establishing and communicating a commitment from the employer that health and safety is a top priority.

- Setting goals for the health and safety program/policy by:

- Setting objectives to help the organization evaluate its program

- Establishing clear standards of performance

- Holding subordinates accountable for their performance

- Setting rewards for effective performance

- Establishing an effective system to implement the desired performance by:

- Setting up an effective chain of command

- Providing adequate operating authority and responsibility

- Providing adequate delegation of responsibility

- Hiring and training competent staff

- Establish a policy/system to ensure the workplace is designed and maintained properly by:

  - Properly engineering operations, workplace layout and production (through suitable ergonomics, lighting, ventilation, temperature control and so on)

  - Ensuring health and safety is a major consideration in equipment purchasing and general procurement

  - Implementing effective orientation and training programs

  - Involving workers in developing the program through the occupational health committee or worker health and safety representative (representative)

  - Establishing an effective occupational health committee or designating a capable representative

  - Providing a work environment free of harassment

  - Knowing and complying with the legislation

- Accountability is proportional to responsibility. The employer is ultimately responsible and accountable for health and safety.

#### Manager behaviour

Manager behaviour refers to the role of middle management in allocating adequate resources for health and safety. It also describes their responsibilities for ensuring that supervisors under them carry out their duties properly. Managers can help the employer to establish a health and safety culture by:

Following the policies of the employer and ensuring supervisors also follow them

Establishing goals that will help implement the employer’s policies and programs in the local work area

Developing and enforcing safe and healthy work procedures

Allocating adequate resources for health and safety

Ensuring supervisors are properly selected and trained in their responsibilities

Providing supervisors with the necessary authority and responsibility for health and safety

Holding supervisors accountable for their performance

Ensuring there are enough supervisors to effectively supervise workers and control hazards

Assisting the local committee to carry out its duties effectively

- Knowing and complying with the legislation

- Scheduling (such as shifts and work processes) with safety in mind

As representatives of the employer, the law holds managers and supervisors accountable for their health and safety actions. Sometimes, someone with a higher responsibility fails to carry out a duty. Under the legislation, this does not relieve others of their duties.

#### Supervisor behaviour

In the legislation, the definition of supervisor includes any worker who is given authority over other workers. While supervisors have all of the rights and duties of workers, they also share some of the employer’s responsibilities.

Supervisors are critical to effective health and safety practices. Supervisors can help the employer keep the workplace healthy and safe by:

Taking training in their health and safety duties and responsibilities

Establishing goals that help implement the employer’s program in the local work area

Setting an example by working and acting safely

Following the policy of the employer and ensuring workers also follow it

Allocating adequate resources for health and safety

Conducting regular inspections, group meetings and other health and safety activities

Ensuring workers are properly orientated and trained

Coaching workers in health and safety

Establishing and enforcing safe work procedures, rules and practices

Providing workers with the necessary authority and responsibility for health and safety

Holding workers accountable for their performance

Identifying the reasons for accidents, dangerous occurrences and other departures from safe practices and taking corrective action

Responding to workers’ concerns promptly and creating an environment where workers are encouraged to bring concerns forward

Cooperating with the occupational health committee

Knowing and complying with legislation

#### Shop floor errors

Shop floor errors refer to unsafe acts and conditions that directly cause accidents and dangerous occurrences. Shop floor errors are usually at the level of the worker, but can involve self-employed persons and others. While shop floor errors often arise because of systemic failures in the IRS, they are sometimes caused by the behaviour or decisions of those performing the job. The IRS can control many aspects of worker behaviour, but it cannot control all behaviour.

#### Accident/near miss

Accidents cause injury or illness and often result in property loss. The seriousness of the loss and injury is often a matter of chance. A “near miss” is a near accident. Near misses may or may not cause property loss.

Accidents and near misses often have several causes. Several things go wrong at once. Each error may be traced to a variety of root causes. For example, an employer may have no health and safety policy, or the policy may have inadequate standards. Supervisors may tolerate unsafe conditions and work practices. Workers may not report hazards and “cut corners” to increase production. When something goes wrong, the causes can usually be traced back through each “domino” in the chain of command.

While the employer usually controls the root causes, workers and supervisors often control the direct causes. Therefore, to significantly improve the health and safety system and prevent the dominos from falling, everyone must work together. This is why occupational health and safety is important to everyone and why employers and workers need each other’s help to prevent accidents. Accident prevention starts with an effective internal responsibility system for occupational health and safety.

## The internal responsibility system for occupational health and safety

The Occupational Health and Safety Act supports every workers’ right to a safe and healthy workplace. The duty for creating and maintaining a healthy and safe workplace falls on every person in the workplace, to the degree they have the authority and ability to do so. Whether they are the Chief Executive Officer or the newest worker hired, everyone has a personal and shared responsibility for working together co-operatively to prevent occupational injuries and illnesses.

Because employers have the greatest degree of control over the workplace, they also have the greatest degree of legal responsibility for health and safety. But, this does not relieve supervisors and workers from their duty to co-operate in controlling workplace hazards and to take the necessary precautions to protect themselves and others from hazards.

The Act also recognizes that only workers who are adequately informed and empowered can effectively fulfill their responsibilities. It grants three important rights to workers:

- The right to know about workplace hazards, including how to identify hazards and

protect themselves from those hazards; and about the rights afforded to workers

under the Act.

- The right to participate in decisions related to occupational health and safety, free of reprisal for their participation. Participation, in part, is achieved through consultation with the committee or representative.

- The right to refuse unusually dangerous work.

The Act protects these rights by prohibiting employers from imposing discipline or other sanction on workers for fulfilling their responsibilities or exercising their rights. This helps workers participate on a more even footing with employers and supervisors in preventing workplace injuries and illness.

Taken together, these components are often called the internal responsibility system (IRS) for occupational health and safety. But, good health and safety cannot rely on the internal responsibility system alone. Ongoing monitoring and enforcement by the Occupaitonal Health and Safety Division are also required.

The combination of internal monitoring by occupational health committees, and external monitoring and enforcement by the Occupational Health and Safety Division ensures better legislative compliance and a more effective internal responsibility system in the workplace.

### What is the goal of an internal responsibility system?

The goal of an internal responsibility system is to get people working together to identify and control situations (hazards) that could cause harm. Its ultimate objective is to ensure everyone integrates health and safety into their work. Committees and representatives help workers and employers to communicate and work together to identify and control hazards.

Internal responsibility has many advantages:

- it places responsibility for controlling hazards on those in the workplace, making
everyone a contributor to workplace health and safety

- it applies everyone’s knowledge to improve health and safety

- it is better suited to developing solutions for each workplace than traditional government-driven “command and control” systems

- it encourages employers and workers to take joint action to identify and control hazards through co-management of health and safety

- properly handled, it promotes cooperation and motivates everyone to protect their health and safety and that of their fellow workers

### What is the philosophy of internal responsibility?

The philosophy of internal responsibility recognizes that employers have a legal and moral duty to provide a healthy and safe workplace. Worker participation is crucial to effective health and safety, because only worker participation allows hazards to be properly identified and controlled. The philosophy of internal responsibility is based on:

personal responsibility

inclusiveness

cooperation

sharing of information

accountability

#### Personal responsibility

Each individual is responsible for integrating health and safety into work life. Health and safety is everyone’s responsibility. The committee is not the IRS—it is the system’s auditor.

#### Inclusiveness

Everyone is involved in the IRS, both as individuals and as members of a group (workers, managers and committee members). Shared responsibilities complement each other.

#### Cooperation

Everyone’s objective should be the same—to improve health and safety. Workers and employers are expected to work together to identify and resolve health and safety problems. The committee functions as a health and safety team. Differences of opinion and position are expected. However, if the approach taken is unduly confrontational or uncooperative, then the committee may not be effective.

#### Sharing of information (the right to know)

Everyone has a right to receive information needed to identify and control hazards in the workplace. Information is needed before each person can adequately assume responsibility for health and safety. Good information is particularly important for occupational health committee members.

| For accountability | For conditions of work | For facilities and equipment | For relations with people | For direction of work | For work performance | For people | For work | Responsibilities | Internal Responsibility for Health and Safety |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspect tools and equipment. Report hazards to supervisor | Implement and maintain standards. Cooperate with committee | Safely use tools, equipment and machinery | Follow policies, programs and procedures | Follow safe work practices and cooperate with supervisor | Use training, knowledge and skills to perform work | Direct helpers/new hires | Perform job | Workers |  |
| Inspect work areas, tools, equipment and machinery. Report problems to managers and recommend solutions | Implement standards and train workers. Cooperate with committee | Provide adequate tools, equipment and machinery | Coordinate implementation of programs, policies and procedures on shop floor | Follow safety policies and programs | Specify who does what and assign authority | Orientate and train new hires | Assign tasks and schedule work | Supervisors |  |
| Develop effective solutions to problems. Accountable to employer for operations | Help employer develop standards. Train supervisors to implement standards. Help committee to be effective | Provide supervisors with adequate resources | Implement policies. Conduct daily business in compliance with employer’s policies and legislation | Implement safety policies and programs | Assign jobs to supervisors and delegate authority | Select and develop supervisors | Determine objectives | Managers |  |
| Account to directors and society for safe operation of work | Determine health and safety philosophy and policies. Maintain effectiveness of committee | Authorize expenditures and assign adequate resources to managers | Determine policies, procedures and programs and ensure compliance | Establish safety policies, programs and procedures | Determine who does what and delegate authority | Establish hiring policies. Select and develop managers | Establish goals and objectives | Employer |  |

#### Accountability

Our responsibility for maintaining workplace health and safety comes with our right to a healthy and safe workplace. Everyone is accountable (as an individual) for carrying out their responsibilities. The greater the authority, the greater the accountability. An effective IRS requires competent management and supervision, employees who work and act safely and a committee that is effective.

External auditing function of Saskatchewan Labour occupational health officers

### What are the components of an effective IRS?

Outer ring – This is the external auditing function of occupational health officers. Officers monitor the effectiveness of the IRS and help correct deficiencies through education, assistance and enforcement.

Inner ring – The role of the OHC as an internal auditor of the IRS.

Small circles – The duties and rights of those in the workplace and the role they play in maintaining the IRS. These rings support and complement each other.

## Direct responsibility for occupational health and safety

In Saskatchewan, The Occupational Health and Safety Act, 1993 and regulations assign duties for the control of workplace hazards to those people who are directly responsible for:

the organization of work

the design of the work process

the manner and conditions under which work is performed

In simple terms, employers, supervisors and workers all have responsibilities for:

identification of hazards

analysis of the risks posed by those hazards

communication of information throughout the workplace about the risks

control of those risks

The employer’s responsibilities are the broadest and cover all areas of health and safety in the organization. Usually, managers are responsible for the health and safety of all employees who report to them. Supervisors are responsible for the health and safety of their workers in all areas where they work. Workers are responsible for protecting their health and safety and helping supervisors ensure the safety of their work areas, tools, equipment and machinery.

Others with direct responsibilities include:

self-employed persons (including consultants)

contractors (someone who directs the activities of one or more self-employed persons or employers)

owners

suppliers

### What are the legislated duties of employers, supervisors and workers in the IRS?

| Employers | Supervisors | Workers |
| --- | --- | --- |
| - Providing a safe and healthy workplace. - Establishing a committee or designating a representative as needed and ensuring the committee or representative is effective. - Consulting and cooperating with the committee or representative. - Providing the information and training needed to protect the health and safety of workers. - Ensuring committee members or the representative to obtain training in their duties. - Ensuring that supervisors are competent and that supervisors provide competent supervision. - Arranging for the regular examination of the workplace, tools and equipment to ensure safety. - Ensuring tools, machinery and equipment are properly maintained. - Ensuring workers are not exposed to harassment. - Enabling the committee or representative to inspect the workplace regularly. - Promptly correcting unsafe conditions and activities reported by the committee or representative. - Ensuring that the committee or representative investigates reportable accidents and dangerous occurrences. - Knowing and complying with health and safety requirements. | Supervisors are "workers" and bear all of the health and safety responsibilities assigned to workers. However, they are also agents of the employer and are often assigned significant responsibility for carrying out the employer's duties under the legislation. Supervisors have specific duties under the regulations for: - Ensuring that workers under their direction know and comply with health and safety requirements. - Ensuring that workers under their direction receive adequate supervision. - Ensuring that workers know and follow health and safety requirements. - Cooperating with the committee or representative. - Knowing and complying with health and safety requirements. | - Taking reasonable care to protect their health and safety and that of other workers. - Cooperating with employers, supervisors, committee members and representatives. - Using safe work procedures, safeguards and personal protective equipment. - Reporting hazards (such as unsafe situations and activities) to the employer immediately. - Refraining from harassment. - Participating in training and health and safety meetings. - Knowing and complying with health and safety requirements. |

### What is due diligence?

The legislation can’t cover everything that can happen on the job. Sometimes you must take measures to protect your workers and service providers that go beyond the legislation. This is what is meant by “due diligence”. What you must do to provide a healthy and safe workplace depends on the circumstances, the risks, the opportunities for control, and so forth. The Act requires you to think, be engaged, creative, plan ahead and do everything reasonably practicable to prevent accidents.1

For information about how to show due diligence, see:

	Elements of An Occupational Health and Safety Program; and

	Setting Up An Occupational Health and Safety Program, A Guide.

Both are available from the division in Regina or Saskatoon at no charge.

Due diligence means that a person has a legal duty to take every precaution reasonably in the circumstances to avoid both harm and an offence against the law.

Due diligence describes a very high standard to take reasonable care.  In the context of the Act and regulations, this due diligence standard is reflected in the following principles:

- General duties – The Act imposes a duty on everyone in the workplace to take reasonable care of their health and safety and that of others, to the degree that they have the authority and ability to do so. This general duty is in addition to, and goes far beyond, merely complying with the law.

- Regulatory compliance – If someone is charged with contravening the legislation, they cannot successfully defend themselves by saying that they did not intend to break the law or did not mean to fail to comply. To defend themselves adequately, they must be able to show that they took every reasonably practicable action to ensure compliance.

- Reasonably practicable – One must show that they took every possible precaution, unless they can show that the benefits of taking the precaution are greatly exceeded by the cost in time, trouble, and money. The greater the risk, the greater the health and safety measures required.

- Proactive – Due diligence requires a proactive and systematic approach to health and safety. This standard can best be met within a workplace by establishing and implementing a health and safety program that:

- identifies hazards

- assesses the risks associated with those hazards

- implements measures to eliminate or minimise those risks

- monitors each part of the program to ensure that it is adequate and effective

Employers must develop and implement this plan in consultation with their workers. Workers must comply with the program to the extent that they have the knowledge, authority, and ability to do so.

### What is shared responsibility for health and safety?

Responsibility can be shared among the workplace parties, such as the employer, supervisors and contractors. This is expressed in clauses 5-9 of regulation 5.

(5)	Where a provision of these regulations imposes a duty or requirement on more than one person, the duty or requirement is meant to be imposed primarily on the person with the greatest degree of control over the matters that are the subject of the duty or requirement.

(6)	Notwithstanding subsection (5) but subject to subsection (8), if the person with the greatest degree of control fails to comply with a provision described in subsection (5), the other persons are not relieved of the obligation to comply with the provision if it is possible for them to comply, and they shall comply with the provision.

If the person with the greatest degree of control complies with a provision described in subsection (5), the other persons are relieved of the obligation to comply with the provision:

(a)	only for the time in which the person with the greatest degree of control is in compliance with the provision;

(b)	only if simultaneous compliance by more than one person would result in unnecessary duplication of effort and expense; and

(c)	only if the health and safety of workers is not put at risk by compliance by only one person.

(8)	If the person with the greatest degree of control fails to comply with a provision described in subsection (5) but one of the other persons complies with the provision, the other persons, if any, to whom the provision applies are relieved of the obligation to comply with the provision in the circumstances set out in clauses (7)(a) to (c), with any necessary modification.

(9)	Where a provision of these regulations imposes a duty or requirement on a person to ensure that another person carries out or refrains from carrying out a specified action, the person on whom the duty or requirement is placed has complied with the provision if that person establishes that he or she took all reasonable steps to ensure that the second person carried out or refrained from carrying out the specified act.

In this case, the person with the greatest degree of control over the situation has the greatest responsibility. Employers exercise the greatest degree of control over the organization of work, the design of the work process and the manner and conditions under which work is performed. The employer and his or her agents therefore have the greatest degree of moral and legal responsibility for providing a healthy and safe workplace. However, supervisors and workers must help the employer maintain a healthy and safe workplace.

### Worker rights

Recognizing that power is not shared equally among employers, supervisors and workers, the Act sets out three basic rights of workers within the IRS:

- The right to know about the hazards in the workplace, how to identify them and how to protect themselves.

- The right to participate in making occupational health and safety decisions through consultation with committees, representatives and other workers.

- The right to refuse work believed by the refusing worker to be unusually dangerous.

It also prohibits employers from acting against a worker for fulfilling responsibilities or exercising rights under the legislation. This protection is fundamental to making the IRS work. Protection motivates and empowers workers to fulfil their legislative duties and participate in a meaningful way in the control of workplace hazards. In this way it encourages workers to help the employer provide a safe and healthy workplace.

In short, the IRS sets up a system of checks and balances within the workplace that enables workers, supervisors and employers to integrate the best possible health and safety practices into daily activities.

## Critical role of committees in the IRS

Committees have been required in all workplaces with ten or more workers since 1972. Now, smaller, high-hazard workplaces must have a worker occupational health and safety representative.

The Division interprets “consult” to mean that, while not obliged to obtain the approval or permission of the representative, an employer is obliged to consider, in good faith, the news and opinions of the committee or representatives in the process of making a decision.

Employers have a duty to consult and co-operate with the OHC representative for the purpose of resolving concerns on matters of health, safety and welfare at work.

To fulfill the obligation to consult in good faith, an employer must give the committee or representative a real opportunity:

To be informed of information essential to making a reasonable and informed assessment;

To review and assess the information and possible alternatives or options;

To comment and, or make recommendations on the possible options and alternatives; and

To be considered. This means the employer will consider the recommendations of the committee or representative and where applicable, give the committee or representative credible reasons for not accepting or implementing the committee or representative’s recommendations.

Committees and representatives don’t have direct responsibility (and the accompanying legal liability) for controlling hazards. Their role is to monitor the IRS to ensure it is working properly. They have no management authority. Their role is to advise and recommend.

Effective committees and representatives are essential to creating the workplace partnership needed to effectively protect workers’ health and safety. By consulting with committees, representatives and workers, employers benefit from workers’ expertise in detecting occupational health and safety hazards and developing practical approaches to controlling those hazards.

The committee and representative are the mechanism for bringing health and safety concerns into the open and focusing attention on them until they are resolved. Committees and representatives serve as a check on the potential for some employers and supervisors to ignore or minimize their health and safety responsibilities.

Committees and representatives shine a spotlight on weaknesses in the IRS and recommend ways of correcting those weaknesses. They provide a bridge to the external responsibility system when assistance and advice are needed.

Committees and representatives must do more than simply help the employer correct unsafe acts and conditions. They should look at the root causes for hazards (such as policies, orientation, training, workplace layout and so on) and help the employer resolve them.

### What are the major responsibilities of committees?

- Major responsibilities include:

- participating in the identification and control of hazards

- helping identify and resolve health and safety concerns of workers

- receiving and distributing information, including publications sent from the OH&S Division to employers and employees

- inspecting the workplace regularly

- meeting regularly to discuss occupational health and safety concerns

- maintaining records of meetings and returning copies to the OH&S Division

- investigating reportable accidents and dangerous occurrences

- helping establish and promote health and safety programs and policies

- helping develop and promote health and safety training

- investigating refusals to work under section 23 of the Act

- carrying out responsibilities set by specific regulations

- If the committee does not deal with a problem, the employer is responsible for taking the initiative.

## Role of the legislation in supporting internal responsibility

The original Saskatchewan Occupational Health and Safety Act became law in 1972. It was the first legislation of its kind in North America. Every other jurisdiction in the country has since followed our lead. The Act was revised in 1977 and in 1993 to strengthen worker involvement and to better protect their health and safety.

The Occupational Health and Safety Regulations were passed under authority of the Act to:

elaborate on duties established by the Act

specify responsibilities for those within the IRS

set health and safety standards for all places of employment

set rules and requirements for controlling hazards in specific industries, processes or conditions

The regulations were revised in 1996 to reflect changes in the Act and advances in health and safety standards.

The legislation has one aim—to help those in the workplace to establish and maintain healthy and safe working conditions. It does this by:

- ensuring societal expectations are reflected in workplace behaviour and practices

- providing a way of protecting the health and safety of workers

- supporting the internal responsibility system

- providing employers and workers with information and tools to carry out their health and safety responsibilities

- ensuring safety-conscious, responsible employers are not at a competitive disadvantage to employers who are less so

## Role of the Occupational Health and Safety (OH&S) Division

The OH&S Division’s fundamental objective is to prevent work-related injuries and illnesses. It attempts to do this by assisting committees and representatives, workers, supervisors and employers to understand and carry out their responsibility for the control of workplace hazards.

Ensuring compliance is the most important responsibility of the OH&S Division’s occupational health officers (inspectors). However, inspectors also help ensure each workplace’s IRS, committee or representative is effective by providing assistance and advice.

Inspectors, hygienists and other staff are responsible for over 40,000 workplaces. Many workplaces are complex and have a variety of hazards. Those hazards cannot be controlled solely through periodic government inspections. When inspectors find problems, it is a sign that the IRS is not working properly.

When the IRS is working properly, the enforcement role of inspectors lessens. Their role becomes one of consulting, advising and supporting the IRS.

Over the past 25 years, the OH&S Division has learned that effective committees are much more effective in focusing attention on health and safety concerns than periodic inspections. Work-related injuries and illnesses can be prevented when workers understand their rights and responsibilities and employers are genuinely committed to protecting the health and safety of their employees.

### How does the IRS link up with the OH&S Division?

- Routine inspections – When an inspector visits the workplace, a member of the committee (or a worker health and safety representative) should accompany the inspector. The committee member or representative can ask the officer for advice about concerns at the workplace. In turn, the officer provides the committee with a report on the results of the inspection.

- Complaints – Anyone can contact the OH&S Division for assistance and advice if the IRS has not dealt with a concern effectively. The legislation protects those who call.

- Refusals – Often, a refusal is a sign of a failure in the IRS. Refusals can usually be prevented by paying enough attention to job safety and to the concerns of workers.

Refusals are often resolved by the worker and supervisor involved. Some are resolved by the committee. During a refusal to work, contact the OH&S Division if the matter has not been dropped or resolved, the refusing worker is not satisfied with the committee’s decision, or the committee cannot agree on the refusal. In such cases, an occupational health officer must investigate and make a ruling. The worker may continue the refusal until the officer rules. The officer will provide a written decision to the refusing worker, each co-chairperson and the employer. Part VIII of the Act allows anyone directly affected by an officer’s decision to appeal it to the Director of the OH&S Division. The appeal does not suspend the operation of the officer’s decision.

- Investigations – Employers must notify the OH&S Division about accidents and dangerous occurrences set out in the regulations. If an inspector investigates, the committee or representative has an opportunity to inform the officer about concerns. The officer will provide a report to the employer and committee or representative.

- Failure of the committee – If the committee cannot function effectively, an inspector should be asked for help. Committee minutes sent to the OH&S Division often provide the first clue that things are not going well. A dysfunctional committee is a sign of deeper problems with the IRS. An inspector may intervene.

- Disputes – An inspector may intervene to resolve conflict in the committee. If necessary, an officer can issue a notice of contravention requiring the employer (or anyone else covered by the legislation) to take corrective action.

## Review

Occupational health and safety includes physical and mental health factors as well as social well being. The goal of an effective IRS is to get everyone involved in identifying and controlling hazards before they harm a worker. Worker involvement through occupational health committees or representatives is crucial in making this happen.

Modern accident theory suggests that fundamental failures in the work environment cause most accidents. When something goes wrong, problems can be found at every level in the chain of command. While the employer often controls the root causes, workers and supervisors control the direct causes for accidents. In order to control both root and direct causes, employers and workers must work together within the internal responsibility system.

Under the IRS everyone has responsibilities and accountabilities for occupational health and safety. The degree of accountability is proportional to the degree of responsibility. While the employer retains ultimate control, only worker participation is effective in systematically identifying and controlling hazards. Committees and representatives perform a critical function by serving as means of communication and cooperation between the employer, workers and the OH&S Division.

Legislation sets up a framework to support the IRS and maintain fundamental health and safety standards within the workplace. Occupational health officers monitor the effectiveness of each organization’s IRS and provide assistance and advice. Enforcement of the legislation is a last resort.

Identifying, assessing and controlling hazards

- Definitions

- Principles of hazard identification, assessment and control

- Identifying and assessing health hazards

- Identifying and assessing safety hazards

- Controlling hazards

Introduction

Helping the employer to identify, assess and control hazards is one of the most important functions of your occupational health committee. This chapter discusses how your committee can help the employer protect the health and safety of workers within the internal responsibility system. We will begin by defining a few important terms.

Definitions

What is a hazard?

A hazard is any activity, situation or substance that can cause harm. Occupational hazards are divided into two broad categories: (1) health hazards, and (2) safety hazards. Generally, health hazards cause occupational illnesses, such as noise induced hearing loss (NIHL). Safety hazards cause physical harm, such as cuts, broken bones and so forth. Hazards exist in all workplaces.

What is an occupational illness?

Occupational illnesses are usually caused by health hazards. An occupational illness is a condition that results from exposure to a chemical or biological substance, a physical agent (an energy source such as noise) or other stressors (such as harassment, work demands and so forth) capable of causing harm. The time that it takes an illness to develop after exposure to a health hazard is called the “latency period”.

What is probability?

Probability is the chance that a hazard will cause harm. In risk management systems, probability is often categorized as:

- frequent (workers are frequently at risk)

- probable (the hazard is likely to cause harm)

- occasional (workers are occasionally at risk)

- remote (the hazard could cause harm, but is very unlikely to do so)

- improbable (the hazard is unlikely to ever cause harm)

What is severity?

Severity is the seriousness of the harm that could result from contact with a hazard. It is described as:

- catastrophic (death and/or severe destruction)

- critical (serious injury and/or property damage)

- marginal (minor injury and/or property damage)

- negligible (no injury and/or property damage)

What is a risk?

Risk describes the odds that a hazard will cause harm. It refers to the probability and severity of potential accidents and dangerous occurrences (so called "near misses"). Risk management is a technique used to identify and control risk caused by hazards.

What is risk analysis?

The combination of identifying hazards and assessing their risk is called risk analysis. Risk analysis can help committee members and the employer to set priorities. Risk is calculated by using the formula: Risk = Probability x Severity.

Several commercial systems assign mathematical values to probability and severity to help calculate risk ratios for hazards. Normally, hazards with the highest risk that affect the most workers should receive the greatest attention.

Principles of hazard identification, assessment and control

The health and safety of workers depends on the committee, employer and workers working together to identify, assess and control hazards. This does not mean that the committee must be involved in everything.

Ideally, the IRS should prevent a hazard from becoming a concern. The role of the committee should be to audit the IRS and help the employer keep it functioning properly. However, if a hazard is reported to the committee, the committee must act. Use the following steps to identify, assess and control hazards:

- collect information about hazards and potential hazards

- assess the risk

- set priorities

- communicate information about the hazards and risks

- develop, select and implement controls and monitor their effectiveness

Collect information

Collect information from sources such as:

- Workers, supervisors and managers – Workers often know, or suspect, what hazards exist and where they are located.

- Associations – Many provide training and can recommend appropriate publications from the Canadian Centre for Occupational Health and Safety, the U.S. National Safety Council and others.

- Suppliers and manufacturers – Equipment manuals, users' guides, hazard warnings and so forth are a first line of defense.

- Occupational health officers – Officers can provide technical advice. The OH&S Division has a library of safety videotapes.

- Legislation – The regulations and related codes of practice are an excellent guide to identifying and controlling hazards.

- Unions – Many unions provide health and safety training and information about hazards to their members.

Assess the risk

Once a hazard is identified, your committee should help the employer to determine how dangerous it could be. Assessment may involve research and monitoring. For example, suppose a noise hazard is discovered. The next step would be to use monitoring equipment (noise meters) to find out how loud the noise is, where and when the noise is a problem, how long the noise is at an unacceptable level and so on.

Look for any factors that could contribute to the hazard. For example, consider work processes, work process design, existing hazard controls, related training and so forth. In the case of a noise hazard, the problem may be made worse by such things as metal on metal contact, equipment vibration, inadequate work practices, etc.

Assess the risk posed by each hazard. Ask these questions:

- How likely is the hazard to cause harm?

- Under what conditions is harm likely to occur?

- How quickly could an unsafe condition arise?

- What type of harm is involved?

- How many workers could be hurt?

- Is there a history of problems, accidents or dangerous occurrences resulting from this hazard?

- What monitoring is needed to evaluate the risk?

Set priorities

Priorities can be set by using the formula (Risk = Probability x Severity). Factors such as the limits of technology, fiscal resources and potential problems raised by hazard controls will have to be considered. The following table illustrates one way of assigning probability and severity values. Adapt it to suit your needs.

Communicate information

Workers must know about the hazards in the workplace in order to protect themselves. Communicating hazard information to workers and the employer is one of the most important functions of a committee or representative. Committees can help the employer communicate hazard information. To do this, committee members should:

- Post information such as: minutes of committee meetings and meetings with the employer; the results of inspections; summaries of workplace monitoring; investigations; warning signs; and hazard labels.

- Help the employer to provide health and safety information to supervisors. Supervisors can then provide it to their workers.

- Discuss hazards with workers, supervisors and managers.

- Hold meetings to discuss concerns.

- Help the employer to arrange worker training and education.

- Help the employer to keep containers for chemical and biological substances properly labeled.

- Help the employer to keep material safety data sheets (MSDSs) current, centrally located and readily available to workers.

- Keep workers who raise concerns informed about the status of the investigation.

Develop, select and implement controls

Risk assessment is meaningless unless effective controls are developed and put in place. Control means removing the hazard or reducing its risk of harm to an acceptably safe level. An effective control must meet four standards:

- It must adequately prevent the hazard from causing harm.

- It must protect everyone who could be harmed by the hazard.

- It must not create new hazards, or production and quality control problems. (If it does, employees may be tempted to subvert it).

- It must not create a hazard to the environment or public outside of the workplace.

Identifying and assessing health hazards

A health hazard is any agent, situation or condition that can cause an occupational illness. There are five types:

- Chemical hazards, such as battery acid and solvents.

- Biological hazards, such as bacteria, viruses, dusts and molds. Biological hazards are often called "biohazards".

- Physical agents (energy sources) strong enough to harm the body, such as electric currents, heat, light, vibration, noise and radiation.

- Work design (ergonomic) hazards.

- Workplace stress. In this publication, this is restricted to harassment as defined under the Act.

A health hazard may produce serious and immediate (acute) affects or it may cause long-term (chronic) problems. All or part of the body may be affected. Someone with an occupational illness may not recognize the symptoms immediately. For example, noise-induced hearing loss is often difficult for victims to detect until it is advanced.

How can health hazards be identified?

- Prepare a list of known health hazards in the workplace.

- Review floor plans and work process diagrams to identify hazard sources and locations.

- Interview workers, supervisors and managers to identify known and suspected health hazards not already on the list.

- Use your five senses.

Part VI (General Health Requirements) of the regulations discusses how to identify, assess and control health hazards. Parts XXI and XXII deals with how to identify, assess and control hazards posed by chemical and biological substances.

Prepare a list of known health hazards in the workplace

As a first step, the committee should help the employer prepare a current list of chemical and biological substances, physical agents, work design hazards and harassment problems at the workplace. The employer is expected to provide the committee with a copy of existing lists of chemical and biological substances. To prepare or update a list:

- Check current product labels and material safety data sheets (MSDSs). Each chemical and biological substance controlled under the Workplace Hazardous Materials Information System (WHMIS) in the workplace must have appropriate container labels. Current MSDSs must be readily available to the committee and workers. Look at container labels and MSDSs for hazard warnings and symbols (such as the skull and cross bones). Identify substances controlled under WHMIS and products exempted from WHMIS, but of concern to workers (such as pesticides).

- Conduct inspections to identify defects, such as substance containers and pipes that are not properly labeled.

- Read inspection and accident reports, complaint files, shop plans, first aid registers, product literature and other documents.

- Monitor the workplace (measuring noise, temperatures, concentrations of airborne chemicals and so forth). The employer must provide the committee with the results of any measurements of biological and chemical substances taken in the workplace.

Review floor plans and work process diagrams

Floor plans may show for example, that certain points in the production line release chemicals into the air. Put monitoring equipment at these locations to determine what hazardous substances are present and in what quantities. Check for work design problems that may cause back injury and other ergonomic hazards. Look for tasks associated with accidents, complaints and ill health.

Interview workers, supervisors and managers

Interview workers, supervisors and managers during inspections. Ask them what hazards they work with and what work-related health problems they know about. However, deal with the concerns of workers at any time, not just during inspections.

Talk to the vendor or supplier if you need more information about a specific product, tool, or piece of equipment. Contact the OH&S Division if you need more information about specific hazards.

Use your five senses

Some substances and physical agents can be detected with your five senses. For example, dusts and fumes sometimes form a haze. Vibration and temperature can be felt. An abnormal taste may be a sign of airborne chemicals. Some substances have a distinct colour, visual appearance or odour.

Odour is a common warning property. Be careful to check the substance’s odour threshold in the Physical Properties section of its MSDS. Only use odour to detect a substance if it can be smelled at levels below hazardous concentrations.

Unfortunately, many hazardous agents and conditions cannot be detected with the senses. Others, such as hydrogen sulfide (H2S) gas, are often dangerous when strong enough to be detected in this way. Using your senses is not always a safe way of detecting hazards.

##### Quick health hazard identification checklist - What chemical substances are produced, used, handled, stored or shipped in the workplace? - Are any vapours, gasses, dusts, mists or fumes present (including chemical by-products of work processes)? - Are biological substances (such as bacteria, viruses, parasites, dusts, molds and fungi) present in the workplace, the ventilation systems and other components of the physical plant? - Are physical agents (energy sources strong enough to harm the body, such as electric currents, heat, light, vibration, noise and radiation) present? - Are temperature extremes present? - Do ergonomic hazards exist—such as work requiring lifting, awkward posture, repetitive motions, excessive muscular force or computer use? - Could any work processes, tools or equipment cause health hazards (such as back injuries, soft tissue injuries, whole body vibration, hearing loss, infections and so forth)? - Could departures from safe work practices cause illnesses? - Can any potential health hazards be detected with the senses (smell, taste, touch, hearing and sight)? - Is harassment present in the workplace? - Are there any complaints from workers about workplace related health problems?

How can health hazards be assessed?

Once a health hazard is identified, the risk it poses to workers must be assessed. Committees can help the employer do this by using monitoring equipment to assess exposure levels and by determining the probability and severity of any potential exposure.

There are many different monitors for detecting and assessing health hazards. Some, such as air monitors, sample the work environment at specific places for specific chemical hazards. Others measure the levels of noise, vibration and so on. Committees can get advice on how to use monitors from the supplier. Advice on how to interpret monitoring results can be obtained from consultants and agencies.

Chemical hazards

If possible, use monitoring equipment to determine exactly what the exposure levels for health hazards are in the workplace and at workstations. Different hazards require different monitoring techniques and equipment. The employer may decide to bring in experts to do the monitoring. If so, the committee or representative has a right to be involved and informed of the results.

Once an exposure level is determined, compare it with standards set by the organization, industry, legislation and so on. Review MSDSs as well as industry and product literature for advice. (See Table 21 of the Appendix to the regulations for chemical contamination limits).

Biological hazards

Some biological hazards can be detected by monitoring. However, the risk of catching an illness can usually be assessed by applying knowledge of the disease involved, including how it spreads and infects people. Biological safety data sheets provide useful information such as survival characteristics of the microorganism outside of the body, how it is transmitted and how likely workers are to contract the disease.

Physical health hazards

Physical health hazards are sources of energy strong enough to cause harm. They include noise, vibration, heat or cold and radiation.

Noise – Common noise sources include equipment, work processes, compressors, ventilation systems and power tools. Generally, if ordinary conversation cannot be understood at normal distances, noise levels are too loud. Hazard identification techniques, such as inspections, monitoring and conversations with workers will usually detect noise concerns.

Vibration – Vibration is a rapid back and forth or up and down motion that may affect all or part of the body. It can gradually damage nerves and circulation systems in limbs and affect internal organs. Standard hazard identification techniques can detect what jobs and workers are exposed to vibration. Monitoring and assessing vibration usually requires technical specialists.

Heat and cold – The health effects of too much heat include heat cramps, heat exhaustion and heat stroke. Cold can produce frostbite and hypothermia. As well as causing serious health problems, heat and cold stress disorders can reduce performance and increase the risk of accidents.

Regulations require the employer to maintain thermal conditions that are reasonable and appropriate for the work performed. If it is not reasonably practicable to adequately control indoor conditions, or where work is done outdoors, the employer must take effective measures to protect workers from heat and cold stress disorders.

The employer must provide suitable monitoring equipment if workers are concerned about thermal conditions. The assessment must consider factors such as temperature, humidity, airflow, wind and work levels.

Radiation – Radiation is made up of moving particles or waves of energy. It is divided into two groups: (1) Ionizing radiation; and (2) Non-ionizing radiation.

Ionizing radiation is given off by decaying radioactive elements, such as uranium. Specialized monitoring equipment is used to measure and assess radiation exposures. Radiation workers are also required to wear badges that measure the radiation dose they receive.

Non-ionizing radiation includes:

- Ultraviolet radiation given off by sun lamps and welding equipment. Ultraviolet radiation can burn the skin and cause eye damage.

- Infrared radiation (radiated heat) used in cooking and warming equipment in food processing and industrial packaging.

- Lasers producing concentrated beams of light used in a variety of commercial, medical and industrial purposes. Care must be taken to ensure lasers are set up properly, adequately shielded and cannot damage the eyes or skin of workers.

- Microwave and high radio frequency radiation used in cooking equipment, radar as well as in high-energy radio transmission and communications equipment. If not properly shielded, some equipment may injure the skin, eyes and other organs.

- Long wave radiation used in radio and other communications equipment. Some equipment can heat the entire body.

Assessment of radiation hazards is often a specialized area. Industries using radioactive substances and radiation emitting devices are governed by The Radiation Health and Safety Act and its regulations.

Physical demands (ergonomic hazards)

Hazards can exist in the design of the workplace, the workstation, tools and equipment and the workflow. Ergonomics is concerned with identifying and controlling these hazards by reducing the physical, environmental and mental stresses associated with a job. It does this by trying to balance the capabilities of the worker with the demands of the job. Ideally, the job should fit the person's mental, physical and psychological characteristics.

Common problems caused by work design hazards include repetitive strain injuries (RSIs), cumulative trauma disorders (CTDs) and musculoskeletal injuries (MSIs), including back injuries. Ergonomic related injuries are the fastest growing occupational health problem.

Examine these factors when assessing the risk of ergonomic hazards:

- The posture a worker must use to do the job (stooping, bending and crouching).

For example, in a static posture, such as when sitting or standing without a break, the muscles are held in a fixed position without movement. Over time, work requiring a static posture can cause health problems. Complaints of back, shoulder and neck pain can indicate static posture problems.

- The muscular force (exertion) required (lifting, pulling, pushing, and twisting).

Muscular force describes the amount of force required to do the work. Consider the weight of the loads or tools involved; the fit of handgrips to the worker; the force required; the muscles used; and the adequacy of work gloves provided.

- The number of repetitive motions needed (frequency, speed, duration and position).

Doing the same job rapidly over and over again can cause injury. Jobs that must be repeated in less than 30 seconds, such as data entry, are classed as highly repetitive.

- The physical condition of the person doing the job.

- Vibration of all or part of the body such as when using jackhammers and chainsaws or when operating mobile equipment.

- Work organization factors such as where, when and how the work is done and at what pace.

Poorly designed tasks can force workers to do too much too fast. This can increase stress and reduce work efficiency, increasing the risk of accidents.

- Work environment problems including vibration, heat, cold and contaminants in the atmosphere.

- Remember that these factors can interact, worsening the situation. A good rule of thumb is: the more awkward or static the posture required by a job; the more excessive the force needed to do the work; and the more repetitive the tasks, then the greater will be the risk of injury.

Factors contributing to ergonomic problems can include:

- Problems in the work environment (light, heat, cold, vibration and so forth) as well as the health of the worker can promote ergonomic health problems.

- Lack of work variation during shifts can prevent workers from being able to rest their muscles adequately.

- Poorly shaped, heavy or vibrating hand tools can encourage workers to grip the tool too hard, reducing blood flow to muscles and increasing fatigue. Bulky or clumsy gloves can do the same thing.

Stress hazards

OH&S legislation deals with stress hazards caused primarily by workplace harassment and shiftwork. Workplace harassment is defined in section 2(1)(l) of The Occupational Health and Safety Act, 1993. Regulation 82 deals with stress caused by shiftwork. The Occupational Health and Safety Division has publications dealing with both harassment and shiftwork.

Workplace harassment – Harassment may seriously harm the health and wellbeing of victims. It can also interfere with efficiency and productivity. Section 3 of the Act requires employers to ensure workers are not exposed to harassment. Section 36 of the regulations requires each employer to implement a policy to protect workers. The regulation lists what a policy statement must include. The policy must be developed with the local committee. The employer must put the policy into practice and post a copy in a conspicuous location in the workplace. The OH&S Division has a bulletin containing a model policy.

To assess the risk of harassment, the employer and committee can:

- Check to ensure the harassment policy has been implemented.

- Check for signs the policy is not taken seriously.

- Look for complaints or concerns from workers.

Shiftwork – Shiftworkers have irregular patterns of eating, sleeping, working and socializing that may lead to health and social problems. Shiftwork can also reduce performance and attentiveness. In turn, this may increase the risk of accidents and injuries. Statistics suggest that certain shiftworkers (such as employees in convenience stores and other workplaces that are open 24 hours a day) are more likely to encounter violent situations when working alone.

The employer and workers can work together to identify and control these hazards. Cooperation is essential. Regulation 82 requires the employer to work with the occupational health committee to:

(a)	assess the risks to the worker's health and safety posed by the work; and

(b)	inform the worker about the nature and extent of the risks and how to eliminate or reduce them.

The committee should also be sensitive to stress caused by such things as workload, the pace of work and so forth. Remember to address only stress concerns relating to health and safety. Use the OH&S Division publication Managing Shiftwork as a guide.

Identifying and assessing safety hazards

A safety hazard is anything that could cause an injury. Unlike the harm caused by many occupational illnesses, an injury caused by a safety hazard (such as a cut or fracture) is usually obvious. Safety hazards cause harm when workplace controls are not adequate. Some examples of safety hazards include:

- slipping/tripping hazards (such as electrical cords across floors)

- fire and explosion hazards

- moving parts of machinery, tools and equipment (such as pinch and nip points)

- work at height (such as work done on scaffolds or ladders)

- ejection of material (such as from molding operations)

- pressure systems (such as steam boilers and pipes)

- vehicles (such as forklifts and trucks)

- lifting and other manual handling operations

- materials falling from height, rolling, shifting or caving-in

- unsafe use of explosives

- violence

- hazards posed by working alone or in isolated workplaces

Standard identification methods can be used on safety hazards. In addition, your committee can encourage the employer to have a job safety analysis (JSA) performed on each dangerous job. A JSA involves breaking down each job into its steps and analyzing the hazards present at each step. Product literature, industry publications and legislation are useful starting points for developing JSAs. Many health and safety publications contain examples of various formats for JSA forms as well as detailed instructions on how to perform a JSA.

Controlling hazards

Ideally, hazards should be controlled by applying modern management principles. Use a comprehensive, proactive system to control hazards rather than a reactive, “piece meal” response to each concern as it arises.

To be proactive, an employer should:

- Establish a health and safety policy, in consultation with the committee, to communicate to employees that the employer is committed to health and safety. Build health and safety into all aspects of the organization such as tendering, purchasing, hiring and so on. Ensure everyone understands health and safety is as important as any other area of the organization.

- Communicate the health and safety policy through the management structure. Ensure everyone understands his or her duties.

- Train managers, supervisors and workers to carry out their responsibilities under the policy.

- Equip managers and supervisors to apply modern management and supervisory practices in their safety responsibilities.

- Administer the policy in the same way that other policies are managed.

Example: managing the safety of controlled products under WHMIS

When managing the safety of a controlled product under the workplace hazardous materials information system (WHMIS), an employer should apply a comprehensive approach to control hazards.

- Review chemical and biological substances in the workplace:

- List all chemical and biological substances in the workplace of concern to workers.

- Identify things such as work areas and production processes where products are stored, handled or produced.

- Determine which substances are controlled products under WHMIS and exempt from WHMIS but of concern to workers.

- Obtain current material safety data sheets (MSDSs) for controlled products and make sure:

- each MSDS is less than three years old and complete

- a mechanism is in place to update MSDSs regularly and as new information becomes available

- transport, handling, storage, disposal practices, etc. for each product meet standards set by product MSDSs

- MSDSs are readily available to workers throughout the organization

- Assess the risk associated with handling, use, storage and disposal of the product. For example, consider its flammability, toxicity, corrosiveness, reactivity and explosiveness. Include emergency response requirements in your assessment. For example, what happens if there is a spill, fire, explosion or other mishap? How will the emergency be handled? What must be done to care for the injured and protect others at risk?

- Determine what measures need to be taken to control those risks and apply the technical steps discussed in this chapter.

- Establish a system to ensure containers are adequately labeled:

- Ensure suppliers attach appropriate supplier labels to their controlled products. Labels should include the hazards of the product, precautionary and first aid measures.

- Replace damaged or missing labels.

- Provide appropriate workplace labels for containers holding a product taken from its original container.

- Develop written work procedures based on the control measures.

- Arrange for the training of workers, in consultation with the committee:

- Train all workers in the general requirements of WHMIS.

- Train workers and supervisors handling controlled products how to identify and control the hazards of the products they use.

- Update workers and supervisors when new information becomes available.

Technical steps in hazard control

As a first step in hazard control, determine if the hazards can be controlled at their source (where the problem is created) through applied engineering. If this does not work, try to put controls between the source and the worker. The closer a control is to the source of the hazard, the better. If this is not possible, hazards must be controlled at the level of the worker. For example, workers can be required to use a specific work procedure to prevent harm.

One type of hazard control may not be completely effective. A combination of several different types of hazard controls often works well. Whatever method is used, the committee or representative should try to find the root cause of each hazard and not simply control the symptoms. For example, it might be better to redesign a work process than simply improve a work procedure. It is better to replace, redesign, isolate or quiet a noisy machine than to issue nearby workers with hearing protectors.

Control at the source

Elimination – First, try eliminating the hazard. Getting rid of a hazardous job, tool, process, machine or substance may be the best way of protecting workers. For example, a salvage firm might decide to stop buying and cutting up scrapped bulk fuel tanks (due to explosion hazards).

Substitution – If elimination is not practical, try replacing hazardous substances with something less dangerous. For example, a hazardous chemical can be replaced with a less hazardous one. A safer work practice can be used. Be sure to also identify, assess and control the hazards of substitutes.

Redesign – Sometimes engineering can be used to redesign the layout of the workplace, workstations, work processes and jobs to prevent ergonomic hazards. For example, containers can be redesigned to be easier to hold and lift. Engineering may be able to improve workplace lighting, ventilation, temperature, process controls and so forth.

Isolation – Isolating, containing or enclosing the hazard is often used to control chemical hazards and biohazards. For example, negative-pressure glove boxes are used in medical labs to isolate biohazards.

Automation – Dangerous processes can sometimes be automated or mechanized. For example, spot welding operations in car plants can be handled by computer-controlled robots. Care must be taken to protect workers from robotic hazards.

Control along the path from the hazard to the worker

Hazards that cannot be isolated, replaced, enclosed or automated can sometimes be removed, blocked, absorbed or diluted before they reach workers. Usually, the further a control keeps hazards away from workers, the more effective it is.

Barriers – A hazard can be blocked. For example, proper equipment guarding can protect workers from contacting moving parts. Screens and barriers can block welding flash from reaching workers. Machinery lockout systems can protect maintenance workers from physical agents such as electricity, heat, pressure and radiation.

Absorption – Baffles can block or absorb noise. Local exhaust ventilation can remove toxic gasses, dusts and fumes where they are produced.

Dilution – Some hazards can be diluted or dissipated. For example, general (dilution) ventilation might dilute the concentration of a hazardous gas with clean, tempered air from the outside. Dilution ventilation is often quite suitable for less toxic products. However, it is not effective for substances that are harmful in low concentrations. It may also spread dusts through the workplace rather than completely removing them.

Control at the level of the worker

Control at the level of the worker usually does not remove the risk posed by a hazard. It only reduces the risk of the hazard injuring the worker and lessens the potential seriousness of an injury. Therefore, most safety experts consider control at the level of the worker to be the least effective means of protecting workers.

Administrative controls – These include introducing new policies, improving work procedures and requiring workers to use specific personal protective equipment and hygiene practices. For example, job rotations and scheduling can reduce the time that workers are exposed to a hazard. Workers can be rotated through jobs requiring repetitive tendon and muscle movements to prevent cumulative trauma injuries. Noisy processes can be scheduled when few workers are in the workplace. Standardized written work procedures can ensure that work is done safely. Employees can be required to use shower and change facilities to prevent absorption of chemical contaminants. The employer is responsible for enforcing administrative controls.

Work procedures, training and supervision – Supervisors can be trained to apply modern safety management and supervisory practices. Workers can be trained to use standardized safe work practices.

The committee or representative should help the employer periodically review and update operating procedures and worker training. Refresher training should be offered periodically. The employer is expected to ensure that employees follow safe work practices.

Emergency planning – Written plans should be in place to handle fires, chemical spills and other emergencies. Workers should be trained to follow these procedures and use appropriate equipment. Refresher training should be provided regularly.

Housekeeping, repair and maintenance programs – Housekeeping includes cleaning, waste disposal and spill cleanup. Tools, equipment and machinery are less likely to cause injury if they are kept clean and well maintained.

Hygiene practices and facilities – Hygiene practices can reduce the risk of toxic materials being absorbed by workers or carried home to their families. Street clothing should be kept in separate lockers to avoid contamination from work clothing. Eating areas can be segregated from work areas. Eating, drinking and smoking should be forbidden in toxic work areas. Where applicable, workers may be required to shower and change clothes at the end of the shift.

Personal protective equipment (PPE) and clothing – PPE and clothing are used when:

other controls aren't feasible, (for example, to protect workers from noise exposure when using chainsaws)

where additional protection is needed

where the task or process is temporary (such as periodic maintenance work)

Personal protective equipment is much less effective than engineering controls since it does not eliminate the hazard. It must be used, properly and consistently to be effective. Awkward or bulky PPE may prevent a worker from working safely. In some cases, PPE can even create hazards, such as heat stress.

The employer must require workers to use PPE wherever its use is prescribed by the regulations or organizational work procedures. Workers must be trained to use, store and maintain their equipment properly. The employer, supervisor and workers must understand the limitations of their personal protective equipment.

Selecting controls

Selecting a control often involves:

- evaluating and selecting temporary and permanent controls

- implementing temporary measures until permanent (engineering) controls can be put in place

- implementing permanent controls when reasonably practicable

For example, suppose a noise hazard is identified. Temporary measures might require workers to use hearing protection. Long term, permanent controls might use engineering to remove or isolate the noise source.

Monitoring the effectiveness of controls

Sometimes hazard controls do not work as well as expected. Therefore, the committee or representative should monitor the effectiveness of the corrective action taken by the employer during inspections and other activities. Ask these questions:

- Have the controls solved the problem?

- Is the risk posed by the original hazard contained?

- Have any new hazards been created?

- Are new hazards appropriately controlled?

- Are monitoring processes adequate?

- Have workers been adequately informed about the situation?

- Have orientation and training programs been modified to deal with the new situation? Are any other measures required?

Document the effectiveness of hazard controls in your committee minutes.

What else can be done?

Once hazards have been identified, assessed and controlled, the employer and committee or representative should work together to develop:

- training programs for workers

- emergency response procedures

- health and safety requirements for contractors

The committee or representative should monitor these activities to ensure they are effective.

Review

The employer is responsible for ensuring that workplace hazards are identified, assessed and appropriately controlled. Workers must be told about the hazards they face and taught how to control them.

The employer is expected to consult and involve the occupational health committee in the hazard control process. Likewise, the committee is expected to help the employer maintain a healthy and safe workplace.

Helping the employer identify, assess and control hazards is one of the most important roles of the committee or representative in the internal responsibility system.

Hazards are broadly divided into two groups: hazards that cause illness (health hazards); and those that cause injury (safety hazards). Hazards can be identified by asking what harm could result if a dangerous tool, process, machine, piece of equipment and so forth, failed.

Health and safety hazards can be controlled at the source, along the path or at the level of the worker. Once controls are in place, they must be checked periodically to make sure they are still working properly.

The committee should audit the hazard controls in the internal responsibility system and help the employer keep them effective.

Notes

The committees’ role in accident prevention

- How committees are expected to function

- Where committees are required

- How committees are structured

- Duties of committees

- Protection for committee members

Introduction

The Occupational Health and Safety Act, 1993 requires employers to do all that is reasonable and practicable to protect the health and safety of workers. This includes providing safe equipment, a safe working environment, adequate supervision, information and training. Managers, supervisors and workers have a responsibility to help the employer carry out these responsibilities.

To maintain an effective internal responsibility system (IRS) for health and safety, workers must be involved in decisions that affect them. Involving workers has been proven to reduce accidents and illnesses in the workplace.

This is why the Act requires the employer in workplaces with ten or more workers to organize an occupational health committee. The committee is to be composed of worker and employer representatives. Its purpose is to help the employer maintain an effective IRS. However, the employer remains ultimately responsible for providing a safe, healthy workplace.

A safety-conscious employer will want to help the committee be effective. Therefore, the employer should provide adequate resources, time and training to help the committee function effectively. All employees are expected to cooperate with the committee.

This chapter reviews how committees are expected to help the employer maintain an effective IRS.

How committees are expected to function

The role of the committee is to help the employer and workers prevent accidents and illnesses by helping them make the IRS effective.

The employer may delegate responsibilities to the committee. But, the committee is not expected to be a safety officer or enforce health and safety rules. In other words, the committee is not expected to make decisions about what is adequate to protect the health and safety of workers. The committee’s role is to advise and assist, not assume managerial functions. The committee cannot assume any of the legal obligations, duties or responsibilities of either the employer or the employees.

The committee should not be involved in disciplinary matters. Maintaining compliance and adequate levels of health and safety at work is the responsibility of the employer, managers and supervisors. However, as an internal auditor of the IRS, the committee can tell the employer about general problems with worker compliance and recommend corrective action.

Where committees are required

Generally – The employer must set up a committee if the workplace has ten or more workers [Act, section 15].

Construction sites – The contractor (usually the prime contractor) must establish a committee if the site has ten or more employees or self-employed persons working (or likely to be working) for more than 90 days (regulation 38).

Organizations with variable numbers of workers – The employer must set up a committee once the organization has ten or more workers. When the number drops below ten, the employer has three options:

Voluntarily maintaining the committee. For example, the employer could keep a committee of one employer and one worker member.

- Designating a worker occupational health and safety representative if the workplace falls under Table 7 of the Appendix to The Occupational Health and Safety Regulations, 1996.

- Consulting workers on health and safety issues directly where neither a committee nor a representative is required.

Organizations with dispersed workforces – The employer must set up a committee if the workplace has ten or more workers. Employers with ten or more workers at a remote site are encouraged to set up a committee at that site. Employers with less than ten workers at a remote site should encourage their workers to participate in the site committee.

How committees are structured

- The employer must ensure that the committee is structured and maintained as required by law [Act, section 15(1)]. All employees have a duty to help the employer make the committee effective.

- Each committee must have at least two, but not more than 12 members [Act, section 15(2)]. At least half the members must be workers not involved in management.

- In a non-union workplace, the workers must elect their committee members. In a unionized workplace, the constitution of the union determines the appointment process. All employees have a responsibility to help committee members carry out their duties.

- The employer must select management members. Management members must not outnumber worker members.

- The employer may provide clerical support to the committee. Support personnel who are not committee members should not vote or participate unduly in committee deliberations.

Selecting members

The employer should ensure the committee fairly represents workers with significantly different concerns (regulation 39). For example, shift workers should be involved in committee meetings and have their concerns fairly represented.

At a unionized workplace, committee members must be appointed as required by the union’s constitution. If several unions are present, the unions must work out an agreement about how their committee members are to be appointed [Act, section 15(4)]. The employer must provide work time for employees to choose their members.

Posting names

The employer must post the names of committee members in the workplace [Act, section 17].

Selecting co-chairpersons (regulation 43)

Each committee must have two chairpersons. One must be designated by the employer and one selected by worker members of the committee at its first meeting.

Both co-chairpersons have the same rights and responsibilities, including the right to call and chair meetings. The employer co-chairperson is expected to keep the employer informed about the committee’s activities. The worker co-chairperson is responsible for keeping workers informed. Both co-chairpersons are entitled to receive information sent to them by the OH&S Division and distribute it to workers.

Terms of office (regulation 39)

Each committee member serves for three years. Members may serve for more than one term. If a member doesn’t want to serve on the committee anymore, that person should remain in office until someone is ready to take over.

Quorum at meetings (regulation 40)

A quorum for committee votes and decisions means at least:

- half of all members must be present

- half of the members present must be workers

- at least one employer member must be present

The OH&S Division recommends that alternate (replacement) committee members be selected so a quorum can always be formed.

Frequency of meetings (regulation 41)

First meeting – The employer must ensure that a new committee meets within two weeks of being set up.

Next three meetings – The committee must then meet at least once each month for three months. These meetings will help the committee to “get going” and gain credibility.

Regular meetings – Thereafter, the committee must meet at least once every three months.

Additional meetings

- The Director of the OH&S Division can require any committee to meet more than once every three months [regulation 41(2)].

- Either co-chairperson may call special and emergency meetings (regulation 44).

- An occupational health officer may also call special meetings

(regulation 49).

Minutes of meetings (regulation 42)

Committees must return summaries of their meetings to the OH&S Division. The OH&S Division provides a minutes form, but “home made” forms are acceptable if they use the same format. How to fill out a minutes form is discussed in the appendix of this manual.

The co-chairpersons must sign the minutes after each meeting. Within two weeks of each meeting, the employer should ensure that a copy is:

- sent to the OH&S Division

- posted in the workplace

- filed for future reference

The employer should provide the committee with a bulletin board in the workplace for posting minutes and health and safety information.

Duties of committees

Committees help the employer by:

- advising and assisting the employer

- helping the employer to identify, assess and control hazards

- dealing with the concerns of workers

- communicating with workers

- helping orientate new and inexperienced workers

- helping the employer with policies, plans and programs

- helping the employer plan worker training

- handling refusals to work under section 23 of the Act

Beyond this, the committee should work with the employer to continually maintain the effectiveness of the IRS.

Dealing with the concerns of workers

Committees can be invaluable in encouraging workers to discuss their concerns and suggest solutions. Methods include:

- conversations

- contacts during inspections and investigations

- meetings

Steps in dealing with concerns

- Encourage workers to bring specific concerns to their supervisor and general concerns to the committee. For example, a defect in a machine should be promptly drawn to the attention of the local supervisor. A concern about the adequacy of orientation provided to new workers can be brought to the committee.

- If the problem cannot be resolved with the supervisor (management level), take it to the local committee member for investigation. Keep the worker and supervisor informed during the investigation.

- If the committee member cannot resolve the concern, call in the co-chairpersons.

- If the co-chairpersons cannot resolve the problem, deal with the issue at the next committee meeting. If the concern is serious, such as a refusal to work under section 23 of the Act, call an emergency committee meeting.

The committee can help the worker and employer by gathering information on the risks posed by each identified hazard and various alternative courses of action. Information can be obtained from industry safety associations, the OH&S Division and from equipment, tool and material suppliers. Recommendations should be taken to the employer for corrective action. Workers should be kept informed.

The employer is expected to act on the committee’s recommendation, or explain in writing why he or she does not agree with it. If the employer takes corrective action, he or she should send the committee a written report describing what has been done. The committee should file a copy of the employer’s report with the minutes of its meetings.

- If the committee cannot resolve the concern with the employer, an occupational health officer should be asked for help.

- The status and final resolution of the concern should be communicated to everyone involved. This can be done by posting minutes of meetings, distributing bulletins, or holding discussions with workers, supervisors and managers.

- The committee can help everyone concerned by monitoring the effectiveness of the corrective action taken by the employer.

Protection for workers who raise concerns

Under the Act, workers who raise concerns are protected from discriminatory action. Protection against discriminatory action is explained in the text box on the next page.

#### Definition of discriminatory action

2(1)(g):	“discriminatory action” means any action or threat of action by an employer that does or would adversely affect a worker with respect to any terms or conditions of employment or opportunity for promotion, and includes dismissal, layoff, suspension, demotion or transfer of a worker, discontinuation or elimination of a job, change of a job location, reduction in wages, change in hours of work, reprimand, coercion, intimidation or the imposition of any discipline or other penalty, but does not include:

(i)	the temporary assignment of a worker to alternate work, pursuant to section 36, without loss of pay to the worker; or

(ii)	the temporary assignment of a worker to alternate work, without loss of pay to the worker, while:

(A)	steps are being taken for the purposes of clause 23(a) to satisfy the worker that any particular act or series of acts that the worker refused to perform pursuant to that subsection are not unusually dangerous to the health or safety of the worker or any other person at the place of employment;

(B)	the occupational health committee is conducting an investigation pursuant to clause 23(b) in relation to the worker's refusal to perform any particular act or series of acts; or

(C)	an occupational health officer is conducting an investigation requested by a worker or an employer pursuant to clause 24(a);

##### Discriminatory action prohibited in certain circumstances

27	No employer shall take discriminatory action against a worker because the worker:

(a)	acts or has acted in compliance with:

(i)	this Act or the regulations;

(ii)	a code of practice; or

(iii)	a notice of contravention or a requirement or prohibition contained in a notice of contravention;

(b)	seeks or has sought the enforcement of this Act or the regulations;

(c)	assists or has assisted with the activities of an occupational health committee or occupational health and safety representative;

(d)	seeks or has sought the establishment of an occupational health committee or the designation of an occupational health and safety representative;

(e)	performs or has performed the function of an occupational health committee member or occupational health and safety representative;

(f)	refuses or has refused to work pursuant to section 23;

(g)	is about to testify or has testified in any proceeding or inquiry pursuant to this Act or the regulations;

(h)	gives or has given information to an occupational health committee, an occupational health and safety representative, an occupational health officer or other person responsible for the administration of this Act or the regulations with respect to the health and safety of workers at a place of employment;

(i)	is or has been prevented from working because a notice of contravention issued pursuant to section 33 with respect to the worker's work has been served on the employer.

##### Referral to officer

28(1)	A worker who, on reasonable grounds, believes that the employer has taken discriminatory action against him or her for a reason mentioned in section 27 may refer the matter to an occupational health officer.

(2)		Where an occupational health officer decides that an employer has taken discriminatory action against a worker for a reason mentioned in section 27, the occupational health officer shall issue a notice of contravention requiring the employer to:

(a)	cease the discriminatory action;

(b)	reinstate the worker to his or her former employment on the same terms and conditions under which the worker was formerly employed;

(c)	pay to the worker any wages that the worker would have earned if the worker had not been wrongfully discriminated against; and

(d)	remove any reprimand or other reference to the matter from any employment records maintained by the employer with respect to that worker.

(3)	Where an occupational health officer decides that no discriminatory action has been taken against a worker for any of the reasons set out in section 27, the occupational health officer shall advise the worker of the reasons for that decision in writing.

(4)	Where discriminatory action has been taken against a worker who has acted or participated in an activity described in section 27, there is, in any prosecution or other proceeding taken pursuant to this Act, a presumption in favour of the worker that the discriminatory action was taken against the worker because the worker acted or participated in an activity described in section 27, and the onus is on the employer to establish that the discriminatory action was taken against the worker for good and sufficient other reason.

Release No. 55

3813--17

Order to reinstate worker wrongfully discriminated against

29  Where an employer is convicted of taking discriminatory action against a worker contrary to any provision of this Act, the convicting judge or justice shall order:

(a)  the employer to cease the discriminatory action and to reinstate the worker to his or her former employment under the same terms and conditions under which the worker was formerly employed;

(b)  the employer to pay to the worker any wages the worker would have earned if the worker had not been wrongfully discriminated against; and

(c)  any reprimand or other reference to the matter in the employer's records on the worker to be removed. 1993, c.O-1.1, s.29.

Handling refusals to work under section 23 of the Act

Refusals to work represent a failure of the internal responsibility system. An effective IRS should be able to detect and resolve concerns that could cause a refusal. Fortunately, refusals happen very rarely.

The intent of the legislation

Under section 23 of the Act, each employee has the right to refuse work that the worker believes is unusually dangerous. The unusual danger may threaten workers or others. An unusual danger could include:

- a danger that is not normal for the job

- a danger that would normally stop work

- a situation for which the worker isn’t properly trained, equipped or experienced

Section 23 applies only to health and safety issues. It is an individual decision that has to be based on a personal belief that the work is unusually dangerous. During a refusal, the refusing worker is protected from discriminatory action, as defined in section 2(1)(g) and described in section 27 of the Act.

The refusal may continue until either the worker is satisfied the job is no longer unusually dangerous or an occupational health officer has ruled against the refusal. During the refusal, the refusing worker must remain at the workplace unless the employer advises otherwise.

How to investigate a refusal to work

Occupational health committees must investigate refusals. During the investigation, principles set out in Part IV of the Act must be followed. Anyone involved may contact the OH&S Division for help and advice. However, the committee should try to resolve the matter internally before contacting the OH&S Division.

The principles used to deal with concerns can be applied to resolve refusals to work. Follow these steps when investigating a refusal to work.

Inform the supervisor

Steps in a refusal

- The worker should inform the supervisor.

- If the refusing worker and supervisor cannot resolve the concern, involve the committee chairpersons.

- If the refusing worker is not satisfied—have the full committee investigate, hold a committee meeting and vote on the refusal.

- If the refusing worker is not satisfied—contact an occupational health officer.

- Inform employees about the investigation’s findings, or the officer’s ruling. Anyone involved may appeal the officer’s ruling to the Director of the OH&S Division.

- Monitor the effectiveness of the corrective action.

The worker should tell the supervisor that the refusal is because of a health and safety concern. Most refusals are resolved at this point. The refusing worker should not leave the site without the employer’s permission.

Reassignment – The supervisor has the right to assign the refusing worker to other work (at no loss in pay or benefits) until the matter is resolved [Act, section 2(1)(g)(ii)].

Replacement workers – Under section 26 of the Act, an employer cannot assign another worker to the disputed job unless the replacement worker is advised in writing about:

- the refusal and the reasons for it

- why the employer believes that the replacement worker can do the disputed job safely

- the right of the replacement worker to refuse

- the steps to follow when exercising this right

The employer must also inform workers who are expected to do the same job on other shifts about the refusal and the reasons for it.

- Involve the committee co-chairpersons

If the supervisor and worker cannot resolve the refusal, the committee

co-chairpersons should be asked to help. The co-chairpersons have no right to rule on whether or not the disputed job is unusually dangerous.

If there is no committee – Any of the parties involved may contact the OH&S Division.

- Hold an emergency committee meeting

If the co-chairpersons cannot resolve the refusal, they should:

- Convene an emergency committee meeting and have the committee investigate the refusal. If necessary, have the committee rule on whether or not the work is unusually dangerous. This ruling must be made by a vote of a quorum of the committee. A unanimous vote by a quorum of the committee is required to rule against a refusal.

- Send recommendations for corrective action to the employer. Include a summary of the investigation with the minutes of the committee meeting. The employer should act and report to the committee.

- Look for underlying causes of the refusal. Often, a refusal is a sign of an inadequate IRS. The committee can help discover underlying problems leading to the refusal and prevent others.

- Document the investigation. During the investigation the committee should review applicable legislation, work procedures, product documentation and so forth. Occupational health officers may need to review the committee’s investigation file.

- Contact the Occupational Health and Safety Division

During a refusal to work, contact the OH&S Division if the matter has not been dropped or resolved, the refusing worker is not satisfied with the committee’s decision, or the committee cannot agree on how to resolve the refusal.

In such cases, an occupational health officer must investigate and make a ruling. The worker may continue the refusal until the officer rules. The officer will provide a written decision to the refusing worker, each co-chairperson and the employer.

Part VIII of the Act allows anyone directly affected by an officer’s decision to appeal it to the Director of the OH&S Division. The appeal does not suspend the operation of the officer’s decision.

- Communicate the results to employees

The results of the investigation should be summarized on committee minutes and posted for the information of employees. Everyone involved should be kept informed during the investigation.

- Monitor the effectiveness of corrective action

The effectiveness of corrective action should be checked by the committee during inspections, conversations with workers and other activities.

Protection from discriminatory action

Section 27(f) of the Act protects any worker who refuses or has refused to work pursuant to section 23 from discriminatory action. The legislation permits the employer to reassign a refusing worker to other work (at no loss in pay or benefits) until the matter is resolved. The refusing worker is protected until an occupational health officer investigates the situation and rules against the refusal. If the officer rules against the refusal, the worker should return to work.

Where discriminatory action is taken

Under section 28(1) of the Act, an occupational health officer must investigate any complaint of discriminatory action. An officer who finds that discriminatory action has been taken will issue a notice of contravention under section 28(2) of the Act, requiring the employer to:

- cease the discriminatory action

- reinstate the worker to his or her former terms and conditions of employment

- pay to the worker back wages that would have been earned had the discriminatory action not been taken

- remove any reference of the matter from the worker’s employment records

- provide the occupational health officer with a progress report on remedial action [Act, section 35]

The occupational health committee or representative must receive a copy of the progress report. Another copy should be posted.

Where the officer does not find discriminatory action

An officer who does not find evidence of discriminatory action will advise the parties in writing. Anyone directly affected by the officer’s decision may appeal.

Advising and assisting the employer

The role of the committee is to help the employer make the internal responsibility system function effectively. The committee has no direct responsibility within the IRS. It functions as an internal auditor of the IRS.

As an internal auditor of the IRS, the committee should help the employer assess the effectiveness of each aspect of the system. Things that should be considered include:

- the health and safety systems, policies, plans and program

- policies (such as harassment and violence prevention)

- the effectiveness of standards

- the resources allocated to health and safety

- accountability and authority for health and safety decision making

- the importance of health and safety in purchasing and tendering decisions

- the importance of health and safety in the design of the workplace, work stations and work processes

- the effectiveness of worker orientation and training

- the effectiveness of existing hazard controls and work procedures

- the effectiveness of housekeeping and maintenance programs

- the effectiveness of the committee’s activities (such as inspections, investigations, dealing with concerns and so forth)

Worker involvement through the committee increases the likelihood that all hazards and concerns will be identified and solutions will be practical and cost-effective.

Helping the employer with policies, plans and programs

The committee can help the employer to develop policies, plans and programs. Some of these are required by the regulations, including:

Safety programs – The employer at any workplace prescribed in the regulations must develop a health and safety program in consultation with the committee (Act, section 13, regulation 22 and Table 7 of the Appendix to the regulations).

Safety procedures – Where safety procedures are developed the employer should consult the committee or workers to ensure that each procedure is adequate and workable. Use the regulations as a guide.

First aid – Committees are expected to help the employer prepare for accidents and organize a system to provide first aid (Part V of the regulations).

Harassment – Every employer is expected to develop a policy to prevent harassment. The requirements for the policy are explained in regulation 36. Harassment is defined under section 2(1)(l) of the Act.

Violence – The employer at a workplace listed in the regulations is expected to develop a policy to control violence in consultation with the committee. Violence includes actions or threats that have caused (or could cause) injury (Act, section 14 and regulation 37).

Exposure to infectious substances – An employer in a prescribed workplace is expected to develop a policy to control exposure to infectious materials. The employer is expected to consult the committee when developing the policy (regulation 85).

Noise – In certain workplaces, the employer is expected to develop a system to evaluate exposures to noise and recommend corrective action in consultation with the committee (regulation 114).

Smoking – The employer is expected to consult the committee when smoking areas are designated (see regulation 77 for more information).

Committees also have a duty to help the employer control specific problems, such as chemical hazards (Parts XXI and XXII of the regulations). A list of regulations requiring the employer to involve the committee is included in the legislation chapter in this manual.

Helping the employer orientate new and inexperienced workers

New and inexperienced workers have more accidents than experienced workers do. Therefore, before starting work, new hires and workers assigned to new duties or work areas are to receive an orientation (regulation 19). Ideally, orientation records should be kept for each worker.

The committee can help the employer develop the orientation program and monitor its effectiveness. New workers should be told who their committee member is. They should know what to do if they have a concern. The local committee member should look out for the health and safety of these workers.

Helping the employer plan worker training

The employer is expected to provide training for workers and supervisors. Training must include safe work practices and any procedures, plans, policies and programs required by the legislation. The committee can help the employer prepare and monitor worker training.

The employer must consult the committee during the development of any worker training program required by the regulations, including WHMIS instruction, the use and maintenance of respirators and so forth. The committee can help the employer to determine what training is required, select who will deliver the training and evaluate its effectiveness.

Supervisory training

The employer has a responsibility to provide adequate supervision and ensure supervisors are competent. The employer can prepare workers for supervisory positions by helping them get training in:

effective supervisory techniques

- the technology, work processes and so forth associated with the job

- identification and control of workplace hazards

- the role of supervisors within the IRS

- health and safety programs and policies at the workplace

- the role of the committee or worker health and safety representative

- responsibilities under standards set by the organization

- responsibilities under OH&S legislation

Worker training programs

The employer is responsible for ensuring workers are trained in all matters necessary to protect their health and safety. Committees can help the employer do this by:

- Identifying dangerous jobs and jobs with a history of health and safety problems and reviewing:

- the training provided for these positions

- current standards for this training

- required standards set by the industry, the legislation and so on

- Reviewing the orientation program provided to new and inexperienced workers. The orientation should include:

- fire and other emergency procedures

- the location of first aid facilities

- prohibited or restricted areas, tools and equipment

- precautions to be taken when dealing with physical, chemical and biological hazards

- precautions to be taken to deal with other health and safety hazards

- plans and procedures at the workplace

- work rules and any other information required by the organization, industry and legislation

- the role of the worker and committee within the IRS

- worker rights (to know, to participate and to refuse)

- how to report concerns and contact the committee

Review the need for and scheduling of refresher training. Many organizations provide a follow-up session three months after orientation.

- Reviewing standards for on-the-job instruction, coaching and “safety tips” provided by supervisors. Most organizations require the supervisor to show each worker how to do the job safely. This training must reinforce standards set by the organization and legislation. Supervisors must be equipped to provide this instruction.

- Reviewing standards for safety meetings and talks. Regular safety talks provide supervisors with a chance to reinforce health and safety practices. They also demonstrate the commitment of the organization.

- Reviewing health and safety information in documents provided to workers. For example, these publications should contain information about the safety policy, the health and safety requirements of the employer and so forth.

- Reviewing safety rules and job descriptions, including requirements for the use of personal protective equipment (PPE). Ideally, all tasks should have written job descriptions. Descriptions about when, where and how to use PPE should be provided. Workers must be trained to correctly use, store and maintain their personal protective equipment.

- Reviewing documents such as job descriptions, inspection reports, accident investigations and so forth. Look for potential training needs.

- Discussing training needs with managers, supervisors and workers.

- Reviewing procedures for regularly auditing and updating worker training.

Reviewing concerns dealing with training provided to workers. ## Quick Training Checklist # Do Workers: - Know the safety requirements and standards for their jobs? - Know material contained in the orientation program? - Recognize the hazards of each task within the job and the risks associated with those hazards? - Know how to safely use the tools, equipment, substances and materials required for their jobs? - Know the workings and use of machinery guards and other safety devices? - Know when and how to use personal protective equipment and clothing? - Know what safe work procedures to use? - Know how to avoid ergonomic injuries by using safe lifting practices, setting up computers safely and so forth? - Know how to safely handle, use, store and dispose of biological and chemical substances at the workplace? - Know how to report concerns? - Know how to comply with work rules, legislation and other standards? More information about worker training is in the appendix of this manual.

Communicating with workers

The committee is expected to help the employer to communicate information to workers about workplace hazards, safety rules, work practices, worker rights and so forth. In short, the committee helps the employer to put the right to know into practice.

Effective communication is crucial to the success of every committee. It raises morale, increases support for health and safety and prevents conflict. Good communication skills are especially useful during meetings.

More information about communication is included in the appendix of this manual.

Protection for committee members

The right to contact the OH&S Division - Anyone covered by the legislation has this right [Act, section 21], including:

- workers

- employers and supervisors

- contractors and the self-employed

- committee members and worker health and safety representatives

Protection from discriminatory action – Discriminatory action is defined in section 2(1)(g) of the Act. Sections 27 and 28 of the Act prohibit an employer from taking discriminatory action against anyone covered by this section, including committee members.

Immunity from legal action [Act, section 85] – Committee members and worker health and safety representatives are protected from legal action if they carry out their duties in good faith.

Right to appeal – Anyone directly affected by an occupational health officer’s decision may appeal under Part VIII of the Act.

Review

The ultimate purpose of a committee is to help the employer make the IRS effective. The committee audits the IRS, but is not the IRS. It helps the employer provide a healthy and safe workplace, but it is not a safety officer. The employer can share duties with the committee, but cannot share ultimate responsibility for health and safety.

Generally, committees are required in any workplace with ten or more workers. It is up to workers or their unions to select employee members, including one chairperson. The employer is responsible for selecting management representatives and one chairperson. Committees can have as few as two members or as many as 12. Whatever the size, employer representatives may not outnumber worker members. Members serve for one or more three-year terms.

Employers and workers are encouraged to select and prepare alternate (replacement) members to ensure it will always be possible to form a quorum at committee meetings. Be sure to keep alternates informed of committee activities.

During its first year, a committee should meet at least once a month for three months. Thereafter, the committee should meet four times each year. Additional meetings can be held if necessary. Copies of minutes of meetings should be posted and sent to the OH&S Division.

The function of the committee is to help the employer maintain an effective internal responsibility system for accident prevention. This can be done by:

- advising and assisting the employer

- helping the employer to identify, assess and control hazards

- dealing with the concerns of workers

- communicating with workers

- helping plan and monitor orientation and training

- helping the employer with policies, plans and programs

- helping the employer plan worker training

- handling refusals to work under section 23 of the Act

Beyond this, the committee should work with the employer to continually maintain the effectiveness of the IRS.

Committee members and workers who report concerns are protected from discriminatory action by the legislation.

An effective way of resolving concerns is one of the most important components of an effective internal responsibility system. Committees and representatives provide the workplace with a mechanism to identify concerns and resolve them with the employer. The OH&S Division recommends workers report concerns to their supervisor. If the supervisor and worker cannot resolve the concern, the OHC or representative should be involved. If the committee or representative cannot resolve the concern with the employer, the OH&S Division may be asked for help.

Section 23 of the Act gives each employee the right to refuse work that the worker has reasonable grounds to believe is unusually dangerous. While the refusal is being investigated, the worker is protected from discriminatory action, as described in the Act.

Occupational health committees should investigate refusals, following the requirements set out in Part IV of the Act. If the matter cannot be resolved to the satisfaction of the refusing worker, an occupational health officer must investigate. The investigating officer will provide a written ruling to the parties. The officer’s decision may be appealed under Part VIII of the Act. The summary (flowchart) on the next page outlines these procedures.

- The refusing worker informs the supervisor that the job is being refused for health and safety reasons.

- The supervisor and worker attempt to resolve the concern. The supervisor may reassign the worker during the investigation.

- The worker does not leave the site without the permission of the employer.

- The committee co-chairpersons are contacted and asked to assist in the resolution of the refusal.

- They interview the worker and supervisor.

- They refer to standards of the organization, legislation, etc.

- They attempt to resolve the refusal.

- The co-chairpersons convene an emergency committee meeting. The committee investigates the refusal.

- If required, a quorum of the committee votes to decide if the disputed work is unusually dangerous. Unanimity is required to vote against the refusal.

- The committee sends its recommendations for corrective action to the employer. The employer acts and reports to the committee.

- An occupational health officer investigates and provides a written ruling to the parties.

- Anyone directly affected by the officer’s decision may appeal under Part VIII of the Act.

Start

Worker has reasonable grounds to believe job is unusually dangerous

Step 1:

Involve the supervisor

Worker satisfied?

Return to work

Step 2:

Involve the committee

Worker satisfied?

Return to work

Step 3:

Committee investigates

Worker satisfied?

Step 4:

Involve the OH&S Division

Return to work

YES

YES

YES

NO

NO

NO

## Procedural summary for investigating a refusal to work

Notes

Responsibilities of employers and employees towards the committee

- Responsibilities of employers

- Responsibilities of managers, supervisors and workers

Introduction

The employer and employees support effective committees. An effective committee can be invaluable in helping the employer and workers maintain a healthy and safe workplace. This chapter discusses how the employer, managers, supervisors and workers can help the committee to help them.

Responsibilities of employers

Effective employers take the lead in helping the committee to function properly. They create a workplace environment that enhances the effectiveness of the committee, in addition to meeting requirements of the legislation. To do this, the employer should:

Inform workers about the requirements for committees, including what they are expected to do in the workplace.

- Inform managers and supervisors about the committee and their responsibilities towards it.

- Arrange for workers on various shifts and in distinct work areas to be fairly represented on the committee.

- Provide time for workers to select their committee members.

- Appoint influential employer members and/or personally serve as the employer co-chairperson.

- Promptly hold an initial committee meeting to select co-chairpersons and plan activities of the committee.

- Post the names of committee members in the workplace and provide the committee with a bulletin board to post minutes of meetings and health and safety information.

Arrange for the committee to inspect the workplace and discuss concerns with workers to give the committee “momentum”.

- Promptly take corrective action when the committee reports a concern. This will help the committee gain credibility with managers, supervisors and workers.

- Provide the committee with information.

- Provide the committee with adequate time and resources to do its job.

- Ensure the co-chairpersons obtain training in their responsibilities.

- Help the committee meet at least once every three months and ensuring minutes of meetings are sent to the OH&S Division.

- Help the committee inspect the workplace regularly and promptly investigate accidents and dangerous occurrences.

- Involve the committee in the development of policies, plans and programs.

- Involve the committee in worker orientation and training, including WHMIS instruction.

- Ensure that new hires know who their committee member is and how to report concerns.

- Empower managers and supervisors to correct problems brought to their attention by committee members.

- Maintain contact with the committee to ensure it is functioning effectively.

Training co-chairpersons

Training is crucial to the success of the committee. It gives the

co-chairpersons the necessary tools for the job. The employer is expected to help the co-chairpersons obtain training in their duties and functions. In turn, the co-chairpersons are expected to attend training programs and apply their learning at work.

Where a committee member gives reasonable notice, the employer is expected to allow the member to take up to five working days per year to attend occupational health and safety training programs, seminars or courses of instruction. The member does not lose pay or benefits when attending courses conducted or approved by the OH&S Division (regulation 46).

Training provided or approved by the OH&S Division is meant to satisfy regulatory requirements for the basic training of committees and representatives. However, it is not intended to equip committees to deal with specific hazards at the workplace. Progressive employers arrange workplace specific training.

Employers and committee members are encouraged to get training in the following topics:

how to identify and control industry-specific hazards, including ergonomic hazards

- how to conduct inspections

- how to investigate accidents and dangerous occurrences (“near misses”)

- Saskatchewan occupational health and safety legislation

- The Workplace Hazardous Materials Information System (WHMIS)

It is recommended that alternate committee members (those who take the place of co-chairpersons or other members who cannot attend meetings) be trained in their duties. Keep alternate members informed about committee activities by providing them with the information, agendas and so forth that is sent to regular committee members.

The employer and senior managers are encouraged to attend committee training provided by the OH&S Division.

Providing information

The employer is expected to provide the committee with information listed under section 9 of the Act. This includes any information an employer, contractor, owner or supplier knows, or may reasonably be expected to know, that:

- May affect the health or safety of any person who works at a place of employment; or

- Is necessary to identify and control any existing or potential hazards with respect to any plant or any process, procedure, biological substance or chemical substance used at a place of employment; and

- includes any prescribed information.

Regulation 15 requires the employer or contractor to:

(a)	make readily available for reference by workers a copy of:

(i)	the Act;

(ii)	any regulations made pursuant to the Act that apply to the place of employment or to any work done there; and

(iii)	any standards adopted in the regulations that address work practices or procedures and that apply to the place of employment or to any work done there; and

(b)	where the information mentioned in clause (a) or in section 9 of the Act will be posted, provide a suitable bulletin board to be used primarily to post information on health and safety related to the place of employment.

The employer should ensure that the co-chairpersons receive material sent from the OH&S Division. Committee members should share the material with workers.

Committee members should also be informed about:

Any general results of work environment monitoring or biological monitoring. However, confidential (personal) medical information may not be released.

- Reports from consultants that impact on the occupational health and safety of workers.

- Accident and dangerous occurrence notifications and investigation reports.

- Health and safety hazards associated with the place of employment.

Providing time

Committee members will need adequate work time to carry out their duties (at no loss in pay or benefits). This will include time for reviewing and providing recommendations on matters requiring the committee’s input, such as:

- discussing and addressing worker’s concerns

- identifying hazards (through inspections and investigations)

- providing information to employees and the employer

- meeting with the employer and others to obtain information, discuss issues and develop solutions

- reviewing lost time injuries

- investigating certain accidents and dangerous occurrences

Helping the committee to inspect the workplace regularly

The employer should help the committee or the co-chairpersons to regularly inspect the workplace and discuss concerns with workers (regulation 28). The OH&S Division suggests that inspections be made a day or so before each regularly scheduled committee meeting. Committees should summarize inspection reports in the minutes of meetings to keep workers informed.

Inspections with an occupational health officer – The worker

co-chairperson has a right to accompany an officer inspecting the workplace (regulation 20). The employer should encourage the

co-chairperson to discuss workplace concerns with the officer.

Providing progress reports

Where an occupational health officer finds a contravention (violation) of the legislation, the officer will issue a Notice of Contravention. It specifies what legislation must be complied with and by what time. The officer must provide the employer and committee with a copy of the notice.

The employer must then file a Progress Report on Remedial Action within seven days of the deadline set by the officer on the contravention. The report states what has been done to correct the contravention. Copies must be provided to the officer and the committee or worker health and safety representative, or posted in the workplace for the information of workers where there is no committee or representative.

Any committee member has a right to contact the officer about the corrective action taken by the employer.

Helping the committee conduct investigations

Finding the root causes of accidents and dangerous occurrences can prevent recurrences. Investigations can also identify problems within the internal responsibility system (IRS) that need attention.

The employer must notify the OH&S Division of accidents described in regulation 8 and dangerous occurrences listed in regulation 9. These include accidents and dangerous occurrences that cause death or require hospitalization for 72 hours (3 days) or more.

The employer is expected to help the committee investigate and report on accidents and dangerous occurrences (regulations 29 and 31). These include any accident that causes (or may cause) the death of a worker or that places a worker in hospital for 24 hours or more. The employer must file an incident report at the request of an officer and correct concerns raised in the report.

The employer can help the committee review problems in the workplace by providing it with a report listing all lost time injuries over the past year that required medical attention (regulation 32).

The following chart defines reportable accidents and dangerous occurrences and describes what must be in notifications sent to the OH&S Division under regulations 8 and 9.

Accidents causing serious bodily injury

8(1)	An employer or contractor shall give notice to the division as soon as is reasonably possible of every accident at a place of employment that:

(a)	causes or may cause the death of a worker; or

- will require a worker to be admitted to a hospital as an in-patient for a period of 72 hours or more.

- The notice required by subsection (1) must include:

(a)	the name of each injured or deceased worker;

(b)	the name of the employer of each injured or deceased worker;

(c)	the date, time and location of the accident;

(d)	the circumstances related to the accident;

(e)	the apparent injuries; and

(f)	the name, telephone number and fax number of the employer or contractor or a person designated by the employer or contractor to be contacted for additional information.

(3)	An employer or contractor shall provide each co-chairperson or the representative with a copy of the notice required by subsection (1).

## Dangerous occurrences

9(1)	In this section, dangerous occurrence means any occurrence that does not result in, but could have resulted in, a condition or circumstance set out in subsection 8(1), and includes:

(a)	the structural failure or collapse of:

- a structure, scaffold, temporary falsework or concrete formwork; or

(ii)	all or any part of an excavated shaft, tunnel, caisson, coffer dam, trench or excavation;

(b)	the failure of a crane or hoist or the overturning of a crane or unit of powered mobile equipment;

(c)	an accidental contact with an energized electrical conductor;

(d)	the bursting of a grinding wheel;

(e)	an uncontrolled spill or escape of a toxic, corrosive or explosive substance;

(f)	a premature detonation or accidental detonation of explosives;

(g)	the failure of an elevated or suspended platform; and

(h)	the failure of an atmosphere-supplying respirator.

(2)	An employer, contractor or owner shall give notice to the division as soon as is reasonably possible of any dangerous occurrence that takes place at a place of employment, whether or not a worker sustains injury.

(3)	A notice required by subsection (2) must include:

(a)	the name of each employer, contractor and owner at the place of employment;

(b)	the date, time and location of the dangerous occurrence;

(c)	the circumstances related to the dangerous occurrence; and

(d)	the name, telephone number and fax number of the employer, contractor or owner or a person designated by the employer, contractor or owner to be contacted for additional information.

(4)	An employer, contractor or owner shall provide each co-chairperson or the representative with a copy of the notice required by subsection (2).

Resolving concerns reported by the committee

Committees are expected to bring health and safety concerns to the employer for corrective action, including unsafe conditions found during an inspection or investigation (regulation 28).

If the committee reports an unsafe condition, the employer can show leadership and commitment by:

- Immediately protecting the health and safety of workers at risk until the unsafe condition or hazard is corrected.

- Correcting the basic cause of the problem.

Informing workers and the committee about the corrective action taken or planned.

- Helping the committee to monitor the corrective action to ensure that it solved the problem.

The employer may not agree with the committee’s recommendations. If so, the employer is expected to provide the committee with a written explanation. Any committee member has a right to contact the OH&S Division for assistance and advice (section 21 of the Act).

Where a matter is referred to an officer

- The officer may determine that there is no problem or concern and inform the person(s) involved.

- The officer may try to mediate a solution. If a resolution cannot be reached, the officer may provide a written report to the employer and committee stating why a solution cannot be reached.

- The officer may identify violations of the legislation and issue a notice of contravention.

Anyone has a right to contact an occupational health officer.

Providing access to records

The employer can help the committee identify problems in the workplace by allowing it to review records, logs and books the employer must keep (regulation 48). Examples include first aid registers, crane operators’ logs and lists of biological and chemical substances. However, personal medical information may not be released (regulation 10).

Responsibilities of managers, supervisors and workers

The responsibilities of managers and supervisors reflect the employer’s duties.

Managers

To help the committee, managers should:

- Allocate sufficient resources to committee activities (such as money, time, facilities, tools and equipment).

- Encourage supervisors and workers to cooperate with the committee and get involved.

- Involve the committee in all health and safety activities.

- Help to schedule committee activities, such as inspections and investigations.

- Separate health and safety issues from unrelated concerns.

- Work and act safely and ensure that supervisors work and act safely.

Supervisors

- To support their managers, supervisors can:

Ensure workers know what the role of the committee is and who their committee member is.

- Promptly correct concerns reported by the committee and workers and informing the appropriate manager of concerns that cannot be resolved on the shop floor.

- Introduce new workers to the local committee member.

- Involve the committee in shop safety meetings, safety talks, local area inspections and so forth.

- Help the committee carry out inspections and investigations. For example, the supervisor might allow a technical specialist to help the committee inspect a piece of equipment in another area.

- Distribute information provided by the committee and ensuring that workers are aware of the latest committee minutes.

- Encourage workers to serve on the committee.

- Work and act safely and ensure workers do the same.

Workers

To support the committee, workers can:

Serve on it.

- Cooperate with the committee during its activities. For example, a specialist might help the committee inspect a piece of equipment or demonstrate a safe work practice.

- Promptly report concerns to the supervisor and informing the local committee member about concerns the supervisor cannot resolve.

- Work and act safely and help inexperienced workers work safely.

- Help the employer, managers and supervisors support the committee.

To help the committee and employer, the worker’s union can:

Select influential workers to serve on the committee.

- Encourage union members to support the committee.

Share union safety information with committee members and the employer.

- Encourage committee members to attend occupational health and safety training sanctioned by the collective agreement.

- Separate health and safety issues from unrelated concerns.

Review

Everyone has a responsibility for health and safety within the internal responsibility system. The committee helps the employer and workers maintain the system’s effectiveness. In order to do this the committee must be supported.

The employer can support the committee by providing leadership. This begins by identifying each of the committee’s responsibilities and helping members carry them out properly. Effective leadership within the committee will be required to do this. That is why it is so important for the employer to serve on the committee or appoint influential employer representatives.

The support of managers and supervisors is also critical to the success of the committee. Managers can help the committee by allocating adequate resources and obtaining the support of their subordinates. Supervisors can help the committee by involving committee members in supervisory safety activities and dealing with concerns. Finally, workers can support the committee by getting involved.

In the end, the committee will only be as effective as the employer and workers want it to be.

Notes

Holding effective committee meetings

Meeting requirements

- Planning meetings

- Managing meetings and solving problems

- Developing recommendations

- After the meeting

- Role of the employer

Introduction

This chapter discusses how committees can get the most out of their meeting time. Meetings keep committees concentrated on their central role as internal auditors of the IRS. Meetings offer committee members a chance to discuss concerns and make suggestions for corrective action. By meeting regularly, discussing and resolving concerns, the committee and employer demonstrate that health and safety is taken seriously. Items that should be discussed at meetings include:

- the effectiveness of the IRS

the effectiveness of health and safety programs and policies

- new concerns

- inspection and accident investigation reports

- the status and disposition of ongoing concerns

- consultations with occupational health officers, equipment vendors, safety consultants and so forth

- health and safety orientation and training

- the effectiveness of the WHMIS program and worker instruction

- refusals to work under section 23

- information and directives from government agencies relating to health and safety

Organizing an effective meeting often involves planning, managing, solving problems and preparing recommendations. Minutes and meeting planning forms, communications and problem solving techniques are discussed in the appendix.

Meeting requirements

Committees are expected to meet at least once every three months. Committees in workplaces with hazardous conditions are encouraged to meet more frequently.

- A co-chairperson, an occupational health officer, or the Director of the OH&S Division may call special or emergency meetings.

- Management representatives may not outnumber worker representatives at meetings.

- Copies of minutes must be returned to the OH&S Division within two weeks of each meeting. Another copy must be posted at work. File a third copy for future reference.

Planning meetings

You may wish to plan your committee meetings by following these steps:

Provide members with a chance to contribute to the agenda.

- Put unresolved concerns from previous meetings on the agenda.

- Finalize the agenda, select the meeting location and decide when the meeting with start and stop.

- Distribute the meeting announcement, the agenda and copies of the last minutes a few days before the meeting so members can prepare.

- Arrange for necessary committee members to attend and for a quorum to be present.

- Prepare adequate and comfortable meeting room facilities.

Managing meetings and solving problems

Steps in running a productive meeting include:

Welcoming the members and opening the meeting.

- Outlining the agenda and getting a consensus on the topics to be discussed.

- Reviewing the minutes of the previous meeting.

- Dealing with unfinished business from previous meetings.

- Allowing a full, but business-like discussion of each agenda item.

Allowing everyone to present his or her views and have a voice in decision making, but discouraging anyone from dominating the meeting.

- Keeping track of meeting time.

- Summarizing progress of the meeting periodically.

- Getting a consensus on recommendations.

- Ending the meeting with a review of:

what has been accomplished

- who is responsible for carrying out each task agreed to

- what has yet to be resolved

- upcoming events, such as the next inspection or meeting

- Periodically evaluating the effectiveness of meetings.

One co-chairperson should run the meeting and maintain order. The chair should proceed down the agenda systematically; keeping the committee focused on each item under discussion. The chair should also allow everyone to state their opinion, avoid having people speak at once and keep discussions on health and safety. The committee secretary summarizes comments and takes minutes.

If the committee agrees, the employer can provide secretarial, word processing and support services. People who are not committee members should not vote or participate in discussions.

Some items to concentrate on

- Inspection reports and the concerns of workers – These should help evaluate the organization’s IRS. For example, inspection reports may:

- identify a need for better training of new hires

- explain why mishaps are happening in certain areas

suggest improvements in existing hazard controls

- Root causes – Discussions should focus on the root causes of problems. A root cause, such as inadequately designed work processes and equipment, allows a hazard to develop and persist.

- Ongoing concerns – Some concerns, such as ventilation problems, may take a long time to fix. Ongoing concerns should be monitored during inspections and discussed regularly to ensure they are not forgotten. Committee members should be aware of applicable technological advances.

- Health and safety promotion – The committee could discuss what areas of health and safety would be improved by a promotional program, for example back injury prevention, housekeeping, fire prevention, home safety and so forth.

- Orientation and training programs – The committee should periodically review programs and help the employer keep them effective.

| ## Problem solving ideas |  |
| --- | --- |
| ## Use team problem solving techniques - Recognize the problem. - Label the problem. - Analyze the cause of the problem. - Explore possible solutions to the problem. - Make a recommendation to solve the problem. - Prepare an action plan to implement the recommendation. | ## Ask “Why” Five Times - Why did the machine stop? A fuse blew because of overload. - Why was there an overload? There wasn’t enough lubrication in the bearings. - Why wasn’t there enough lubrication? The pump wasn’t pumping enough. - Why wasn’t the lubricant being pumped? The pump shaft was vibrating because of abrasion. - Why was abrasion caused? The filter wasn’t replaced correctly during maintenance due to a misunderstanding caused by a poorly written work procedure. The improperly replaced filter allowed metal chips to get into the pump shaft. |
| ## Use Checklists Ask: “Why,” “Where, ” “When,” “Who,” “What” and “How” to help clarify issues. - Why is it necessary? - Where should it be done? - When should it be done? - Who should do it? - What should be done? - How should it be done? Sometimes, asking the question “what other uses” can be useful in solving problems. For example, how can an issue be: - adapted? - modified? - substituted? - magnified or maximized? - minimized or eliminated? - rearranged? - reversed - combined with something else? | ## Use Brainstorming - Have everyone put ideas forward to resolve the problem. - Write ideas out on flipchart sheets and post them to the wall. - During idea generation, do not criticize or reject anyone’s ideas. Do not discuss any idea until idea generation is complete. - When everyone has run out of ideas, begin to systematically discuss the pros and cons of each idea. List the attributes of the situation. - Below each attribute, place as many alternatives as you can think of. - When completed, make random runs through the alternatives, picking a different one from each column and assembling the combinations into entirely new forms of your original subject. - Continue the process until the best solution(s) is identified. |

Look for the underlying causes of problems

Some problems are like icebergs, only 20 percent can be seen above the surface. This is why it is so important for committee members to look for the root causes of concerns. For example, are back injuries being caused by unsafe workplaces, or by deeper issues, such as poorly designed work areas, inadequate mechanization and so on?

Once a problem is recognized, its components must be identified and categorized. Once this is done, the problem and its components must be analyzed and the root cause recognized. For example, a back injury problem may have components relating to work procedures, work process layout, equipment and so forth.

Consider both facts and feelings

Once everyone’s views are recorded, it may be useful to divide the evidence into facts (hard information) and feelings (such as values, beliefs, attitudes and emotions). Your committee can then consider how to address each issue. This is illustrated in the following table.

| Problem analysis table |  |  |  |
| --- | --- | --- | --- |
| Data | Fact | Feeling | Possible solution |
| - Nurses injuring their backs while lifting patients out of bed, particularly during urgencies and emergencies on night shift. - Shift short staffed due to budget cuts. - Only one patient lift available on floor during shift. - Nurses reluctant to use lift because they do not like its design. - Nursing supervisor feels she is unfairly blamed for the problem. Feels nurses aren’t using safe lifting procedures. - Nurses want more staff and newer patient lifts. Management says it has no money. |    |    | - Review back injury reports and discuss with nurses and employer. Develop written program with the committee specifying procedures required to classify patients (weight, size, health status), label the risk posed by lifting each class of patient and steps to be taken to minimize risk to nurses. Include the lifting techniques and number of staff required for each class of patient. Specify the number of lifts that must be available in the ward. Until the program can be finalized, we recommend that, on the night shift, supervisors ensure two nurses are always used to lift patients. Explore arranging a back-injury prevention course. Ensure a patient lift is always in hall by rooms. Discuss implementation with employer. See regulation 470. - Find out what other staff can be summoned to help lifting patients on the night shift. Examine the costs of injury claims against the cost of hiring new staff. Recommend management request money from the health board to hire more staff. - Determine if lifts can be moved from other floors during the night shift. Find out if new lifts can be purchased. Can lifts be borrowed from other facilities? - Find out exactly why the nurses do not like the design. Have nurses been trained adequately to use the lift? Is the unit obsolete, unstable or broken? Do patients resist getting into it? Can the patient lift be modified or fixed? - Discuss with nursing supervisor. Get her views on possible causes and solutions for the problem. Reassure her. - Explore budgetary situation with employer. See recommendations #2 and 3. Propose purchasing new lifts if more lifts cannot otherwise be made available. |

Involve stakeholders

Involving stakeholders will help committee members identify the root causes of problems. Consider the most effective way of gaining their support. Obtaining their views on cost-effective solutions will build support for your recommendations.

Consider what is known and not known about the problem

Sometimes it is useful to list what is presently known about a problem and what is unclear. This can identify areas for research and break up blind spots.

Review the sequence of events

Consider when the concern was recognized. Find out what happened immediately before it surfaced. For example, was a work process modified? Were new materials or substances introduced?

Developing recommendations

Once a problem has been recognized and its components outlined, the committee can conduct in-depth research and develop recommendations.

- Define the problem – Identify the immediate problem, its components and the root cause from your initial research.

- Research the issues – Involve all committee members. Do not jump to conclusions. Review relevant legislation, standards at other workplaces, equipment manuals, MSDSs, records, diagrams and so forth before making recommendations. Look for the underlying causes for the problems. Ask stakeholders for their opinions and suggestions.

- Select practical choices – Remember that corrective action is usually taken to protect workers and improve performance. Consider cost-effective ideas to help the employer meet both objectives.

Review the alternatives and select those with the most chance of success. Prepare several potential solutions (such as improved worker training and job procedures, better equipment and tools, better work environment controls, replacement of equipment or workplace redesign). Consider controls at the source, along the path to the worker and at the level of the worker.

Develop short-term measures to deal with the immediate causes and long term solutions to remove the root cause. Outline the advantages, disadvantages and costs of each option. Consider the consequences and costs of not implementing each solution (such as continuing injury claims, poor product quality and so forth).

Consider:

the nature of the hazards involved and their risk

- the cost and inconvenience of corrective action

- any new hazards that corrective action might create

- the costs of corrective action versus the status quo

- the cost advantages of corrective action

- compliance issues

- total costs and final cost comparisons of the solution versus the status quo

- Reach agreement – Discussion and consensus should be used to gain agreement rather than votes, which can split the group into competing factions. Plan an implementation schedule (such as who does what, by when and so forth) for each recommendation. Both co-chairpersons should sign the final recommendations.

- Present the recommendations – Only the employer has authority to take corrective action. The committee must be able to convince the employer that its recommendations should be implemented. This is why it is so important for the employer and workers to select influential members.

Recommendations must clearly state what must be done to resolve each concern. Proposals and implementation schedules must be put forward in a way that supports agreement and promotes action.

- Follow-up – The employer and workers should be kept informed while corrective action is being taken. The effectiveness of the improvements should be checked during the next inspection.

# Implementation Schedule Checklist

YES     NO	NO

- Are the actions measurable?							

- Does each person have clear duties and performance standards?			

- Are the deadlines practicable (realistic)?						

- Are resources identified? 								

- Are actions logically sequenced?							

- How will actions be monitored?							

- Is there a contingency plan if things go wrong? 					

- Is there a way of evaluating the results of the plan?                                	

	

After the meeting

Minutes of the meetings should be promptly drawn up and signed by both co-chairpersons. Copies should be:

- distributed to each committee member

- posted in the workplace for the information of workers

returned to the OH&S Division within two weeks of the meeting

- reviewed by the committee’s inspection team immediately before the next inspection

- distributed with the agenda of the next committee meeting

The co-chairpersons should follow-up with the employer on the status of corrective action. They should also check to see that agreed upon tasks have been carried out by the applicable committee members.

Role of the employer

The employer is expected to act on the committee’s recommendations and inform it about what action is planned or being taken. If the employer does not agree with the committee’s recommendation, the employer must provide a written explanation to the committee.

When the employer does not act, it is often for the following reasons:

- The employer believes that action cannot be taken immediately because resources are not available. For example, the employer may not act because of lack of money, production commitments, insufficiently trained staff and so forth.

- The employer may be presented with an inadequate number of choices. For example, the committee may only propose the most expensive and time-consuming solution to the concern.

- The recommendations may be inadequately supported. For example, the committee may not have provided the employer with enough information about the nature and severity of the problem.

- The employer may not agree with the committee’s recommendation or may believe that no hazard exists. In this case, the employer will often want to study the matter.

Try to anticipate these problems when conducting research and preparing recommendations.

Review

Committee meetings help representatives of the employer and workers to communicate with each other about health and safety. They allow the committee to discuss concerns and develop ways of resolving them.

Committees are required to meet at least once every three months. Quorums must be present at all committee meetings. Copies of minutes must be posted for the information of workers. Another copy must be returned to the OH&S Division within two weeks of the meeting. Keep a third copy on file for future reference.

Meetings should be carefully planned, managed and followed up on to ensure the committee benefits from its meetings. Use problems solving techniques to analyze concerns and resolve differences of opinion.

Ultimately, meetings are of little use if the recommendations they produce do not convince the employer to take corrective action. This is why workers and the employer should select influential representatives. Committees will be more likely to have their recommendations acted on if they work hard at carefully identifying the root causes of problems and developing cost-effective options for the employer.

The employer is expected to act on the committee’s recommendations and report on the planned corrective action. If the employer does not agree with the committee’s recommendations, he or she is expected to state why in writing.

The committee can help evaluate the effectiveness of the corrective action by checking to see that it has solved the problem. Keep everyone involved informed.

Maintaining committee effectiveness

- Teambuilding

- Training

- New members

Introduction

This chapter discusses how committee members can cooperate to improve their effectiveness. The more skilled committee members become in problem solving and interpersonal communication, the more successful the committee will be. We’ll begin by describing how members can become a problem solving team. Next, we’ll discuss training and finally, we’ll look at what the committee can do to help new members become effective.

Teambuilding

A team is a group of individuals working together to achieve an agreed upon goal. Successful teams are much more effective than any individual member working alone is. Occupational health committees are intended to be teams of employer and employee representatives working cooperatively towards the goal of maintaining a healthy and safe workplace.

Committee members are expected not to bring their management or union “hats” into committee business. Management representatives must not simply represent the views of the employer, or worker members the views of employees. Both employer and worker representatives are expected to work together to protect the health and safety of everyone in the workplace. For this reason, do not deal with issues that are not health and safety matters.

Establish membership

Committee members must feel able to express their views freely and without risk of retaliation. Members must feel accepted by the group. Leadership and authority within the committee should therefore be established and managed in a way that encourages information sharing, cooperation and compromise.

Clarify the expectations of committee members

Committee members should have a clear idea of what the committee is for, what it can do and what it cannot. Members should have a chance to express what they want to get out of participating in the committee.

Develop group norms

Your committee should develop operating rules dealing with:

- how disagreements among members will be handled

how information will be shared

- how members can support each other

- how decisions will be made

- how priorities will be set

The employer and union or workers should consider periodically rewarding or recognizing the contributions of committee members.

Agree upon goals

Effective committees have a clear idea of what they want to accomplish over the short and long term. The co-chairpersons should provide leadership and help set the tone for the committee. Each member should have a chance to participate and contribute towards setting goals. The goals of the committee should be clearly stated and understood by each member. Consider circulating a list of committee goals and objectives with the agenda of meetings or posting them with the minutes.

Set measurable objectives to achieve the goals

Once the committee knows where it wants to go, whenever possible, members should carefully plan how to go there. In other words, the committee should develop a “work plan” for each year. The plan should be measurable so the committee can evaluate how well it has met its objectives. For example, suppose one goal is to organize a staff safety meeting three times each year. The committee will then have to carefully plan how this is to be achieved. Operational planning will help set priorities and establish responsibilities. At the end of the year the committee can review the plan’s progress, determine if this objective was accomplished and what the results were.

Establish roles and responsibilities for each member

The committee should decide who is going to do what. If tasks such as the chairing of meetings will be rotated, this should be agreed on when the committee is formed. Establish procedures for:

- assigning responsibilities (such as taking minutes, chairing meetings, organizing inspections and so forth)

- making decisions

- communicating and coordinating efforts

- monitoring progress

- evaluating results

The roles and responsibilities of each member should be clear. This will avoid duplication of effort, close gaps in responsibilities and improve efficiency.

Use internal communications to get all committee members involved. Periodically “quiz” or survey committee members about their expectations, roles and responsibilities. Keep members in touch with each other. Make sure each person understands their role and the roles of the other committee members.

Tie individual responsibilities to the goals of the committee. Make sure everyone understands the link.

Agree on ways of handling disagreements

From time to time, committee members will disagree. For example, members may not be able to agree on how a hazard should be controlled. Likewise, the committee may not agree with corrective action proposed by the employer. The committee should decide how to resolve these problems. Methods might include:

- using consensus to make decisions

- asking a neutral third party to mediate

- negotiating mutually acceptable compromises

- using project teams to recommend options to solve difficult technical problems

Clarify how members will support each other

Co-chairpersons need each other’s support. Members have expectations of other members. For example, the inspection team may expect a machinist on the committee to help them inspect certain equipment. Encourage members to express their needs and concerns to clarify expectations.

Consider how to involve influential employees not on the committee

Who represents essential groups within the organization that the committee must influence? How can these individuals be convinced to support the committee? Can they be involved in committee activities such as inspections, investigations and resolving technical concerns? Can they advise the committee on practical methods to improve health and safety?

Consider expectations placed on the committee

Consult workers, union members, supervisors, managers and the employer about their expectations of the committee. Think about how these needs can be served most effectively. Make sure everyone knows what the committee can do and what it cannot. State how concerns should be brought to the committee and how it will deal with them. Consider how to handle complaints about the committee’s performance.

Consider organizational factors affecting the work of the team

The structure of the organization can impact how the work is done. For example, some committee members may work on the night shift; others on the day shift or sections of the organization may not share information. The committee should identify and discuss these concerns with the employer. The employer can demonstrate commitment by resolving them and providing the committee with adequate resources (such as money, time and support).

Consider how to evaluate the performance of the committee

Each year the committee should compare its performance against its stated goals. Draw up a plan to deal with shortcomings. Tell workers about successes so that they will have confidence in the committee. Let the employer know about committee members who have performed well so that they can be recognized for their service.

The employer is ultimately responsible for evaluating the effectiveness of the committee. An effective reward system will help committee members develop a sense of accountability and pride in their contribution to health and safety.

Training

The committee should plan to improve its performance through training. Training should include the roles and responsibilities of committees. It should also equip members to deal with specific workplace issues such as hazard control, equipment inspection and so forth. Build training into the committee’s work plan for each year.

New members

How will new members be educated about the committee and helped to become effective members? To help new members you can:

- Set up a “buddy” system to help new members learn about the committee’s purpose and functions.

- Give each new member a specific assignment or responsibilities within the committee. This will help them get involved and “learn by doing”.

- Spend some time at a committee meeting reviewing the goals and objectives of the committee.

- Prepare a small orientation package for new members.

- Arrange for new members to obtain training in their duties and responsibilities.

Review

Teamwork can help a committee accomplish far more than any individual member can alone. To build a team spirit:

- Establish a sense of membership or belonging.

- Encourage members to state what they want to get out of the committee.

- Clarify what the committee is for, what it can do and what it cannot.

- Develop commonly accepted norms of behaviour.

- Get agreement on the committee’s overall goals and its yearly work plan.

- Link the duties of each committee member to the overall work plan of the committee.

- Agree on how disagreements will be settled.

- Encourage members to support each other.

- Get the support of influential outsiders.

- Make sure all employees understand what the committee is for and what it can and cannot do.

- Assist the employer to remove organizational impediments to the committee’s performance.

- Periodically evaluate and improve the committee’s performance.

Teambuilding can be reinforced by ensuring that new members are trained in their responsibilities and smoothly integrated into committee operations.

Inspections

- Types of committee inspections

- Training for inspections

- Planning inspections

- What to inspect

- Pre-inspection meeting

- What to do during inspections

- Handling the results

- When an unsafe condition is found

Introduction

Inspecting the workplace is one of the key duties of the committee. An inspection is a planned walk through or examination of a workplace, selected work areas or particular hazards, machinery, tools, equipment and work practices. Regular inspections have been shown to reduce accidents and occupational illnesses and to improve the internal responsibility system. This is why the employer must arrange for the committee to regularly inspect the workplace (regulation 28). The committee should work out the inspection schedule with the employer. Managers, supervisors and workers have a duty to report hazards and cooperate.

Committees are involved in regular, planned inspections of the workplace, inspections with occupational health officers and intermittent inspections.

Committee inspections should complement those performed by managers, supervisors and workers. For example, the employer should encourage supervisors and workers to inspect tools, equipment, machinery and personal protective equipment at the start of each shift and report defects immediately. Likewise, supervisors should be encouraged to constantly monitor conditions in their work areas and take corrective action as required. Workers should be encouraged to constantly monitor personal protective equipment for defects.

Inspections help the committee to:

- compare existing conditions with standards, such as regulations and industry practices

- determine if gaps exist between workplace practices and standards set by the organization, industry or legislation

- identify the root causes for any gaps that are found

- develop recommendations for corrective action

The committee can support supervisors and workers by finding defects they have become ‘used to’. Examples include housekeeping hazards, unsafe work practices and hazards in ‘out of the way’ places, such as storage areas.

Inspections are an excellent way of communicating with workers and finding and correcting problems before they cause harm. During an inspection, committee members should ask workers about their concerns. Knowing about problems is the first step in resolving them.

The employer can help the committee to carry out inspections by:

- providing training

providing resources and time

- helping the committee plan and schedule inspections

- helping the committee develop checklists of what to inspect

- encouraging the committee to look for the root causes of problems

Effective inspections concentrate on fact finding and not fault finding or blame fixing.

Types of committee inspections

There are three types of inspections that a committee will typically be involved in— regular inspections, inspections with an occupational health officer and intermittent inspections.

Regular inspections

The employer should help the committee to organize and schedule them. Publicizing the schedule may encourage people to ‘hide’ hazards and unsafe work practices. Some committees meet shortly after each inspection. This provides a chance to discuss the root causes for what was found while memories are still fresh.

Access to records – The committee can review records, logs and books that the employer is expected to keep. Examples include first aid registers, crane logs and lists of chemical substances (regulation 48). However, the committee may not access personal medical information (regulation 10).

Inspections with an occupational health officer

The worker co-chairperson of the committee or a designate should accompany an occupational health officer visiting the workplace. The employer should enable the co-chairperson to accompany the officer (regulation 20). This allows the employer to demonstrate commitment to health and safety. It helps the committee discuss concerns and allows the officer to provide advice.

Intermittent inspections

Accidents and dangerous occurrences, equipment installations, new workers beginning work and other special circumstances may trigger additional inspections by the committee.

Training for inspections

Both the employer and committee members should be familiar with:

- the training and information needed to work safely

work processes and work areas

- workplace hazards and hazardous areas

- applicable personal protective equipment and its limitations

- engineering controls in the workplace

- applicable health and safety standards and legislation

- the recommendations of equipment and material suppliers

- how information is to be recorded

- how concerns are to be reported and dealt with

Planning inspections

The more thorough and professional the inspection program, the safer and healthier the workplace. The effectiveness of each inspection depends on being able to measure existing conditions against clearly defined standards. Standards can be gathered from equipment manuals, trade publications, legislation, suppliers and industry associations.

Standards should be built into checklists and other inspection reporting systems. When planning inspections consider:

- What hazards are likely to be encountered and where?

- What needs to be inspected?

- What aspects of each item need to be looked at and how?

- What conditions and work practices need to be inspected?

- How often must these items, conditions and work practices be inspected?

Who will conduct the inspections (such as the co-chairpersons, experienced workers or outside experts)?

Consider the impact of the inspection on work scheduling. For example, will a machine have to be shut down for inspection? Watch for unexpected hazards, such as welding outside of designated areas.

Inventories and checklists

An inventory (of equipment, materials and so forth) tells you what to inspect. A checklist tells you what to look for when you are inspecting each item in the inventory. The committee can help the employer to prepare an inventory of what should be inspected.

Checklists can be prepared from legislation, industry standards and equipment manuals and by interviewing experienced workers. They can also be purchased. Checklists should be updated regularly.

| Checklist for emergency procedures and first aid (partial list) |  |  |
| --- | --- | --- |
| Emergency procedures | Yes | No |
| Are there emergency procedures in place for your workplace? |  |  |
| Have workers and the committee been involved in development? |  |  |
| Is everyone familiar with the procedures and their responsibilities? |  |  |
| Are there enough emergency exits available throughout the workplace? |  |  |
| Are emergency exits clearly marked and free of obstructions? |  |  |
| Are there enough properly serviced and approved fire extinguishers? |  |  |
| First aid |  |  |
| Is there a properly stocked first aid box? |  |  |
| Do workers know where the first aid box is? |  |  |
| Is someone trained in first aid always available? |  |  |
| Do workers know whom to contact to get first aid? |  |  |
| Is the first aid kit inspected and restocked regularly? |  |  |
| Is an up to date first aid register kept to record injuries? |  |  |
| Is the register inspected regularly to look for trends and sources of injury? |  |  |
| Are all accidents and dangerous occurrences reported and investigated? |  |  |
| Are injuries reported to the Workers Compensation Board (WCB)? |  |  |

Inspectors must know exactly what to look for. Checklist questions should be as precise as possible. What to inspect in each item and part should be clearly identified and described.

Checklists for jobs that are rarely done will usually be more detailed than checklists for jobs that are frequently inspected. The employer and committee should look beyond their checklists and identify root causes of problems.

What to inspect

Four things should be inspected regularly: (1) people; (2) vehicles, tools and equipment; (3) chemicals and biological substances; and (4) the work environment. The committee and employer should develop inspection schedules for each of these components.

- People, including:

- orientation and training

- work practices, work rules and safety procedures

- supervision

experience

- Vehicles, tools and equipment, including:

- machines and mobile equipment

- production, machine-tools and related equipment

- engines, electric motors and other power supplying equipment

- electrical equipment, switches, circuits and so forth

- hand tools and equipment, such as wrenches and power tools

- personal protective equipment and clothing

- first aid stations and emergency equipment, such as eye washes

- fire protection and emergency response devices, such as fire extinguishers and water supplies

- walkways, ramps, docks, parking lots, roadways and so forth

- elevators, hoists and lifts

- storage sheds and areas

- Chemicals and biological substances, including:

- products controlled under the Workplace Hazardous Materials Information System (WHMIS)

- biological substances

- other materials of concern to workers

- Work environment, including:

- illumination

- dust, fumes and vapours

- work area design

- light

- hot and cold conditions

Pre-inspection meetings

Before the inspection the committee should review documents that may help to identify, assess and control hazards. Examples include:

- Inspection reports and records of concerns - These files may show degenerative trends, recurring concerns and ongoing problems.

- Accident reports, WCB claims and first aid registers - These may show where and how people are being injured or made sick.

- Product documentation - Documents for chemicals, machinery, equipment and tools can help identify hazards and suggest controls. Material safety data sheets, (MSDSs). OH&S Division publications and industry literature can also provide assistance.

- Plans and diagrams - Reviewing work process and floor plans and so forth can identify hazards, such as work area design flaws.

What to do during inspections

- Follow-up – Ask workers about the effectiveness of corrective action taken since the last inspection. See if workers, supervisors and maintenance personnel are performing necessary inspections.

- Communicate with workers – Ask workers about their concerns.

- Use monitoring equipment – Noise monitors, chemical sensing equipment and other devices may be required to detect and evaluate specific hazards. If monitoring is required, the employer must provide equipment and training. If consultants are used, the committee is to be involved.

- Take careful notes – Carefully describe each hazard, its seriousness and where it was found. Note all hazards, even those corrected at once. Precisely explain how to fix each problem.

- Communicate with supervisors – Inspectors should discuss with the supervisor what they found. The employer should ensure supervisors have a clear idea of what is expected of them and what they should do when a problem is reported. Employers and managers can support supervisors by ensuring they have the authority and resources needed to take corrective action.

Handling the results

Reports should help the committee and employer to identify problems, assess their probability and severity and take action. Inspection results should be reported consistently. Each hazard or concern must be clearly explained and its location precisely identified.

| Inspection Report Probability - frequent (workers are frequently at risk) - probable (the hazard is likely to cause harm) - occasional (workers are occasionally at risk) - remote (the hazard could cause harm, but is very unlikely to do so) - improbable (the hazard is unlikely to ever cause harm) Severity - catastrophic (death and/or severe destruction) - critical (serious injury and/or property damage) - marginal (minor injury and/or property damage) - negligible (no injury and/or property damage) Area inspected: Building A Date and time of inspection: Mon. June 11, 1999; 11 AM Inspector and title: John Doe; worker co-chairperson Date of report: Monday June 11, 1999; 2 PM Date report discussed: Monday June 11, 1999; 4 PM Number of items from previous report: 2 Number of new items noted on this inspection: 3 Total number of items on this report: 5 #### Definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item “*”are old items | Hazard classification |  | Hazard Description | Specific location | Super | Recommendation | Corrective action taken | Type of action |  |
|  | Probability | Severity |  |  |  |  |  | Temp-orary | Perm-anent |
| *1 | Probable | Critical | Guard missing on shear blade on cutting press #2. Guard ordered May 07. Press in temporary use today to meet emergency order. Guard still missing. | S.W. corner in Bay 2 | Herb Hapless | Press not to be used until guard replaced. Check with purchasing and maintenance. Find out why guard has not been received and installed. | Purchasing says they will have guard by tomorrow noon. To be installed by 4 PM. Purchasing to discuss slow delivery with vendor | X | X |
| *2 | Improbable | Negligible | Cracked windows. Work order issued May 07. Not fixed yet. | South wall on Bay 3 | Herman Melville | Check with maintenance about status of work order. | Maintenance has windows. Will begin installation next week. |  | X |
| 3 | Probable | Critical | Flammable debris building up under main motor. Was to be cleaned June 03 | Pump room | John Safetee | Clean area today. Supervisor will implement written housekeeping policy. | Supervisor will hold meeting tomorrow AM. Implement policy by next Monday. |  | X |
| 4 | Occasional | Marginal | Mirror in pedestrian walkway out of alignment. Some risk of pedestrian collisions. | Floor 2 | Gregory Peck | Post temporary warning sign. Call maintenance for adjustment. | Warning sign posted in PM. Maintenance adjusted mirror this evening. | X | X |
| 5 | Probable | Critical | Workers at cleaning tank not wearing respirators. | Floor 2 | Gregory Peck | Supervisor to review policy with workers and give more training. Respirator policy to be vigorously enforced from now on. | Talked to supervisor. He will hold meeting tomorrow. |  | X |

Put the results of the inspection and any unresolved concerns on the agenda. Classify and rank the hazards in order of importance.

The committee should discuss the agenda and develop proposals for corrective action. Some record should be kept of the concerns discussed. The co-chairpersons should discuss the recommendations with the employer.

Keep careful records of inspections. Inspection records can be useful in tracking the progress of corrective action, identifying degenerative trends and so forth.

File copies of each inspection report. Reports may be needed later for WCB claims and investigations, or use by occupational health officers. Post a copy in the workplace to let workers know what is being done about their concerns. Keep workers who have raised concerns informed.

When an unsafe condition is found

The committee is expected to bring health and safety concerns to the employer. These include hazards and concerns identified during an inspection or investigation (regulation 28).

When the committee reports an unsafe condition, the employer is expected to:

- protect the health and safety of workers at risk until the unsafe condition or hazard is corrected

- correct the basic cause of the problem

- inform workers and the committee about the corrective action taken or planned

When the committee submits a concern to the employer, the employer is expected to deal with it. The committee may ask the OH&S Division for advice about the corrective action taken [Act, section 21]. If a concern is not dealt with, the employer is to provide the committee with a written explanation.

Review

Inspections compare conditions with standards. They allow the committee to identify hazards in the workplace, communicate with workers and help the employer correct problems. An effective inspection program can prevent accidents and improve the internal responsibility system. This is why inspections are an essential responsibility of every occupational health committee.

Committees are normally involved in regular, planned inspections of the workplace and those conducted by occupational health officers. Committees are also encouraged to conduct special inspections when necessary. Committee inspections should complement those performed by managers, supervisors and workers.

The employer can help the committee to carry out inspections by:

- providing training

- providing resources and time

- helping the committee plan and schedule inspections

helping the committee develop checklists of what to inspect

- encouraging the committee to look for the root causes of problems

Plan to inspect people; vehicles, tools and equipment; chemical and biological substances; and the work environment. During each inspection:

- follow-up on corrective action

- carefully identify and describe concerns

- use appropriate monitoring equipment and inspection tools

- communicate with workers and supervisors

Classify and rank concerns in order of importance and place them on the agenda. Discuss the results of the inspection at a committee meeting and develop recommendations for corrective action. Have the co-chairpersons discuss the recommendations with the employer. The employer must:

- take corrective action to protect the health and safety of workers until the hazard is corrected

- correct the root cause of the problem

- advise the committee and workers about the corrective action

If the employer does not agree with the committee's recommendations, the employer must provide a written explanation to the committee.

Investigating accidents

- Planning investigations

- Carrying out investigations

- Finding the causes

- Taking action

Introduction

An accident is any unplanned event that causes injury. A dangerous occurrence (regulation 9) is any event that could have caused injury, but did not. Dangerous occurrences are often called "near misses".

Investigations of dangerous occurrences can prevent accidents. Investigations of accidents can prevent the same thing happening again. It is therefore very important to investigate both accidents and dangerous occurrences. All dangerous occurrences and accidents should be investigated, whether they have to be reported to the OH&S Division or not. A sample of an accident investigation form is included in the appendix.

Investigations should identify health and safety problems, help prevent future accidents and dangerous occurrences. They should not be blame fixing exercises. Each accident and dangerous occurrence usually has several contributing factors, not all of which are obvious. Investigators must look for the deeper causes and not simply record events.

The employer must ensure that the committee is involved in investigations of accidents and dangerous occurrences listed in the regulations. These include accidents that cause the death of a worker or require a worker to be hospitalized for 24 hours or more.  The employer, contractor or owner may investigate dangerous occurrences, depending on who has control over the situation. The regulations specify what must be in the reports (regulations 29, 30 and 31).

If the accident involves a fatality, once the injured are cared for and the site is made safe, the scene must not be disturbed until an investigation can be made (regulation 30).

Reportable accidents and dangerous occurrences

The employer must report to the OH&S Division:

- fatalities

accidents that put a worker in hospital for 72 hours or more (regulation 8)

- dangerous occurrences specified in regulation 9

Both regulations list what the reports must include. The employer is expected to provide copies of reports to the committee.

Planning investigations

The better the planning, the better the investigation. The better the investigation, the more easily a similar accident or dangerous occurrence can be avoided. The employer should work with the committee to prepare an investigation plan setting out the following.

- Investigations – What accidents and dangerous occurrences will be investigated besides those listed by the regulations? The employer and committee are encouraged to investigate all accidents and any dangerous occurrence that could have hurt someone.

- Procedures – How will accidents and dangerous occurrences be investigated? Employers, committees, workers and supervisors should know exactly what to do if an accident or dangerous occurrence happens.

- Training – The employer must ensure that committee members are trained in their responsibilities. Each co-chairperson should attend workshops on how to investigate accidents and dangerous occurrences. Employers are also encouraged to attend. Safety associations, consultants and the Occupational Health and Safety Division provide training. Audiovisual aids and publications are available.

- Resources – The employer should ensure that the necessary tools, personal protective equipment and so forth, are available.

Standards

Accidents and dangerous occurrences often happen because of departures from accepted standards. For example, sometimes safety devices are disabled to increase production, contrary to the rules.

Generally, investigations should: (1) compare what should have happened with what actually happened; (2) determine what gap exists between the two; (3) determine why the gap developed; and (4) recommend appropriate corrective action to prevent a recurrence. Government regulations, company standards and industry publications should be used to decide what standards should have been in place. This data can also help to decide what corrective action is required.

Carrying out investigations

Every accident that causes the death of a worker or a worker to be hospitalized for more than 24 hours must be investigated by the committee co-chairpersons. The employer must prepare a report in consultation with the committee, as set out in regulation 29. A copy must be sent to the OH&S Division at the request of an officer.

The employer must notify the OH&S Division about fatalities, serious injuries and dangerous occurrences (regulations 8 and 9). Where there is a fatality or foul play is suspected, call the RCMP or local police.

The site of a fatality must not be disturbed, except to relieve suffering, until an occupational health officer has investigated. Where an officer cannot investigate and gives permission, the site may be cleared once photos and drawings are taken and the committee co-chairpersons have investigated and agreed (regulation 30).

The committee’s investigation should find the root causes of the incident so that it will not happen again.

Investigations normally include these steps:

- secure the scene and report the accident or dangerous occurrence

- study the scene

- interview witnesses

- investigate the physical evidence.

Secure the scene and report the accident or dangerous occurrence

Things that the supervisor or manager must do include:

- assessing and stabilizing the situation

- making the area safe for emergency crews and investigators

- dealing with the injured

- securing the scene and protecting evidence until an investigation starts

- getting the names of witnesses

- keeping witnesses from talking with each other until they can be interviewed (to keep viewpoints distinct)

- reporting any accident or dangerous occurrence listed in the regulations to the OH&S Division

Notifications sent to the OH&S Division must contain:

- the date, time and location of the accident or dangerous occurrence

- a description of what happened

- the name of each employer or contractor at the place of employment, including the employer of any injured worker(s)

- the name, telephone and fax number of the person to be contacted for more information

If the accident involves a serious injury, the report should also include the name of each injured worker and the apparent injuries to each.

Study the scene

- Look at everything involved in the accident or dangerous occurrence. Carefully go over the debris and note the damage.

- Record the exact location of the accident or dangerous occurrence. Note the lighting, visibility, time of day and weather conditions. Look at the pattern of the debris and the location of each piece.

- Take photographs, measurements and drawings of the scene and everything involved. Be sure to make notes for each illustration.

- Label and catalogue each diagram, drawing and photo.

- Find out who was involved and who can provide expert advice on technical issues. Prepare a list of witnesses and experts to interview.

- Prepare a list of questions to ask.

Interview witnesses

Accurate interview records will be needed to reconstruct what happened and why. Therefore, the employer and committee should reach an agreement about how witnesses will be interviewed. For example, if the RCMP, police or occupational health officers have already interviewed witnesses, further statement taking may not be needed.

Everyone who can provide information should be contacted. Interview eyewitnesses in private when possible, while memories are still fresh. Consider their emotional state, particularly if someone has been killed or seriously injured. Use open-ended questions and let the witnesses explain events in their own words. Avoid interrupting during their statements. Ask clarifying questions later if necessary. Use photographs and drawings to help witnesses remember.

If possible, have each witness visit the scene and show you what he or she saw. Witnesses will have seen events from different perspectives and their statements will often disagree. After eyewitnesses have been interviewed, talk to technical specialists, suppliers and experts. At the end of every interview:

- ask each witness to review their statement

- clear up anything you do not understand

- get the phone number and address of each witness

- thank each witness

Investigate the physical evidence

Study the damage done to tools, equipment, products and so forth. Try to find out what the physical evidence indicates happened and why. Look at the details of the work environment. Consider visibility, noise, temperature, humidity and exposure to hazardous substances. Make detailed photographs and drawings. Describe everything involved in detail, including work procedures and safety policies.

Collect product documentation, including the exact names of any substances involved. Gather material safety data sheets (MSDSs), blueprints and workflow diagrams that might help. Find out if any changes in design, products or work procedures were introduced before the accident or dangerous occurrence.

Compare what happened with the requirements of applicable standards. If requirements were not met, find out why.

Finding the causes

The causes of an accident or dangerous occurrence can often be found by asking Who + What + Where + When + How for every essential event in the accident or dangerous occurrence until you know why the accident happened. Separate facts from theory and opinion as much as possible. Look for underlying causes and avoid jumping to conclusions. Analyze the factors surrounding the accident.

Analyze the accident factors

Go through the accident, in stages. Ask "why" each event happened. Evaluate the role of every factor involved, including:

- people factors

- material factors

- system factors (such as policies, plans and procedures)

- environment factors (such as workplace conditions)

- work process factors (such as work flow design)

People factors – For example, what was the role of:

- the supervision provided

- the instructions given at the time of the accident or dangerous occurrence

- the training and experience of everyone involved

- personal factors (such as inexperience, illness, stress, and so forth)

Material factors – For example, what was the role of:

- substances and materials

- tools, equipment and machinery

- personal protective equipment

System factors – For example, what was the role of workplace:

- health and safety policies, procedures and rules

- budgetary allocations toward health and safety

- training and orientation provided for workers and supervisors

Environment factors – For example, what was the role of:

- weather conditions

- workplace conditions (visibility, noise, heat and so on)

- the time of the day, shift or week when the accident or dangerous occurrence happened

Work process factors – For example, what was the role of the design of:

- the job itself

- the work area and work flow

- controls and safety features on equipment and machinery

Find the accident causes

Three types of causes need to be looked at; direct, indirect and root.

Find the direct cause

The direct cause(s) usually occur(s) immediately before the accident or dangerous occurrence. For example, a direct cause might be a collapsing jack that dropped a car onto a worker.

Find the indirect cause(s)

Indirect cause(s) set the stage for an accident and can include:

- lack of training and supervision

- inadequate tools, equipment and materials

- departures from safe work procedures

The committee and employer should try to find if there were any symptoms of a problem before the accident or dangerous occurrence. If so, why were no concerns expressed? Why did the health and safety system fail?

Find the root cause(s)

An accident or dangerous occurrence may or may not have one ultimate "root" cause. Examples of root causes might include:

- inexperience (employer/worker)

- weaknesses in the safety management program

- equipment design flaws

Taking action

Review what happened at each step in the accident or dangerous occurrence. Prepare a report describing events and recommending corrective action. Use photos and drawings to illustrate key points.

- Describe the work being done.

Describe the immediate (direct) cause(s) of the accident or dangerous occurrence.

- Describe the indirect cause(s).

- Explain why the accident happened (the root cause).

- Recommend corrective action, including both short and long-term controls, to prevent the same thing from happening again. Short-term controls should prevent a recurrence until longer-term controls can remove the fundamental causes.

- File a copy of the report and post summaries.

The employer should take appropriate corrective action based on the report and inform the committee. Inspect the effectiveness of the corrective action.

Review

Accidents hurt people and damage property. Dangerous occurrences are near-accidents that could have hurt someone, but did not.

Pay particular attention to finding and correcting the causes of dangerous occurrences in order to prevent accidents.

Investigations can identify fundamental weaknesses in hazard controls, suggest improvements and prevent a recurrence. The employer is expected to involve the committee in investigations.

The employer and committee should prepare a plan for investigating accidents and dangerous occurrences before one occurs. The plan should include the necessary procedures, personnel and resources.

Begin an investigation by securing the scene and reporting the accident or dangerous occurrence to the OH&S Division. Identify witnesses and keep them from talking to each other. Next, note everything involved in the accident or dangerous occurrence. Take pictures of anything that could be of significance. Describe and photograph the scene. Interview each witness individually. Study and record the damage done to property and the scene in detail. Compare what happened with applicable standards.

Next, analyze the evidence. Find out what directly caused the event. Trace each direct cause back to the underlying causes. Write a report once you are sure that you have found the root cause. Agree on and implement corrective action.

Notes

Legislation

- The difference between an act and regulations

- How to locate and apply information in the legislation

- Key legislation for committees

Introduction

As committee members, you should become familiar with The Occupational Health and Safety Act, 1993 and The Occupational Health and Safety Regulations, 1996. Familiarity with the legislation is very useful in assisting the employer to resolve concerns and comply.

Codes of practice provide advice about how to comply with certain sections of the regulations. For example, the Occupational Health and Safety Division has published a code of practice to advise employers how to comply with health and safety requirements for fire fighters.

This chapter will help you develop important skills in locating information in the legislation. Tables identifying key legislation occupational health committee members should know are included in this chapter. These tables will help you acquire basic knowledge about your responsibilities within the internal responsibility system. (See Chapter 1 for more information about the internal responsibility system).

The difference between an act and regulations

Acts must be passed by a legislature. Regulations only require cabinet approval. Each act usually gives government the authority to enact supporting regulations. Therefore, The Occupational Health and Safety Act, 1993 sets out general duties and responsibilities. The regulations state what must be done to meet them. For example, the Act requires employers in certain industries to have a safety program (section 13). The Occupational Health and Safety Regulations, 1996 spell out what must be in the program (regulation 22). The Appendix to the regulations lists those workplaces requiring programs (Table 7).

How to locate and apply information in the legislation

The Act and regulations are printed together in an office consolidation available from the Queen’s Printer. A green page separates the Act from the regulations. Since the Act and regulations were originally separate publications, both begin with a “page 1”.

Use the tables of contents, index and section headings to find information

Both the Act and regulations have a table of contents. In the Act, a “bird’s eye view” of the table of contents is placed first. A more detailed table of contents follows. The table of contents for the regulations lists the regulations in sequence. Use each table of contents to find general topics. The index applies to the regulations only and does not reference sections of the Act. Use the index to find specific regulations.

Both the tables of contents and the index are organized by section number and not by page number. For example, General Duties of Employers in the Act will be found under section 3 and not on page 3. Section (page) headings in the body of the legislation match the titles in the tables of contents and index.

The Appendix to the Regulations (page 224) lists items referenced, but not included in the Act and regulations. For example, Table 7 lists workplaces requiring representatives.

The Queen’s Printer now provides computer versions of the legislation. Use the “search/find” command to locate specific topics such as “Full-body harness” or “Designated signaller”.

Look in the Preliminary Matters sections to find definitions

The Preliminary Matters sections in both the Act and regulations contain the definitions of terms, such as discriminatory action [Act, section 2(1)(g)] and plant [Act, section 2(1)(x)]. Some parts of the regulations contain Interpretation sections that define key technical terms used there. For an example, see Part XX Diving Operations.

Look in the appendix to find reference lists and tables

The appendix contains technical information referenced, but not included in the regulations. For an example, see Part V First Aid and Tables 1-4 and 8-12 in the Appendix to the Regulations.

Key legislation for committees

| Requirement (This is a partial list. Specific regulations set out additional duties.) | Roles Within the Internal Responsibility System |  |
| --- | --- | --- |
|  | Employer | Committee |
| 1. Consult and cooperate: Act, Part II; section 3(b). | The employer is expected to consult and cooperate with the occupational health committee at the place of employment for the purpose of resolving concerns on matters of health, safety and welfare at work. | The committee should consult workers and help the employer to resolve health and safety concerns. |
| 2. Required information: Act, Part II; section 9(2). | Subject to section 10 and Part VI of the Act, an employer shall provide all required information about hazards, risk assessment and risk control to the following at the place of employment: (a) the occupational health committee; or (b) the occupational health and safety representative. | The committee should use the information to help the employer maintain a healthy and safe workplace. |
| 3. Occupational health and safety programs: Act, Part II; section 13(2). Act, Part II; section 13(4). | The program must be developed in consultation with: (a) the occupational health committee; or (b) the occupational health and safety representative. The occupational health and safety program must be in writing and must be available on request to the occupational health committee, the occupational health and safety representative, the workers or an occupational health officer. | The committee should help the employer to develop the program and keep it current. |
| 4. Policy statement on violence: Act, Part II; section 14(1). | An employer at a prescribed place of employment where violent situations have occurred or may reasonably be expected to occur shall develop and implement a policy statement to deal with potentially violent situations after consultation with: (a) the occupational health committee; or (b) the occupational health and safety representative. | The committee should help the employer to develop the policy statement and keep it current. An example of a generic policy is available from the OH&S Division. |
| 5. Establishment of committees: Act, Part III; section 15. | Subject to the regulations, at every place of employment where 10 or more workers of one employer work, the employer shall: - Establish an occupational health committee at the place of employment; and (b) designate persons as members of the occupational health committee in accordance with this section. An occupational health committee must consist of at least two and no more than 12 persons. At least half of the members of an occupational health committee must represent workers other than workers connected with the management of the place of employment. No person may be designated as a member of an occupational health committee who represents workers unless the person: (a) has been elected from the place of employment for that purpose by the workers whom the person would represent; (b) has been appointed from the place of employment in accordance with the constitution of the trade union of which the workers are members; or (c) where more than one trade union represents the workers that the person would represent on the committee, has been appointed for that purpose from the place of employment pursuant to an agreement among all of those trade unions. | Workers are expected to support the employer and committee. Workers should agree to stand for selection as committee members and if selected, agree to serve. Workers can help the employer make the committee effective by selecting influential and dedicated committee members. |
| 6. Duty to post names: Act, Part III; section 17. | An employer who is required to establish an occupational health committee pursuant to section 15 or the regulations shall post the names of the members of the committee in a conspicuous location at every place of employment of workers represented by the committee. | Committee members should make themselves known to the workers they represent. |
| 7. General concern of committees and representatives: Act, Part III; section 18. | The employer shall help the committee or representative to be effective. | The committee or representative shall have a continuing concern with respect to the health, safety and welfare at a place of employment of workers represented by the committee or representative. |
| 8. Duties of committees: Act, Part III; section 19. | The employer shall help the committee: - to participate in the identification and control of health and safety hazards in or at the place of employment; (b) to co-operate with the occupational health and safety service, if any, established for the place of employment; (c) to establish, promote and recommend the means of delivery of health and safety programs for the education and information of workers; - to maintain records with respect to the duties of the committee pursuant to this section; - to investigate any matter mentioned in section 23; - to receive, consider and resolve matters respecting the health and safety of workers; or - to carry out any other duties that are specified in this Act or prescribed in the regulations. | Committee members shall carry out their duties (in section 19 of the Act) properly and help the employer make the committee effective. |
| 9. Reference to officer: Act, Part III; section 21. | Where an employer does not resolve a problem or address a concern raised by an occupational health committee or representative with respect to the health, safety and welfare of the workers at a place of employment, the employer shall provide written reasons for not doing so to the committee or representative. | Where the parties cannot resolve a problem or address a concern, the employer, the committee, a committee member or the representative may refer the matter to an occupational health officer. Nothing prevents a worker from contacting the OH&S Division directly. |
| 10. Provision of reports by officer: Act, Part III; section 22. | The employer receives a copy of the report from the occupational health officer. | The committee receives a copy of the report from the occupational health officer. |
| 11. Refusal to work: Act, Part IV. | The employer must work with the committee and refusing worker to resolve the issue. Refusals that cannot be resolved internally must be referred to an occupational health officer for a ruling. | The committee shall investigate the refusal and help the employer and refusing worker resolve it. |
| 12. Discriminatory action prohibited: Act, Part IV; section 27. | The employer may not take discriminatory action against a refusing worker. The employer may not take discriminatory action against a committee member because that member carries out his or her duties under the legislation. | A committee member who believes he or she has been discriminated against has a right to contact an occupational health officer. |
| 13. Copy of notice of contravention: Act, Part V; section 34. | The employer receives a copy of the notice of contravention from the occupational health officer. | The committee receives a copy of the notice of contravention from the occupational health officer. |
| 14. Progress report: Act, Part V; section 35. | The employer sends a progress report on corrective action to the officer and provides the committee with a copy within seven days of the deadline set on a notice of contravention. | The committee receives a copy of the progress report. It may contact the officer about the corrective action taken by the employer. |
| 15. Appeals: Act, Part VIII. | The employer has a right to appeal a decision of an occupational health officer. The employer may not take discriminatory action against a worker because of the matter. | Committee members have a right to appeal a decision of an officer. Committee members are protected from discriminatory action if they do so. |
| 16. Immunity: Act, Part XII; section 85. | No action lies or shall be instituted against: an occupational health and safety representative; an occupational health committee; or a member of an occupational health committee in his or her capacity as a member of an occupational health committee…where that person or committee is acting pursuant to the authority of this Act or the regulations, for any loss or damage suffered by a person by reason of anything in good faith done, caused, permitted or authorized to be done, attempted to be done or omitted to be done, by the person, or committee, pursuant to or in the exercise or supposed exercise of any power conferred by this Act or the regulations or in the carrying out or supposed carrying out of any function or duty imposed by this Act or the regulations. |  |
| 17. Accidents causing serious bodily harm: Regulations, Part II; section 8. | The employer reports the accident to the OH&S Division and provides a copy of the notification to the committee co-chairpersons. | The co-chairpersons receive a copy of the notification and are entitled to look into any concerns they have about the accident. |
| 18. Dangerous occurrences: Regulations, Part II; section 9. | The employer must report dangerous occurrences that could have injured a worker to the OH&S Division and provide a copy of the notification to the committee. | The co-chairpersons receive a copy of the notice sent to the OH&S Division. |
| 19. Duty of employer or contractor to provide information: Regulations; Part III, section 15. | The employer is to make a copy of the Act, regulations and standards set out by the regulations readily available to workers. Information sent by the OH&S Division for the information of workers should be posted in the workplace. The employer must provide a bulletin board for the committee’s use. | The committee should review information provided by the employer. The committee should use the bulletin board to keep workers informed of its activities. |
| 20. Duty to inform workers: Regulations; Part III, section 18. | The employer is to inform workers about the requirements of the legislation. The employer is to have a system in place to ensure that each worker knows and follows the legislation. | Workers and committee members comply and help the employer to comply. Workers have a responsibility to cooperate. |
| 21. Workers’ contacts with officers: Regulations; Part III, section 20. | The employer shall provide paid work time for the worker co-chairperson, a committee representative or a worker to accompany an occupational health officer inspecting the workplace. | The committee member should consult the officer and workers about health and safety concerns in the workplace. |
| 22. Biological monitoring: Regulations; Part III, section 21(2). | The employer provides the committee with a report containing the general results of biological monitoring done on workers. | The committee receives a copy of the report. |
| 23. Occupational health and safety program: Act; section 13 and Regulations; Part III, section 22 and Table 7 of the Appendix. | The employer at a prescribed workplace must develop an occupational health and safety program in consultation with the committee. | The committee should help the employer develop the program and audit its effectiveness. |
| 24. Inspection of place of employment: Regulations; Part III, section 28. | The employer is to enable the committee to regularly inspect the workplace. The schedule for the inspections is to be set by the committee and employer. Where the committee provides the employer with a written notice of an unsafe condition, the employer must take action. The health and safety of endangered workers must be protected at once. The unsafe condition must be corrected as soon as possible. The employer must provide the committee with a report on corrective action. If action is not taken, the employer must provide the committee with a written report stating why. | The committee schedules inspections with the employer regularly. It reports unsafe conditions to the employer and receives a report on corrective action. The committee receives a copy of a report from the employer. The committee has a right to contact an occupational health officer if it has any concerns. |
| 25. Investigation of certain accidents: Regulations; Part III, section 29. | The employer must helps the co-chairpersons to investigate and prepare a report. The report’s contents are specified in the regulations. The employer must send a report to the OH&S Division at the request of an officer and provide a copy to the committee. | The co-chairpersons must investigate the accidents and prepare reports setting out the required information. The committee receives a copy of the reports. |
| 26. Prohibition, re: scene of accident: Regulations; Part III, section 30. | The employer ensures that the site of a fatal accident is not disturbed (except to relieve human suffering) until an occupational health officer investigates. The employer may obtain permission from the officer to disturb the site if photographs and drawings of the site are made and the committee co-chairpersons agree. | The co-chairpersons do not give permission unless the requirements of the regulation are met. |
| 27. Investigation of dangerous occurrences: Regulations; Part III, section 31. | The employer must help the co-chairpersons to investigate dangerous occurrences and prepare a report containing the information required in the regulation. The employer provides a copy to the committee. | The co-chairpersons must investigate and prepare reports. The committee receives a copy of the reports. |
| 28. Injuries requiring medical treatment: Regulations; Part III, section 32. | The employer provides a report to the co-chairpersons stating what accidents have resulted in lost-time injuries or required medical treatment and enables the co-chairpersons to review lost-time injuries at no loss in pay or benefits. | The co-chairpersons receive and review the reports. |
| 29. Working alone or in isolated places of employment: Regulations; Part III, section 35. | The employer works with the committee to assess the risks of working alone or in isolated places of employment and develops policies and procedures to control those risks. | The committee should help the employer to identify risks associated with the work and identify methods to control those risks. |
| 30. Harassment: Regulations; Part III, section 36. | The employer works with the committee to develop a policy to prevent harassment. A copy is posted in the workplace. Another is provided to the committee. | The committee should help the employer to develop the policy and communicate the information to workers. |
| 31. Designation of committee members: Regulations; Part IV; section 39. | The employer assists workers to select committee members as required by the regulations and ensures that groups of workers with distinct concerns are equitably represented. | Workers agree to stand for selection as committee members; and if selected, agree to serve. |
| 32. Quorum and certain votes: Regulations; Part IV, section 40. | The employer ensures a quorum is present at every committee meeting and rulings on refusals to work are made by a unanimous vote of a quorum of the committee. | Committee members ensure a quorum is present at every committee meeting. |
| 33. Frequency of meetings: Regulations; Part IV, section 41. | Subject to subsection (2), the employer is to ensure that the committee shall: - hold its first meeting within two weeks after being established;; (b) hold three subsequent meetings at intervals not exceeding one month; and (c) after that, hold regular meetings at intervals not exceeding three months. The Director of the OH&S Division may require a committee to meet more frequently than subsection (1) requires because of any of the following factors at the place of employment: (a) the existence of particular hazards or circumstances; (b) the complexity of the operation; or (c) the number of workers. | Committee members assist the employer to meet the requirements of the regulation. |
| 34. Minutes of meetings: Regulations; Part IV, section 42. | The employer must help the committee to: (a) record minutes of each meeting in a format provided by the OH&S Division and keep the minutes on file with the committee; (b) send a copy of the minutes to the OH&S Division within two weeks after the date of the meeting; and (c) post a copy of the minutes at a location that is readily accessible to workers at the place of employment until all concerns recorded in the minutes are resolved. | Committee members assist the employer to meet the requirements of the regulation. Co-chairpersons should sign the minutes of each meeting. |
| 35. Co-chairpersons: Regulations; Part IV, section 43. | (1) The employer is to ensure that, at the first meeting of a committee: (a) members of the committee representing workers shall elect a worker co-chairperson from among their number; and - the employer or contractor shall appoint an employer or contractor co-chairperson from the members of the committee representing the employer or contractor. (2) An employer or contractor co-chairperson shall keep the employer or contractor informed of the activities, concerns and recommendations of the committee and of any information addressed to the committee. - A worker co-chairperson shall keep the workers informed of the activities, concerns and recommendations of the committee and of any information addressed to the committee. - An employer or contractor shall facilitate the discharge of the worker co-chairperson's duties during normal work hours by permitting meetings of workers or by other means that are appropriate in the circumstances. | The committee helps the employer to meet the requirements. Committee representatives can help by selecting influential and effective co-chairpersons. Co-chairpersons keep employees and the employer informed of the committee’s activities. The co-chairpersons help the employer to keep the committee effective. |
| 36. Special meetings: Regulations; Part IV, section 44. | The employer is expected to assist the committee to hold special meetings. Either co-chairperson may call a special meeting of a committee to deal with: urgent concerns; imminent dangers to health or safety and investigations of accidents, dangerous occurrences or refusals to work pursuant to section 23 of the Act. | Co-chairpersons call special meetings as needed. |
| 37. Training of committee: Regulations; Part IV, section 46. | The employer is to ensure that co-chairpersons receive training respecting their duties and functions. Where a committee member gives reasonable notice, the employer shall allow that member to take up to five days per year to attend training. Where the committee member attends training provided by the OH&S Division or by an approved training agency, the employer must credit the member’s time as time at work. The employer shall ensure the member does not lose pay or benefits for attending. | Committee members attend training respecting their functions and duties and use the training at work. |
| 38. Opportunity for necessary activities: Regulations; Part IV, section 48. | The employer shall ensure the committee has access to any log book or other records required by the legislation. The employer shall provide paid work time to allow the committee to investigate concerns and carry out other duties. The employer shall allow the committee to hold meetings with workers to discuss health and safety issues and regulatory requirements. | The committee reviews the records and investigates related concerns. Members bring the concerns of workers to the employer for resolution. |
| 39. Provision of first aid: Regulations; Part V, section 52. | The employer shall consult the committee about the adequacy of first aid personnel, supplies, equipment and facilities. The employer is to help the committee inspect the first aid register. Requirements are set out in the regulation. | The committee should advise the employer about the adequacy of first aid facilities, supplies, personnel and equipment. The committee should inspect the register and look into any related concerns it has. |
| 40. Ventilation systems: Regulations; Part VI, section 67. | An employer, contractor or owner shall ensure that a record of all inspections, maintenance and cleaning of a mechanical ventilation system required by subsection 66(1) is readily available for examination by the committee, the representative or, where there is no committee or representative, the workers. | The committee should examine the records periodically. |
| 41. Smoking: Regulations; Part VI, section 77. | The employer is to consult the committee about designating smoking areas for the workplace if the employer opts to designate smoking areas. | The committee should help the employer to make the decision. |
| 42. Musculoskeletal injuries: Regulations; Part VI, section 81. | The employer is to consult the committee about jobs and situations in the workplace that pose or could pose ergonomic hazards. The employer is to work with the committee to control or remove these hazards. | The committee should consult workers about ergonomic problems and helps the employer to develop a system to control or remove them. |
| 43. Shiftwork: Regulations, Part VI, section 82. | The employer is to consult the committee to assess the hazards of shiftwork at the workplace, inform workers about those hazards and how they can be controlled. | The committee should help the employer identify shiftwork hazards and inform workers how to control them. |
| 44. Visually demanding tasks: Regulations; Part VI, section 83. | The employer is to consult the committee about jobs and situations in the workplace that put a demand on workers’ vision. The employer is expected to work with the committee to control or remove these hazards. | The committee should consult workers about jobs with a high visual demand and help the employer to develop a system to control or remove them. |
| 45. Radioactive substances: Regulations; Part VI, section 84. | The employer shall consult the committee to develop safe procedures to handle, use, store and dispose of radioactive substances and devices containing radioactive substances. At the request of the committee, the employer shall make available copies of any licenses issued under The Atomic Energy Control Act. | The committee should help the employer to develop safe procedures and review applicable licenses. It consults workers who are at risk and helps them to resolve concerns. |
| 46. Exposure to infectious materials, organisms: Regulations; Part VI, section 85. | The employer consults the committee to develop a written plan to protect workers who are exposed to biohazards. The plan must meet requirements set by the regulation. | The committee should help the employer to develop the plan to meet the requirements of the regulation. It consults workers who are at risk and helps them resolve their concerns. |
| 47. Noise control and hearing conservation: Regulations; Part VIII. | The employer shall consult the committee or representative to ensure that a competent person evaluates the presence of noise and the results of any noise monitoring done at the workplace and that a noise control system is in place to protect workers. | The committee should help the employer to evaluate noise levels in the workplace and protect workers. |
| 48. Robotics: Regulations; Part XV. | The employer shall consult the committee or representative to assess real or potential robotic hazards and develop written procedures to protect workers. The employer ensures that workers are adequately trained to identify and control robotic hazards and operate applicable equipment safely. Requirements are set out in the regulations. | The committee should help the employer to protect workers. It should discuss robotic hazards with workers and help them to revolve related concerns. |
| 49. Entry plan for confined spaces: Regulations; Part XVIII, section 272. | The employer shall consult the committee to develop a safe entry plan for confined spaces to protect workers from the associated hazards. Requirements for the plan are set out in the regulation. | The committee should help the employer to develop the plan. It consults workers about the plan and helps them to resolve concerns. |
| 50. Exposure to chemical and biological substances Regulations; PartXXI, section 302 Exposure to chemical and biological substances Regulations; Part XXI, section 302. | The employer shall make available to the committee, the representative or, where there is no committee or representative, the workers: (a) the results of any measurements of worker exposure to, and contamination of a place of employment by, a chemical substance or biological substance; and (b) any steps taken to reduce the contamination of a place of employment by, and eliminate or reduce exposure of the workers to, a chemical substance or biological substance. | The committee reviews the documents and looks at associated problems. It helps the employer and workers to identify and resolve related concerns. |
| 51. List of chemical and biological substances: Regulations; Part XXI, section 303. | The employer consults the committee or worker occupational health and safety representative to list all potentially hazardous chemical and biological substances that are regularly handled, stored, used or disposed of at the workplace. The list shall also contain any substances at work that are of concern to workers. Controlled products shall be identified on the list. The list shall be amended as required. Copies shall be provided to the workers and the committee or representative. | The committee should help the employer to compile the list and identify controlled products under WHMIS. The committee helps the employer keep the list current. |
| 52. Precautions for certain substances: Regulations; Part XXI, section 304. | Where a chemical or biological substance identified under subsection 303(1) is not a controlled product (or is a controlled product that is exempted from Part XXII) the employer, in consultation with the committee, shall develop a written program to instruct workers about the hazards of these substances and train workers in the precautions to be taken. The employer shall implement the program. | The committee should help the employer to identify these substances, develop the program and monitor its effectiveness. |
| 53. Substances listed in Table 21 of the Appendix to the regulations: Regulations; Part XXI, section 307. | The employer shall consult the committee to develop a written procedure to protect workers from these substances where: (1) exposure is more than 8 hours per day to 40 hours per week; or (2) where there are additive or synergistic effects from the chemical substances. Requirements are set out in the regulation. | The committee should help the employer to comply. It consults workers about the plan and helps them to resolve concerns. |
| 54. Accumulations, spills and leaks: Regulations; Part XXI, section 310. | Where there is a possibility of an accumulation, spill or leak of a chemical substance or biological substance that may be hazardous to the health or safety of a worker at a place of employment, an employer: (a) in consultation with the committee, shall develop written emergency procedures to be implemented in the event of an accumulation, spill or leak; (b) shall make readily available for reference by workers a copy of the emergency procedures developed pursuant to clause (a); (c) shall ensure that each worker is trained in and implements any of the emergency procedures developed pursuant to clause (a) that: (i) require the involvement of the worker; or (ii) are necessary to protect the health or safety of the worker; (d) shall ensure that competent persons, equipment, supplies and personal protective equipment are available for the prompt, safe and effective containment, neutralizing and decontamination of any accumulation, spill or leak; and (e) shall ensure that the emergency procedures developed pursuant to clause (a) are implemented in the event of an accumulation, spill or leak. | The committee should help the employer develop the plan and monitor its effectiveness. |
| 55. Report of worker's exposure: Regulations; Part XXI, section 311. | Where an accumulation, spill or leak of a chemical substance or biological substance listed in Table 19 or 20 of the Appendix occurs and results in the exposure of a worker to the chemical substance or biological substance to an extent that may affect the health or safety of the worker, the employer, in consultation with the committee, shall investigate the incident as soon as is reasonably possible and prepare a written report containing the information set out in the regulation. | The committee should help the employer investigate, prepare the report and develop hazard controls. |
| 56. Worker training (WHMIS): Regulations; Part XXII, section 318(4) and 318(6). | The employer must provide training to workers about the hazards of controlled products at the workplace and how to control those hazards. The training must be developed in consultation with the committee. The employer shall review the WHMIS training program each year in consultation with the committee to ensure it is adequate. | The committee should review the program, consult workers and help the employer keep it current. |
| 57. Availability of MSDSs: Regulations; Part XXII, section 327. | The employer shall ensure copies of all materials safety data sheets required under WHMIS are readily available to the committee. | The committee should review the MSDSs where there are concerns and discuss those concerns with the employer. |
| 58. Asbestos: Regulations; Part XXIII, section 334. | The employer shall make records of potentially hazardous asbestos materials available to the committee for review. | The committee should help the employer protect workers from asbestos hazards. |
| 59. Asbestos control plan: Regulations; Part XXIII, section 337. | The employer shall develop an asbestos control plan in consultation with the committee to protect workers from asbestos hazards. | The committee should help the employer develop and monitor the plan and keep it current. |

Review

You will use the legislation as you carry out your responsibilities. Use it as a guide to help you:

- monitor the effectiveness of the internal responsibility system

- identify, assess and develop recommendations to control hazards

- communicate information about health and safety hazards and issues to workers

- help workers resolve concerns

- maintain the effectiveness of your committee

Remember that your primary job as a committee member is to help the employer maintain a safe and healthy workplace. You are not a safety officer for the employer. Compliance is the employer’s responsibility. Your job is to provide assistance and advice.

Appendices

- Appendix 1:	Notification requirements for new operations

- Appendix 2:	Forms:

- Appendix 3:	Communication, conflict resolution and problem solving techniques for committee members

- Appendix 4:	Helping the employer plan worker training

Appendix 1:	Notification requirements for new operations

Part II of the regulations requires that:

7(1)	As soon as is reasonably possible, an employer, contractor or owner shall give notice to the division of the intention to:

(a)	begin work at a construction site, manufacturing plant or processing plant where 10 or more workers are to be employed for six months or more;

(b)	dig an excavation, a trench or an excavated shaft:

(i)	that is more than five metres deep; and

(ii)	that a worker will be required or permitted to enter; or

(c)	dig a tunnel that a worker will be required or permitted to enter.

(2)	Not later than 14 days before beginning the process, an employer, contractor or owner shall give notice to the division of

the intention to begin a high risk asbestos process listed in Table 5 of the Appendix.

(3)	A notice required by subsection (1) or (2) must include:

(a)	the legal name and business name of the employer, contractor or owner;

(b)	the location of the site, plant, process or place of employment;

(c)	the mailing address of the employer, contractor or owner;

(d)	the nature of the work or process to be undertaken;

(e)	the number of workers to be employed;

(f)	the telephone number and fax number of the employer,

contractor or owner; and

(g)	the estimated starting date and expected duration of the work or process.

Appendix 2:	Forms

How to complete a minutes form

- Submit the complete name, mailing address with postal code, and the phone number of the organization. This information is used to mail out information to your committee and to file your minutes correctly.

- Enter the site address, if it is different from the mailing address, to help us locate your committee.

- Record the total number of persons employed at your workplace together, including both full-time and part-time employees.

- Fill in the meeting date (Day/Month, Year).

- Ensure meetings are held regularly. The minimum requirement for meetings is every three months. Indicate when the next meeting is to be held. If no date is set, scheduling of your meeting becomes more difficult.

- Enter the names of your co-chairpersons. Co-chairpersons are key people and the names must be posted to ensure workers can contact them when necessary.

- List the names and occupations of all your current committee members, identifying each as a worker or management member. Management members must not outnumber worker members.

- Indicate whether each committee member was present or absent. This is necessary to determine whether or not a quorum was present at the meeting.

- Ensure a quorum is present at all of your meetings. A quorum exists when both management and workers are represented, at least one half of the committee is present and at least one half of those present are workers.

- Review all the old concerns from the previous meeting.  Do not remove any item from the minutes until you have recorded it as completed.

- Describe all new concerns, problems and other business discussed at your meeting. An example of an occupational health committee agenda is included in this manual.

- Indicate the action taken or proposed for each item listed, along with the name of the person responsible for handling it. This is essential to track issues, ensure they are dealt with, and communicate the action to those not at the meeting.

- Decide on and record a reasonable target date for each item. Target dates assist in the problem-solving process and give employers and workers objectives to strive for.

- Review all injuries and incidents that have occurred in your workplace and ensure steps have been taken to prevent a recurrence.

- Ensure both co-chairpersons, or their designates, sign the minutes after reviewing the contents to determine accuracy.

- Send one copy of the minutes to the OH&S Division within two weeks of the meeting. This is required by section 42 of The Occupational Health and Safety Regulations, 1996.

- Post one copy of the minutes on your committee bulletin board for worker information. Send one copy to the employer. Retain one copy in your files.

- Use the same format as the OH&S Division shown in this appendix if you wish to set up your own minute form on computer. Following this arrangement will help you identify, assess and solve the problems in your workplace.

- Please enter your OHC# in the top right hand corner of your minutes form and use this number on any correspondence from your committee.

| Saskatchewan Labour Occupational Health and Safety |  |  |  |  |  |  |  |  | #### Minutes Occupational Health Committee |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Complete all information on top: Type or print firmly OHC# 1234 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name of firm |  |  | ABC GADGETS INC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mailing address & Postal Code Worksite address |  | P.O. BOX 12, REGINA |  |  |  |  |  |  |  |  |  | Total # of workers in workplace Meeting date Date of next meeting (min. 3 months) |  |  | 25 |  |  |
|  |  | S4P 4V4 |  |  |  |  |  |  | Phone: 555-1234 |  |  |  |  |  |  |  |  |
|  |  | 1600 - 33rd St., Regina |  |  |  |  |  |  |  |  |  |  |  |  | 18/Sept/99 |  |  |
|  |  | S4P 5V6 |  |  |  |  |  |  | Phone: 545-1225 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Fax: 545-1007 |  |  |  |  |  | 18/Dec/99 |  |  |
| Employer Co-chairperson Judy Smith |  |  |  |  |  |  |  |  | Worker Co-chairperson Victor Jones |  |  |  |  |  |  |  |  |
| Management members |  |  |  | Occupation | Present | Absent |  |  |  | Worker members |  |  |  | Occupation |  | Present | Absent |
| Judy Smith Peter Murphy |  |  |  | C.E.O. Shop supervisor | X X |  |  |  |  | Victor Jones Sandra Roberts Jack Sinclair Tim Banner |  |  |  | Operator Steno IV Maintenance Shipping |  | X X X | X |
| Item Date & No. | ## Problem or Concern Give full explanation and details Divide old/new concerns |  |  |  |  |  | ## Action Taken or Proposed name person responsible |  |  |  |  |  |  |  |  |  | Target Date |
| 1.May 10/99 2.May /99 1.Sept 10/99 2.Sept 10/99 3.Sept 10/99 | OLD CONCERNS Complaints of bad air in main office continue. Several staff sick. They Report bad headaches. Brakes on small forklift in shipping not working properly. NEW CONCERNS Workers in insulation are using compressed air to clean their clothing. Workers complain that they didnt know they werent supposed to use compressed air. May be a training issue. Guards missing on gadget stacker. |  |  |  |  |  | OLD CONCERNS Jack has inspected and cleaned the ventilation system. Defects found. Has asked the maintenance company to service and adjust the system. Repairs to finish within two weeks. Complaints reduced. Staff feeling better. Judy has removed the forklift from service and sent it to the vendor for service. Peter re-scheduled the large forklift to handle the extra work. Jack and Peter have implemented a preventative maintenance program. NEW CONCERNS Peter ordered practice stopped and will order a vacuum for workers to clean their clothing with. Victor and Peter will review the training and safety communication program and improve as necessary. Jack will re-install. Peter and Jack will discuss installing an improved guard with the vendor. |  |  |  |  |  |  |  |  |  | 01/October/ 1999 Complete 05/June/99 01/October/99 01/November/99 20/October/99 |
| Other Business (including requests to Occupational Health and Safety) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Copy 1 – Permanent Committee Files |  |  |  |  | In my opinion the above is an accurate record of this meeting Judy Smith Victor Jones Employer Co-chairperson Worker Co-chairperson Page 1 of 1 |  |  |  |  |  |  |  |  |  |  |  |  |

Committee recommendation form

The government does not issue any “recommendation form” for committees. Many organizations develop their own form for major recommendations that will involve a great deal of money.

Forms help the organization to standardize recommendations and help committees identify exactly what the problem is and what should be done about it. Please adapt the form used in this appendix to your needs. When filling in a recommendation form:

- Describe precisely the item or problem and its location. Identify the workers affected by the problem.

- Provide any necessary background information and research the committee has done to quantify the problem. In the example provided, the committee has done noise monitoring to quantify the exact noise levels workers are exposed to while on the job.

- State what could happen if the problem is not resolved.

- Describe precisely what is being proposed. Include the estimated timelines and costs for each recommendation.

- Provide information to support each proposal in the “Reasons for recommendation” section. In the example provided, the committee has stated why each recommendation is being proposed, what services will be required, who can provide them and when.

- If warranted, propose both short term (immediate) and long term (engineering) solutions. In the example, the committee has suggested a number of actions that can be taken immediately. It has also proposed a longer term engineering solution (sound insulated control room).

- State who is going to follow up on the corrective action and when.

| Meeting date: December 01, 1999 | Chairperson: Jack Mack | Recommendation Number: 1-5 | Date submitted: December 01, 1999 |
| --- | --- | --- | --- |

| Occupational Health Committee Recommendation Form (adapt to suit your needs) |  |
| --- | --- |
| Description of item/concern: Staff are worried about loud noise from boilers and burners in the mechanical room. Many report ringing in their ears and decreased hearing after their shifts. Noise monitoring taken last week showed sound levels of from 91-97 dBA during peak working hours. Our occupational health committee believes workers could suffer permanent hearing loss if this condition persists. |  |
| We recommend: - All workers be issued with high quality hearing protectors to protect their hearing. - Timeline: within two weeks - Estimated cost per year: $500 - Follow-up: Peter Smith, the workers’ manager, will follow-up on January 05, 2,000 - Workers be trained about the hazards of noise and how to use the hearing protectors properly. - Timeline: within two weeks - Estimated cost per year: none—Joe’s Safety Supplies will provide training free of charge if we buy hearing protectors from them. - Follow-up: Jack Mack will follow-up on this by February 01, 2,000 - Measured noise levels be posted in the workplace to inform workers of the hazards present. - Timeline: within two weeks - Estimated cost per year: $30 - Follow-up: Local area supervisors will follow-up on January 05, 2,000 - Workers be encouraged to have their hearing tested. - Timeline: employer must arrange with our health insurer to cover the cost of the hearing tests. The health insurer has a designated hearing test facility. Appointments will have to be arranged by each worker. - Estimated cost per year: none (covered by health insurance). Time at the tests counts as work time under the regulations. - Follow-up: Joe Smoe, the human resources manager, will follow-up with workers every two weeks for the next two months to ensure they are aware of the need for hearing tests and are making appointments with our health insurer. - Relocate all controls to a central area and build a sound-proofed, separately ventilated room around them. Put several windows in the control room to allow workers to monitor operations. - Timeline: during the next fiscal year (suggest completion by May 01/2000) - Estimated cost: $4,000 - Follow-up: Harry Secum and Jack Mack will follow-up with the employer on May 05, 2,000. |  |
| Reasons for recommendation: Short term - All workers be issued with high quality hearing protectors to protect their hearing. Joe’s Safety Supplies informs us they have disposable units that will reduce noise reaching the ear to no more than 85 dBA. These units will not interfere with the ability of staff to communicate. The estimated yearly cost of purchasing a yearly supply of these units for all employees is $500. Joe’s Safety Supplies is willing to train our workers about the hazards of noise and how to use the protectors for free. - Noise levels be posted in the workplace to warn workers of the noise hazard present. Four signs will be required. Joe’s will sell them to us for $30 in total. Workers be encouraged to have their hearing tested. Avaricious Health Insurance, our private health insurer, allows a member one free hearing test each year. The regulations require the employer to arrange hearing tests during work time. Time at the tests will count as work time. Tests will help our workers determine if their hearing is being damaged and make appropriate life choices. It will also bench mark their hearing for future reference. Long term - We relocate all controls to a central place and build a sound insulated control room around it. The room will be separately ventilated and have several windows to allow operators to monitor conditions in the shop. Profit Corporation says they can do the work for us for about $4,000. However, they are busy with other work and will not be able to service us for at least two months. |  |
| Signed Jack Mack Harry Secum |  |
| Employer Co-chairperson | Worker Co-chairperson |
| Copy posted for the information of workers? Yes X No |  |

| Meeting date: | Chairperson: | Recommendation Number: | Date submitted: |
| --- | --- | --- | --- |

| Occupational Health Committee Recommendation Form (adapt to suit your needs) |  |
| --- | --- |
| Description of item/concern: |  |
| We recommend: |  |
| Reasons for recommendation: |  |
| Signed |  |
| Employer Co-chairperson | Worker Co-chairperson |
| Copy posted for the information of workers? Yes  No  Follow-up date: |  |

Committee accident investigation form

The government does not issue investigation forms. Commercially available or “home-made” forms may be used. Use Chapter 8 (Investigating accidents) as a guideline when completing the form.

Guidelines for a report

- The report format should fit the needs of the workplace.

- Information should be complete and easy to understand.

- It should only be as long as is necessary to communicate effectively.

- For best results, write the report as soon as possible after the investigation is finished.

- If the investigation will take a great deal of time, submit an interim report.

- File a copy of the report in case it is needed later.

- Post a summary in the workplace to keep workers informed.

- Attach a copy to the yellow occupational health committee minutes form that is sent to the Occupational Health and Safety Division.

Report format

The format used in this manual includes the following main headings.

Summarize how the incident happened

Provide a brief description of what happened without conclusions (accident summary).

Summarize the direct cause here

Describe the immediate cause of the accident (what happened just before the incident)?

Summarize the root cause here

Describe the underlying problems in the internal responsibility system that set the chain of events in motion that produced the incident.

What actions are recommended to control the immediate causes of the incident

Describe short term controls that should be implemented to prevent another incident until longer term (engineering) controls can be put in place.

Describe what actions are recommended to control the root causes of the accident

Identify engineering and other measures that will remove the fundamental flaws in the internal responsibility system that caused the incident.

Sketch the site of the accident here

A picture is worth a thousand words. Use illustrations to show what happened.

The example used here

In this case, Iwanna Sue, a newly hired leaf spring/suspension mechanic, was asked to service the brakes of a low boy (Bobcat tractor) trailer as soon as possible. Iwanna rushed the job. He placed a bottle (hydraulic) jack under each of the four leaf springs of the trailer. Floor stands or blocks were not used. He removed the wheels to service the brakes. Since the brakes were badly rusted, Iwanna had to do a great deal of pounding and prying to work the parts loose. This caused the trailer to move on the jacks. The trailer slid off the jacks, breaking Iwanna’s left leg.

The direct cause of the incident was found to be using an unsafe work practice (inadequately supporting the trailer). Indirect causes included:

- Assigning a leaf spring/suspension mechanic to do brake repairs. Iwanna turned out to be inadequately trained to repair the brakes on this type of trailer. He was not adequately oriented or supervised.

- Failure by the employer to ensure the bottle jacks were properly labeled and maintained. The jacks used turned out to be too light to support the weight of the trailer. One jack was defective.

The root cause of the accident was lack of a safety program.

| Occupational Health Committee Incident Investigation Report Form OHC FILE: 1234 (adapt to suit your needs) |  |
| --- | --- |
| Company or division: Safetee Motors Inc. | Department: Garage Shop |

Reported to OH&S: Yes X No  Date reported: January 02, 1999Phone # contacted: 1-800-567-7233

| Location: Somewhere Saskatchewan |  |  | Date: January 02, 1999 |  |  | Time: 10:30 AM X PM |
| --- | --- | --- | --- | --- | --- | --- |
| Injury or illness |  | Type of incident (fall, cut, etc) |  |  | Property damage |  |
| Name of injured worker: Iwanna Sue |  | Description: Broken left leg |  |  | Description: Four wrecked bottle jacks. Miscellaneous broken tools. |  |
| Experience: Three hours with this employer |  |  |  |  | Severity of damage/loss: Severe |  |
| Occupation: Leaf spring/suspension mechanic |  | Object or substance inflicting harm: Low boy trailer |  |  | Collateral damage to equipment/ object/ substance related to incident: Damaged trailer chassis. Lost customer. Business closed for a day to investigate and clean up. |  |
| Exact location of accident: Main shop |  |  |  |  |  |  |
| Part of body affected: Left leg |  | Person in control of activity: John Safetee, employer |  |  | Estimated cost: Not known now |  |
|  |  |  |  |  | Final cost: Not known now |  |
| Nature of injury or illness: Broken bone |  | Number of work days lost: 40 (estimated at this time) |  |  | Supervisor: John Safetee |  |
| Summarize how the incident happened: Iwanna was asked to fix the brakes on Jack Rushingman’s low boy trailer. The brakes on the trailer were sticking. Jack needed the trailer right away for a rush job and was putting much pressure on Mr. Safetee to get the thing fixed. Iwanna placed a bottle jack under each of the four leaf springs of the trailer. Floor stands or blocks were not used. The garage hoist was not used because Mr. Safetee had not shown Iwanna how to operate it yet. Iwanna removed the wheels to service the brakes. Since the brakes were badly rusted, Iwanna had to do a great deal of pounding and prying to work the parts loose. This caused the trailer to move on the jacks. The trailer slid off the jacks, breaking Iwanna’s left leg. One of the bottle jacks may have failed, but this is hard to tell because all of the jacks were crushed by the trailer when it fell. |  |  |  |  |  |  |
| Summarize the direct cause here: Use of an unsafe work practice. Bottle jacks should not have been used without other substantial blocks and support. The garage hoist should have been used for the work. |  |  |  |  |  |  |
| Summarize the root cause here: We don’t have an adequate safety program in place to prevent this type of thing. This includes everything from how we orientate and induct new workers to the work procedures and equipment we use. |  |  |  |  |  |  |
| What actions are recommended to control the immediate causes of the incident: - Assign only qualified employees to this type of work and make sure workers are adequately supervised at all times. Turn down or reschedule “rush” jobs if qualified staff are not available. - Use the garage hoist for this type of job. - Inspect tools and equipment for defects. Replace or repair as required. - Develop and enforce work procedures for dangerous jobs. |  |  |  |  |  |  |
| Describe what actions are recommended to control the root causes of the incident: The employer and occupational health committee should develop a safety program that includes written safe work procedures for all dangerous jobs, formal orientation, induction and job training and close supervision for all new hires. |  |  |  |  |  |  |
| Sketch the site of the incident here: Low boy trailer with wheel wells at each corner Iwanna’s final position during the accident |  |  |  |  |  |  |
| Investigators | Position on committee |  |  |  | Signature |  |
| 1. Peter Smith Worker member of committee Peter Smith |  |  |  |  |  |  |
| 2. John Doe Employer member of committee John Doe |  |  |  |  |  |  |
| Signature of occupational health committee co-chairpersons |  |  |  |  |  |  |
| Jack Mack Joe Smoe |  |  |  |  |  |  |
| Employer co-chairperson |  |  |  | Worker co-chairperson |  |  |

Often the report form serves as an incident summary. A more detailed report is sent to the employer.

Occupational Health Committee Incident Investigation Report Form OHC File#: (adapt to suit your needs)

Reported to OH&S: Yes  No  Date reported: Phone # contacted:

| Company or division: |  |  | Department: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location: |  |  | Date: |  |  | Time: AM PM |
| Injury or illness |  | Type of incident (fall, cut, etc) |  |  | Property damage |  |
| Name of injured worker: |  | Description: |  |  | Description: |  |
| Experience: |  |  |  |  | Severity of damage/loss: |  |
| Occupation: |  | Object or substance inflicting harm: |  |  | Collateral damage to equipment/ object/ substance related to incident: |  |
| Exact location of accident: |  |  |  |  |  |  |
| Part of body affected: |  | Person in control of activity: |  |  | Estimated cost: |  |
|  |  |  |  |  | Final cost: |  |
| Nature of injury or illness: |  | Number of work days lost: |  |  | Supervisor: |  |
| Summarize how the incident happened: |  |  |  |  |  |  |
| Summarize the direct cause here: |  |  |  |  |  |  |
| Summarize the root cause here: |  |  |  |  |  |  |
| What actions are recommended to control the immediate causes of the incident: |  |  |  |  |  |  |
| Describe what actions are recommended to control the root causes of the incident: |  |  |  |  |  |  |
| Sketch the site of the incident here: |  |  |  |  |  |  |
| Investigators | Position on committee |  |  |  | Signature |  |
| 1. |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |
| Signature of occupational health committee co-chairpersons |  |  |  |  |  |  |
| Employer co-chairperson |  |  |  | Worker co-chairperson |  |  |

Appendix 3:	Communication, conflict resolution and problem solving techniques for committee members

Introduction

Communication is anything that we do to inform, get information from, or influence others. It includes:

- what we say or write

- our gestures, tone of voice and expression (body language)

- our actions or lack of action (actions speak louder than words)

One-on-one discussions, meetings, committee minutes, bulletins, posters, short publications and safety videos are the most common methods for promoting health and safety. Methods should always provide an opportunity for discussion, questions and feedback.

Four principles of health and safety communication

Effective communication increases motivation

People are more likely to support an idea if they understand the reasoning behind it. For example, workers are more likely to follow a safe work procedure if they understand how the procedure controls the hazards involved in the work. Likewise, people who are kept informed feel involved and believe their opinions are important. Close communication between committee members, the employer, supervisors, managers and workers is critical to the success of the internal responsibility system.

The more people a communication goes through, the more distorted it becomes

The more people there are in a line of communication, the more distorted the message becomes. Each person who passes on the information is likely to change it slightly. To avoid this, try to communicate information directly and provide “back-up copies” in writing. For example, a committee member might inform a worker about the status of a concern. The same information might be included in minutes posted in the workplace.

Effective communication reaches the heart as well as the mind

Communication that is aimed at feelings and attitudes is usually more effective than communication directed only at the mind. People will listen more intently if you mold a message to their personal interests and situations. For example, workers may be more willing to use change and shower facilities if they understand how this can prevent contaminants from reaching their families.

Use it or loose it

The sooner information and skills can be put to work, the more effectively they will be learned and remembered. Help people apply what they are learning as soon as possible. For example, arrangements can be made to immediately apply the safe lifting procedures shown in a safety video. Refresher training and practice can be arranged to keep first aid skills sharp.

Verbal communication techniques

- Use positive body language, including gestures, stance, expression, movement, eye contact and a warm tone of voice. Make the other person feel involved and important.

- Keep the message short and simple. Avoid jargon and unfamiliar terms, particularly with inexperienced workers.

- Show respect for the ideas of others and try to understand their points of view. Consider differences in culture and language.

- Check mutual understanding at the end of the conversation.

Active listening

Active listening is just as important as effective speaking. Six principles are involved in active listening:

Encourage

Encourage the others to fully express their opinions. Use neutral statements or open-ended questions (questions that cannot be answered with a simple “yes” or “no”) such as “I see” or “Please tell me more”. This will keep the others talking and show them you are interested in their views.

Clarify

Use open-ended questions to clarify statements and check your perceptions of what has been said. For example, use questions such as “When did this happen?” or “I’m not quite clear about that point, could you explain it a bit more”?

Restate

Summarize and restate essential points, ideas and comments to check understanding. Put a positive “spin” on what has been said, but do not distort its meaning. Ask the speakers if you are correct in your interpretation of their views. For example: “So your view of the problem is…” “Am I correct in saying that you would like to solve the issue in this way”?

Reflect feelings and opinions

Show you understand the speakers’ opinions and feelings. This will help them evaluate their attitude. For example: “You sound very angry and frustrated”.

Summarize feelings and content

Periodically summarizing ideas, feelings and content allows you to check progress and move on to other topics. For example: “Is this an accurate summary of the cause of your anger”? Encourage the speakers to tell you anything else that is bothering them about the issues. For example, ask: “Is something bugging you”?

Acknowledge and validate

Show the others that you understand, accept and respect them. For example: “I can understand why you might feel this way…” or “ I sympathize with you”.

Pay close attention to the ideas, emotions and opinions being expressed, not to individual words or phrases. Listen to the whole message before passing judgement or interrupting. To avoid bias, try to separate the message from the person sending it.

Use open-ended questions to encourage people to tell you what they know and suggest solutions. State your questions in a positive way to get cooperation. Make sure your body language does not contradict what you are saying.

Written communication

Committee members need to be able to write clear and effective inspection and accident investigation reports, recommendations for the employer and so forth. Information that is unclear or confusing may cause workers and managers to draw the wrong conclusions and fail to take effective action. When preparing a report, consider using the following format outline:

Executive summary – Some formats use this section to summarize the report and its recommendations.

Introduction and purpose – This explains what the report is about and outlines the content.

Methods – This explains how the information was gathered and what methods were used. For example, if the report deals with noise levels in the workplace, this section will describe what type of noise monitors were used, how readings were taken, for how long, where in the workplace and so forth.

Findings – This section describes what was found (the raw evidence). For example, in a noise control report, this section would state what noise levels were found, where, if noise levels at certain work stations changed over the shift and so forth. Some formats summarize the findings here and put detailed information, such as measurements, in the appendix.

Interpretation – This section discusses what the findings mean and builds a case for subsequent recommendations. For example, in a noise control report this section might explain what areas had unacceptably high noise levels, what areas had annoying noise that is of concern to workers and what areas had acceptable noise levels. It would describe the direct causes of the noise (such as machinery, tools and equipment) and contributing factors (such as work scheduling, maintenance practices and so forth).

Conclusions – This section sums up your interpretation and states the root cause of the problems outlined in the report. For example, your noise report might conclude that the root cause of a noise problem is that the organization does not consider noise in its equipment purchasing decisions. Therefore, an extremely noisy machine was purchased instead of a quieter unit with the same capabilities.

Each conclusion should be explained and supported separately. Reference any documents used to reach your conclusions—use footnotes or endnotes. Where possible, include the reference documents with your report.

Recommendations – Recommendations flow from your findings and conclusions. They must be clearly stated and supported by the evidence. Outline the advantages of your recommended course of action. Summarize the possible cost and consequences of not taking it. Where possible, provide options and alternatives. Explain the advantages and disadvantages of each option, including the relative costs.

Supporting documentation – Include material that you have cited in your report or used to support it. For example, include copies of pertinent regulations, safety publications and so forth.

Bibliography – If supporting documentation will not be included, provide a bibliography. This will give the employer or an occupational health officer a chance to check your sources.

Raising controversial issues

Committee members must sometimes raise controversial issues. For example, you might have to talk to a worker who is using an unsafe work practice. A “soft sell” approach is usually much more effective than trying to give orders. Here are some guidelines for raising issues and concerns.

Introduce the topic – Start by briefly describing your role as a committee member, then state why you want to discuss the issue.

State the facts as you see them – Use “I” or  “me” language. For example: “I am curious about why you are using this lifting practice” or “It seems to me that you are using an unsafe lifting procedure, could you explain why”? Focus on behaviour and not the person. Avoid blaming or attacking the other person. Make your points clearly and quickly. The longer you speak, the more likely you are to lose the other person’s attention. Give the other person a chance to respond. If you do not receive a response, ask for one. Listen carefully to the other person’s concerns.

Discuss your interests in keeping the workplace safe – Explain your views clearly, but don’t take rigid positions. Focus on the changes needed to address the issue. Avoid jargon inexperienced workers may not understand. Point out the benefits of doing things the safe way. For example, a worker may not use protective gloves because the chemical he handles does not bother his skin. He may not understand it can be absorbed through the skin. Once understood the worker will likely comply.

Reach an understanding – Listen to the response. Get a commitment to resolve the concern. Check to see that the corrective action has been taken.

Other communications tools

Bulletin boards – These can be used to display committee minutes, inspection reports, summaries of accident investigations, posters and so forth. If safety posters are used, they should be changed every couple of weeks to keep the message fresh.

Circulars and newsletters – Short safety memos and messages can be distributed to employees through the normal channels. For example, a safety brochure can be included in each pay cheque. Newsletters can be distributed or made available in the cafeteria or another centralized location.

Meetings – Committee members should attend supervisors’ group meetings to report on the activities of the committee and the status of concerns. Likewise, the committee and employer can organize employee meetings to discuss health and safety issues.

Safety talks – Safety talks are regular short demonstrations or lectures by supervisors or experienced workers. Each talk deals with a specific topic such as safe lifting procedures, controlling a chemical hazard and so forth. The committee can help the employer plan these safety talks and acquire resources. Materials are available from several safety associations and commercial sources.

Videos – The committee can organize short video screenings to review issues in the workplace. For example, they can be used to reinforce back injury prevention programs. The OH&S Division library has a large selection of health and safety videos.

Handling differences of opinion

Differences of opinion and conflict aren’t necessarily bad. Used properly they can help to improve the quality and quantity of solutions.

The causes of conflict include:

- lack of communication

- different perceptions

- different values

- different expectations about outcomes

People tend to come into conflict when:

- important issues or a preferred outcome are at stake

- at least one side believes it has a greater knowledge or understanding of an issue

- at least one side feels it is “right”

Most disagreements can be resolved through clear communication, trust and mutual respect.

Managing disagreements

Consider these guidelines when dealing with disagreement.

Clarify objectives – People sometimes get into conflict because they have different perceptions about the objectives of the meeting or conversation. There may also be a misunderstanding about the other side’s “real” intentions. Getting agreement on goals can help resolve these problems.

Strive for understanding – Sometimes a disagreement is so strong that people loose track of what the other side’s position really is. The meeting chairperson can deal with this by suspending discussion and asking each side to summarize the other’s position.

Diffuse emotion – Good decisions cannot be reached if people are in an emotional confrontation. Try to keep participants focused on the facts of the issue, the supporting evidence and the consequences of not taking an appropriate course of action.

Generate alternatives – Try to identify solutions that deal with the concerns of all parties. Try to get support for potential solutions from influential participants who are not committed to fixed positions.

Table the issue – When reaching a solution seems unlikely or participants need time to think about the arguments presented, agree to discuss the concern again later. This provides time to “cool off”. However, if the issue is serious, contact the OH&S Division for advice.

Break the issue into components – Consider breaking a controversial issue into its parts. Send each part to a problem solving team for discussion. This may make finding solutions easier.

Appendix 4:	Helping the employer plan worker training

When your review identifies a training need, your committee should work with the employer to:

- Set measurable objectives for the training.

- Determine how the training will be evaluated.

- Obtain or develop the training program.

- Implement the training.

- Evaluate the training.

- Maintain records.

Training procedures

Method 1 – Individualized on-the-job instruction for individuals using the tell, show, try and check (TSTC) method:

- Tell the worker how to do the job safely.

- Show the worker how the job should be performed at each step.

- Let the worker try the job.

Check to ensure the worker can do the job safely.

Method 2 – Classroom training for groups of workers. Consider the following guidelines when using classroom instruction:

- Know the subject.

- Prepare to instruct.

- Develop a lesson plan containing the objectives, content, materials and time required for each section of the instruction.

- Organize and publicize each session.

- Prepare the room and materials.

- Communicate clearly.

- Encourage participation and information sharing.

- Evaluate the training.

- Follow-up to check understanding.

Monitoring orientation and training

Training should be checked during normal committee activities (inspections, conversations with workers, job observations and so forth).

The committee should review orientation and training at least once each year. Recommendations for improvements should be sent to the employer for action.

Notes

Bibliography

Adams, Edward E. Total Quality Safety Management—An Introduction. Des Plaines, Illinois: American Society of Safety Engineers, 1995.

Bird, Frank E. and Germain, George L. Practical Loss Control Leadership. Atlanta Georgia: Institute Publishing, 1985.

Carman, G.A. Re-Energize Your Safety Committee. Mississauga: The Gencor Group Inc., 1996.

Hancock, Jerry and Alexander, Emmie. “Strategic Alignment”. Davidson, NC: Alexander/Hancock Associates, 1997.

Haynes, Marion E. Effective Meeting Skills. Menlo Park, California: Crisp Publications, 1988.

Kaufman, Paul. “10 Innovative Ideas for Successful Teambuilding Events”. Singapore: Active Learning, 1997.

Krieger, Gary R. and Montgomery, John F., eds. Accident Prevention Manual For Business and Industry; Administration and Programs—11th Edition. Itasca, Illinois: National Safety Council, 1997.

Manning, Michael V. “So You’re the Safety Director!”. Rockville, Maryland: Government Institutes, Inc., 1996.

Pokras, Sandy. Team Problem Solving. Menlo Park, California: Crisp Publications, 1995.

Saskatchewan Justice, Mediation Services. Resolving Conflict Constructively. Regina: Saskatchewan Justice, 1997.

Strahlendorf, Peter. Accident Theory, Due Diligence and the

IRS—Parts I-III. Toronto: Peter Strahlendorf, 1997.

Strahlendorf, Peter. “Explaining How Accidents Happen”. Occupational Health and Safety Canada (September/October, 1995): 48-58.

Workplace Health and Safety Agency. Core Certification Training Program Participant’s Manual. Toronto, 1992.

Notes

# Index

Accidents

Causes of	7

Reporting requirements	116

Accident causation theory

Edward Adams model	7

Appeal of officer’s decisions	61

Co-chairpersons (see occupational health committees)

Concerns of workers

Steps in handling	56

Committees (see occupational health committees)

Communicating hazard information	32

Discriminatory action

Definition	58

Protection from	57

Where taken	62

(see also occupational health committees)

Due diligence	18

Employers

Delegation of authority to committee	52

Duties of	17

Duty to report accidents	25

Where the committee does not deal with a problem	22

Ergonomic hazards	38

Falling domino sequences	7

Harassment	40

Hazard

Assessment	30

Control procedures	44

Control selection	47

Communication of information about hazards	32

Definition	27

Evaluation of the effectiveness of controls	48

Health hazards	33

Heat and cold hazards	37

Identification	29

Noise hazards	37

Overview of types of controls	32

Overview of types of hazards	27

Role of employer in control	42

Safety hazards	41

Vibration hazards	37

Inspectors (see occupational health officers)

Internal responsibility system (IRS)

Accountabilities within	16

Advantages	13

Components	16

Definition	12

Goal of	13

Link with OH&S Division	25

Philosophy of	14

Responsibilities for	6

Job safety analysis (JSA)	42

Legislation

Aim	23

Management structure

Role in IRS	8

Middle management

Accountability	 and role in IRS	10

Minute forms

Requirements for	55

Communicating information about refusals to work	61

Near miss	12

Occupational health and safety

Definition of	6

#### Occupational Health and Safety Act

History	23

#### Occupational Health and Safety Regulations

Role	23

Occupational health committees (committees)

Co-chairpersons

Investigating refusals to work	60

Selection	54

Communicating with workers	68

Functioning	52

Hazard control process	29

Investigating refusals to work	59

Meeting requirements		54

Membership and structure	53

Overview of duties	56

Posting names of members	54

Protection	68

Right of member to accompany inspector	25

Role as internal auditor of IRS	14

Role in advising and assisting the employer	63

Role in IRS	21

Role in linking IRS with the OH&S Division	25

Where required	52

Occupational Health and Safety Division (OH&S Division)

Fundamental objective	24

Occupational health officers (Saskatchewan Labour inspectors)

External auditors of IRS	13

Role	24

Occupational illness

Definition	27

Orientation and training of workers	65

Policies, plans and programs	63

Priorities (hazard control)	30

Probability

Definition	28

Protection of workers

Protection of the rights of workers	20

Quorum

At committee meetings	54

Decisions respecting refusals to work	61

Radiation hazards	38

Reassignment of refusing worker	60

Replacement workers	60

Reasonable care (see due diligence)

Refusals to work

Handling	59

Protection of refusing worker	62

Rights of employees and workers

Information sharing	14

Overview of the right to know, participate and refuse	20

Right to refuse	59

Risk

Definition	28

Assessment	30

Risk analysis

Definition	28

Severity

Definition	28

Shared responsibility

Meaning of	19

Shiftworkers	41

Supervisors

Definition of	11

Duties of	18

Role in hazard control	46

Role in IRS	11

Supervisory training programs	65

Unsafe acts and conditions	11

Workers

Duties 	18

Training programs	66

Workplace hazardous materials information system (WHMIS)

Managing the safety of controlled products	43

Total Quality Safety Management—An Introduction. American Society of Safety Engineers, 1995. Adapted from: Accident Theory Part I by Peter Strahlendorf. Occupational Health and Safety Canada magazine, September/October 1995, (pp. 48-58). Southam Magazine and Information Group, Don Mills Ontario.

1 Under section 2(1)(y) of the Act, “Practicable” means possible given current knowledge, technology and invention. Under section 2(1)(aa) of the Act, “Reasonably practicable” means practicable unless the person on whom a duty is placed can show that there is a gross disproportion between the benefit of the duty and the cost, in time, trouble and money, of the measures to secure the duty.

Codes of practice are not legislation. Codes provide advice on how to comply with specific parts of the regulations. For example, recently, the OH&S Division released a code dealing with how to meet safety requirements for fire fighters set out in Part XXXIII of the regulations, Additional Protection for Fire Fighters.